3GPP TSG-RAN WG4 Meeting #110 						                     R4-2400374

Athens, Greece, February 26 – March 1, 2024

Agenda item:	8.5.1.1

Source:	Apple

Title:	Discussion on case 1 requirements of R18 gap enhancement

Document for:	Discussion

# Introduction

Case 1 requirements of R18 gap enhancement was discussed during the previous RAN4 meetings. The last agreements can be found in [1]. In this contribution, we continue discussing the open issue.

# Discussion

The first issue we would like to discuss is about UE feature list for case 1. According to issue 1-1-1 in [1]:

Issue 1-1-1: Rel-18 NR UE features for NR_MG_enh2 WI for Objective 1:

< Way Forward >

- The following feature list will be used as a starting for further discussion in the next meeting.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [32-1] | Concurrent gap with Pre-MG in a FR | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) Pre-MG. Detail in Clause [9.1.x.2] of TS 38.133. | 19-3-x and 19-2 x = 1 or 2 | Yes | No | Network should not configure concurrent gap with Pre-MG | Per UE | No | No | N.A |  | Optional with capability signalling |
| [32-2] | 2 Pre-MG configuration with simultaneous activation/deactivation | Support configurations of 2 Pre-MG with simultaneous activation/deactivation in the same FR. | 32-1 | Yes | No | Unknown activation time for simultaneous Pre-MG is expected | Per UE | No | No | N.A |  | Optional with capability signalling |
| [32-3] | Dynamic collision | Support the RRM requirements when the activation/deactivation delay of Pre-MG overlaps the other measurement gap with lower priority or Pre-MG | 32-1 | Yes | No | UE is not expected to meet the requirements | Per UE | No | No | N.A |  | Optional with capability signalling |
| [32-4] | Concurrent gap with NCSG in a FR | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) NCSG. Detail in clause [9.1.y.2] of TS 38.133. | 19-1 and 19-2 | Yes | No | Network should not configure concurrent gap with NCSG | Per UE | No | No | N.A |  | Optional with capability signalling |
| [32-5] | 2 NCSG configuration in a FR | Support configurations of 2 NCSG in the same FR | 32-4 | Yes | No | Network should not configure 2 NCSG in the same FR | Per UE | No | No | N.A |  | Optional with capability signalling |

[32-4] and [32-5] are for case 2. We only need to review [32-1], [32-2] and [32-3].

We support [32-1]. Since new UE behaviours and corresponding RAN4 requirements are introduced in R18, a new UE capability is necessary. Both R17 Pre-MG and R17 concurrent gaps are prerequisite for R18 case 1.

We are open for [32-2]. Note that the big CR endorsed in RAN4#110 didn’t consider this potential additional UE capability. i.e. 2ms post-processing time is added for fully overlapped activation/deactivation by default.

#### 8.19.x.2 Activation/deactivation delay requirements for fully overlapped activation/deactivation of multiple pre-configured measurement gaps The requirements in this clause only apply when the activation/deactivation procedures of the individual pre-configured measurement gaps fully overlap in time. Fully overlapped activation/deactivation of pre-configured measuremeng gaps can occur in the following cases: - Both pre-configured measurement gaps are triggered by the same event. - Two pre-configured measurement gaps are triggered by two events of the same type at the same time. When multiple pre-configured measuremeng gaps are activated/deactivated simultaneously, the activation/deactivation delay equals the delay requirements defined at clause 8.19.2, 8.19.3 and 8.19.4 for DCI/timer-based BWP switch, SCell activation/deactivation or RRC reconfiguration, respectively, plus an additional 2ms post-processing time.

Regarding [32-3], we don’t see the necessity. Besides, in existing section 9.1.12.3 it is clearly stated that only activated Pre-MG would cause interruption:

#### 9.1.12.3 Collisions involving Pre-MG(s) Collisions between a Pre-MG and a measurement gap may occur only when the Pre-MG is activated. No collisions can occur between a per-FR Pre-MG and a per-FR measurement gap when they are configured in different FRs. Collisions between two Pre-MGs may occur only when both Pre-MGs are activated. No collisions can occur between a per-FR Pre-MGs when they are configured in different FRs. The requirements for [concurrent measurement gaps with Pre-MG] apply provided that the two measurement gaps(at least one of the gaps is activated Pre-MG) colliding with each other are configured with different priorities.

If a new UE capability is introduced to support dynamic collision, RAN4 still needs to develop requirements for the case wherein deactivated would also cause collision, which would not only extend the work but also complicates network implementation, since network needs to consider both kinds of UE.

Therefore, we only see a strong need of [32-1]:

Proposal 1: UE feature list for case 1:

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [32-1] | Concurrent gap with Pre-MG in a FR | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) Pre-MG. Detail in Clause [9.1.12] of TS 38.133. | 19-2, and 19-3-1 or 19-3-2 | Yes | No | Network should not configure concurrent gap with Pre-MG | Per UE | No | No | N.A |  | Optional with capability signalling |

Correspondingly, the following editor’s note can be removed.

#### 9.1.12.2 Requirements If the UE requires measurement gaps and/or Pre-MGs to identify and measure intra-frequency cells and/or inter-frequency cells and/or inter-RAT E-UTRAN cells, and the UE supports [ConMGs with Pre-MG] but does not support independent measurement gap patterns for different frequency ranges as specified in Table 5.1-1 in [18, 19, 20], in order for the requirements in the following clauses to apply, the network can provide the UE with not more than two per-UE measurement gap patterns for monitoring all the frequency layers. Editor Notes: FFS additional capability for UE to support Pre-MG+Pre-MG Two Pre-MGs may be configured by the network only if the UE supports [Pre-MG+Pre-MG]. If the UE supports both [ConMGs with Pre-MG] and independent measurement gap patterns for different frequency ranges as specified in Table 5.1-1 in [18, 19, 20], in order for the requirements defined for concurrent measurement gaps with Pre-MG to apply, the network can provide the measurement gap patterns’combinations specified in Table 9.1.12.1 for monitoring of all frequency layers.

They can be added back if the additional UE capability can be agreed.

Proposal 2: remove context regarding UE capability of Pre-MG + Pre-MG in existing spec.

# Conclusion

In this contribution, we provide discussion on case 1 requirements. After discussion, the following conclusions are provided:

Proposal 1: UE feature list for case 1:

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [32-1] | Concurrent gap with Pre-MG in a FR | Support of multiple per-UE (or per-FR) measurement gap patterns with at least one per-UE (or per-FR) Pre-MG. Detail in Clause [9.1.12] of TS 38.133. | 19-2, and 19-3-1 or 19-3-2 | Yes | No | Network should not configure concurrent gap with Pre-MG | Per UE | No | No | N.A |  | Optional with capability signalling |

Proposal 2: remove context regarding UE capability of Pre-MG + Pre-MG in existing spec.

# References

- R4-2321640, WF for NR_MGE-2_part1, MediaTek inc.

- R4-2317305, WF on NR_MG_enh2_part1, MediaTek inc.

- R4-2314323, WF on NR MG enhancements (Part 1), MediaTek inc.

- R4-2310175, WF on NR MG enhancements, MediaTek inc.

- R4-2306330, WF on Rel-18 NR MG enhancements, MediaTek inc.

- R4-2220359, WF on NR_MG_enh2 Part 1, MediaTek inc.

- R4-2217251, WF on further enhancements on measurement gaps and measurements without gaps, MediaTek inc.

- R4-2202604, Further reply LS on R17 NR MG enhancements – Concurrent MG, MediaTek inc.

- RP-230755, New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps, MediaTek Inc, Intel Corporation