3GPP TSG-RAN WG4 Meeting #110 						                     R4-2400383

Athens, Greece, February 26 – March 1, 2024

Agenda item:	8.6.1

Source:	Apple

Title:	Discussion on RRM core requirements for bandwidth part operation without restriction

Document for:	Discussion

# Introduction

RRM requirements of BWP without restriction were extensively discussed in previous RAN4 meetings. The latest agreements can be found in [1]. However, there are still some open issues left. In this contribution, we continue discussing the open issues.

# Discussion

According to [1], the following issues are to be discussed in maintenance part:

Companies are encouraged to bring analysis on the following issues.

- Whether and how to define requirements/UE behaviour for UE supporting both option B-1-1 and A

- Whether and how to define requirements/UE behaviour for UE supporting both option C and A

- UE supporting both option B-1-1 and A

We believe it is unnecessary for UE to measure both CSI-RS and SSB for RLM/BM/BFD simultaneously even if UE supports both option A and option B-1-1. On the other hand, network does not need to configure both option A and B-1-1 simultaneously. For instance, if network wants UE to perform L1 operations based CD-SSB (B-1-1), network doesn’t need to configure CSI-RS based L1 operation. This is doable since anyway it is per-UE configuration. If network wants UE to rely on CSI-RS for L1 operation, network can configure CSI-RS for each candidate BWP and not configure CD-SSB as RLM-RS. Thus, this issue can be avoided.

Proposal 1: For UE supporting both option B-1-1 and option A, UE shall perform L1 measurement according to network configuration, there is no need to define additional requirements/ UE behaviour.

- UE supporting both option C and A

This issue is similar with above wherein UE supports both option A and B-1-1. Network shall either enable option A (by providing CSI-RS for all candidate BWP) or option C (by configuring BWP to contain either CD-SSB or NCD-SSB), but not both.

Proposal 2: For UE supporting both option C and option A, UE shall perform L1 measurement according to network configuration, there is no need to define additional requirements/ UE behaviour.

# Conclusion

In this contribution, we provide discussion on general apsect of BWP without restriction. After discussion, the following conclusions are provided:

Proposal 1: For UE supporting both option B-1-1 and option A, UE shall perform L1 measurement according to network configuration, there is no need to define additional requirements/ UE behaviour.

Proposal 2: For UE supporting both option C and option A, UE shall perform L1 measurement according to network configuration, there is no need to define additional requirements/ UE behaviour.

# References

- R4-2321607, WF on BWP without restriction, vivo, Vodafone

- R4-2317337, WF on support for bandwidth part operation without restriction, vivo

- R4-2314339, WF on support for bandwidth part operation without restriction, vivo

- RP-232677, Further revised WID: Complete the specification support for BandWidth Part operation without restriction in NR, Vodafone, vivo