3GPP TSG-RAN WG4 Meeting #110 						                     R4-2400387

Athens, Greece, February 26 – March 1, 2024

Agenda item:	8.16.1.1.1

Source:	Apple

Title:	Discussion on general aspects and scenarios of L1/L2 based inter-cell mobility

Document for:	Discussion

# Introduction

General aspects and scenarios for LTM have been discussed widely for several meetings. The latest agreement can be found in the approved WF [1]. There are still some remaining issues that need to be addressed. In this contribution, we provide further discussion on the open issues.

# Discussion

## Sub-topic 1-2 PDCCH-order RACH on neighbor cell

Issue 1-2-1-2: The value of additional time for DL synchronization when needed in the delay requirements for PDCCH ordered RACH before cell switch command

<Agreement>:

- For FR2, one Tssb delay is always assumed before UE transmit PDCCH-ordered RACH.

- For FR1, when TCI state associated the PDCCH-order RACH has not been activated,

  - one Tssb delay is always assumed before UE transmit PDCCH-ordered RACH.

- For FR1, when TCI state associated the PDCCH-order RACH is activated,

  - If L1-RSRP measurement delay is less than or equal to 160ms, Tssb is not needed. UE is required to meet the UL Tx timing accuracy requirements

  - If L1-RSRP measurement delay is more than 160ms, UE is allowed to transmit PDCCH-ordered RACH with or without one Tssb delay. No UE UL Tx timing accuracy requirement will be defined.

Issue 1-2-1-3: The value of additional time for RF/BB preparation and RF re-tuning: when PRACH bandwidth is not within any of the configured UL BWPs of any active serving cell

Thursday ad-hoc agreement:

  - For the case of PRACH bandwidth not within any of the configured UL BWPs of any active serving cell

    - Introduce UE capability to report the time needed for RF/BB preparation and RF retuning, down-select from [1ms, 3ms, 5ms, 10ms].

As can be seen, additional time for DL synchronization when needed in the delay requirements for PDCCH ordered RACH before cell switch command was agreed in RAN4#109. RAN4 also agreed in previous meetings that delay requirement for PDCCH ordered RACH should still be defined in RAN1. For information, the latest delay defined in TS38.213 is as following:

If a random access procedure is initiated by a PDCCH order, the UE, if requested by higher layers, transmits a PRACH in the selected PRACH occasion, as described in [11, TS 38.321], for which a time between the last symbol of the PDCCH order reception and the first symbol of the PRACH transmission is larger than or equal to NT,2+TBWPswitchDelay+∆Delay+Tswitch+TSSB+∆RF/BB preparation msec, where - NT,2 is a time duration of N2 symbols corresponding to a PUSCH preparation time for UE processing capability 1 [6, TS 38.214] assuming μ corresponds to the smallest SCS configuration between the SCS configuration of the PDCCH order and the SCS configuration of the corresponding PRACH transmission - TBWPswitchDelay=0 if the active UL BWP does not change, or if a cell indicator field in the PDCCH order indicates a non-serving cell [5, TS 38.212], and TBWPswitchDelay is defined in [10, TS 38.133] otherwise - ∆Delay=0.5 msec for FR1 and ∆Delay=0.25 msec for FR2 - Tswitch is a switching gap duration as defined in [6, TS 38.214] - TSSB=0 if a cell indicator field in the PDCCH order indicates a serving cell or if cell indicator field is not present, and TSSB is defined in [10, TS 38.133] otherwise - ∆RF/BB preparation=0 if a cell indicator field in the PDCCH order indicates a serving cell or if cell indicator field is not present, and ∆RF/BB preparation is defined in [10, TS 38.133] otherwise

Note that in RAN1 spec they simply refer to RAN4 spec for values of TSSB and ∆RF/BB preparation. However, currently they are not defined in TS38.133 yet.

Observation 1: in RAN1 PDCCH ordered based RACH delay requirements, TSSB, TBWPswitchDelay and ∆RF/BB preparation are referred to TS38.133. However, they haven’t been specified in TS38.133 yet.

Proposal 1: explicitly specify TSSB, TBWPswitchDelay and ∆RF/BB preparation  in TS38.133.

Regarding additional capability for ∆RF/BB preparation , if RAN4 decides to reduce capability signalling overhead, we would like to keep [5ms, 10ms]. However, for the sake of flexibility we are also fine with two bits signalling, i.e. keeping four candidate values unchanged.

## Topic #5: LTM – UE feature

Issue 5-1-1: Capability of supporting RTD>CP

< Way Forward> FFS the following proposals

  - Proposal 1 (MTK):

    - The capability of supporting RTD>CP can be applicable to inter-frequency L1-RSRP measurement. It needs FR1/FR2 differentiation.

  - Proposal 2 (Xiaomi):

    - RAN4 to clarify the meaning of UE capability of supporting RTD>CP

      - For UE capable of RTD>CP, it is assumed that there is at least one spare FFT module used for intra-frequency L1-RSRP measurement on neighbour cells.

    - In the UE capability of RTD>CP, it is necessary to indicate the available number of FFT module used for intra-frequency neighbour cell measurement.

We support to extend capability of supporting RTD>CP to cover inter-frequency scenario. We are fine with FR1/FR2 differentiation. Regarding number of FFT module mentioned in proposal 2, we prefer not to touch such detail in UE feature discussion. In fact, this has already been reflected in UE capability regarding number of cells/SSBs to measure. RAN1 also asked RAN4 to discuss these capabilities. Technically, for UE support RTD>CP, the number of cells can be measured simultaneously depends on how many cells are outside CP window. For instance, if UE indicates support of 5 cells for L1 measurement and all the 5 cells are within CP, thus UE may still be able to support some additional cells for L1 measurement using the additional FFT module. This is more like a dynamic UE capability. Depending on different scenarios UE may be able to support more cells. Considering core part of this work item has been closed, we think it is better to focus on existing agreement and postpone optimization in future release.

Proposal 2: extend capability of supporting RTD>CP to cover inter-frequency.

Proposal 3: no need to let UE indicate number of FFT module. RAN4 only needs to discuss capability for number of cell/SSB for L1 measurement.

Issue 5-1-2: Capability for inter-f L1 measurement without gap

< Way Forward> FFS the following proposal

  - Proposal 1 (MTK):

    - Discuss the capability for supporting inter-frequency L1-RSRP measurement without gap in RAN1 to avoid duplicated discussion.

Since this is not covered by RAN1 feature [2], RAN4 needs to introduce this UE capability.

Proposal 4: introduce UE capability for inter-frequency L1 measurement without gap, as it is not covered by RAN1 feature.

Issue 5-1-3: Number of cells/resources supported

< Way Forward> FFS the following proposal

  - Proposal 1 (MTK):

    - The capability “number of cells supported to measure per frequency layer” indicates the number of neighbor cells UE supports to perform L1-RSRP measurement on per frequency layer. The minimum value reported is 2. This capability can be reported per band.

    - Discuss the detail of the capability “number of neighbor cells supported to measure per frequency layer” in RAN4 UE feature list.

Actually, RAN1 also asked RAN4 to discuss related UE capabilities [3]:

To RAN WG4 ACTION: For RAN1 FG 45-1 under the NR_Mob_enh2 WID, RAN1 respectfully asks RAN4 to discuss the following UE capabilities - Maximum number of measured candidate cells for all periodic reports, activated semi-persistent reports, and triggered aperiodic reports at a given time - For RTD <= CP - The max number of SSB resources configured to measure L1-RSRP within a slot across candidate cells - The max number of SSB resources configured for L1-RSRP across all the candidate cells - RTD > CP - The max number of SSB resources configured to measure L1-RSRP within a slot across candidate cells - The max number of SSB resources configured for L1-RSRP across all the candidate cells ACTION: For RAN1 FG 45-1a under the NR_Mob_enh2 WID, RAN1 respectfully asks RAN4 to discuss the following UE capabilities - Maximum number of measured candidate cells for all periodic reports, activated semi-persistent reports, and triggered aperiodic reports at a given time - With gap - The max number of SSB resources configured to measure L1-RSRP within a slot across candidate cells for intra- and inter-frequency L1-RSRP measurement - The max number of SSB resources configured across all the candidate cells for intra- and inter-frequency L1-RSRP measurement - Without gap - The max number of SSB resources configured to measure L1-RSRP within a slot across candidate cells for intra- and inter-frequency L1-RSRP measurement - The max number of SSB resources configured across all the candidate cells for intra- and inter-frequency L1-RSRP measurement

Since L1 measurement would be configured typically after L3 report (NW uses L3 measurement results to down-select neighbor cells for candidate LTM cells), it is rational to assume less number of cells/SSBs for L1 measurement compared to L3 measurement.

Another point is that for each candidate cell, UE can at most see two SSBs within a complete slot. Since the maximum number of cells configured for intra-frequency L1 measurement has already been as 8 by RAN1, the maximum number of SSB does not need to exceed 2*8=16.

One important thing we would like to highlight is that the new UE capability for neighbor cell measurement shall also counted in existing FG2-24, since the processing capability is shared among serving and neighbor cells. Similarly, the new UE capability shall also be reported per-band, and the total number across all bands shall be the maximum number reported per band by the UE.

Regarding RTD>CP, it is rational to consider smaller values since additional FFT is expected. However, so far there is no clear indication from NW to UE about whether UE can always assume RTD<CP. On the other hand, we don’t think from requirements point of view UE is required to accurately check actual RTD condition to change measurement algorithm from time to time.

As for with gap vs without gap, following above logic the number SSB is expected to be defined per-band. Then probably we don’t need to introduce another set of capability dedicated for gap or without gap measurement. Once NW wants to configure measurement (including both with and without gap), NW can just check the value reported per band by the UE so NW can know how many SSBs UE can support for L1 measurement for each target band.

Therefore, considering relation between L1 and L3 measurement, UE complexity, configuration flexibility and typical use cases, we propose the following capabilities:

Proposal 5: UE capability for number of SSBs to measure is defined as:

    - The max number of SSB resources configured to measure L1-RSRP within a slot across candidate cells is [1, 2, 4, 8, 16]

    - The max number of SSB resources configured for L1-RSRP across all the candidate cells is [1, 2, 4, 8, 16, 24, 32]

Proposal 6: UE capability for number of SSBs to measure is reported per band. It is also counted in existing L1 measurement capability FG 2-24.

Issue 5-1-4: Interruption on DL symbols due to PDCCH-ordered RACH

< Way Forward> FFS the following proposal

  - Proposal 1 (MTK):

    - Regarding the capability for interruption on DL symbols due to PDCCH-ordered RACH, the reported granularity is [per BC per target frequency per victim CC].

- Recommended WF

  - Need more discussion.

DL interruption due to RACH on other carrier is similar to interruption due to SRS switching. In SRS switching UE can indicate whether interruption is needed on each serving band. There is a band pair indication for that. Similarly, we think the same methodology can be reused there. Details of signalling design is up to RAN2.

Proposal 7: Regarding the capability for interruption on DL symbols due to PDCCH-ordered RACH, the reported granularity is [per band pair] (between the target band for RACH transmission and band under UE’s current band combo). Details of signalling design is up to RAN2.

Issue 5-1-5: Early ASN.1 decoding and validity/compliance check

< Way Forward> FFS the following proposal

  - Proposal 1 (Nokia):

    - RAN4 to discuss whether a UE supporting the capability for early ASN.1 decoding and validity/compliance check shall perform early ASN.1 decoding and validity/compliance check for all candidate cells or just for the (most probable) target cell.

  - Proposal 2 (Nokia, Ericsson): The UE supporting capability for early ASN.1 decoding and validity/compliance check can perform these steps before the cell switch command only for the candidate cell for which TCI state is activated and/or early RACH is initiated by PDCCH order.

  - Proposal 3 (MTK): Use TCI state activation to trigger early ASN.1 decoding and validity/compliance check on the candidate cell with the following conditions

    - NW activates TCI state(s) from only one candidate cell

    - Not to consider CA.

In general, we think all three proposals are reasonable. We would to like to polish the wording as follow:

Proposal 8: introduce an optional UE capability for Early ASN.1 decoding and validity/compliance check. For UE supporting the capability, TLTM-RRC-processing = 0 in cell switch delay requirement, provided UE has received TCI activation or PDCCH order to trigger RACH on target cell more than 10ms ago.

# Conclusion

In this contribution, we provide further discussion on general aspects and scenarios for LTM. After discussion, the following conclusions are provided:

Observation 1: in RAN1 PDCCH ordered based RACH delay requirements, TSSB, TBWPswitchDelay and ∆RF/BB preparation are referred to TS38.133. However, they haven’t been specified in TS38.133 yet.

Proposal 1: explicitly specify TSSB, TBWPswitchDelay and ∆RF/BB preparation  in TS38.133.

Proposal 2: extend capability of supporting RTD>CP to cover inter-frequency.

Proposal 3: no need to let UE indicate number of FFT module. RAN4 only needs to discuss capability for number of cell/SSB for L1 measurement.

Proposal 4: introduce UE capability for inter-frequency L1 measurement without gap, as it is not covered by RAN1 feature.

Proposal 5: UE capability for number of SSBs to measure is defined as:

    - The max number of SSB resources configured to measure L1-RSRP within a slot across candidate cells is [1, 2, 4, 8, 16]

    - The max number of SSB resources configured for L1-RSRP across all the candidate cells is [1, 2, 4, 8, 16, 24, 32]

Proposal 6: UE capability for number of SSBs to measure is reported per band. It is also counted in existing L1 measurement capability FG 2-24.

Proposal 7: Regarding the capability for interruption on DL symbols due to PDCCH-ordered RACH, the reported granularity is [per band pair] (between the target band for RACH transmission and band under UE’s current band combo). Details of signalling design is up to RAN2.

Proposal 8: introduce an optional UE capability for Early ASN.1 decoding and validity/compliance check. For UE supporting the capability, TLTM-RRC-processing = 0 in cell switch delay requirement, provided UE has received TCI activation or PDCCH order to trigger RACH on target cell more than 10ms ago.

# References

- R4-2321621, WF on NR mobility enhancements (part 1), MediaTek Inc.

- R1-2312572, Updated RAN1 UE features list for Rel-18 NR after RAN1#115, RAN1

- R1-2312574, LS on Rel-18 RAN1 UE features list for NR after RAN1#115, RAN1

- R4-2317330, WF on NR mobility enhancements (part 1), MediaTek Inc.

- R1-2310694, LS on Rel-18 higher-layers parameter list, RAN1

- R4-2314453, WF on NR mobility enhancements (part 1), MediaTek Inc.

- R4-2314454, Reply LS on PDCCH order RACH on neighbour cell, CATT

- R4-2306395, WF on NR Mobility Enhancements RRM requirements (part 1), MediaTek Inc.

- R4-2303175, WF on NR Mobility Enhancements RRM requirements (part 1), MediaTek Inc.

- R4-2220403, WF on L1/L2 inter-cell mobility, MediaTek Inc.