3GPP TSG-RAN WG4 Meeting # 110	R4-2400409

# Athens, Greece, 26th Feb – 1st Mar, 2024

Source: 	Apple

Title: 	on RF requirement for intra-band non-collocated CA/EN-DC

Agenda Item:	8.7.1

Document for:	Discussion

# Introduction

CR on how to implement RAN2 signalling has been agreed in the last RAN4 meeting [1, 2]. This contribution further addresses the open issue for intra-band non-collocated NR CA/EN-DC.

# Discussion

Power imbalance or PSD imbalance?

In the current specification, “power spectral density imbalance” is used for 6dB between CCs for intra-band collocated CA. While for intra-band non-collocated CA, “power imbalance” is used in the requirement description. When the 2 CCs are unequal, power imbalance becomes unclear. It is proposed to change the power imbalance to power spectral density imbalance in the requirement description. For the tile of this requirement, it is fine to remain power imbalance as is.

Proposal 1: it is proposed to change “power imbalance” to “power spectral density imbalance”.

Requirement applicability

In the last RAN2 meeting, requirement applicability has been clarified for NR CA and EN-DC [1, 2] for Rel-18 onwards UE. The clarification seems assuming the UE always supporting nonCollocatedTypeNR-CA-r18 for CA and nonCollocatedTypeMRDC-r18 for EN-DC. It is true for NR CA case. While it is not the case for EN-DC. According to the incoming LS from RAN2 [3], it is stated that, the new RRC signaling would not be applied to the FDD-FDD inter-band EN-DC with overlapping or partially overlapping bands.

Agreements in RAN2

- For UEs supporting new capability, adopt default type2 for nonCollocatedTypeMRDC-r18.

- For UEs supporting new capability, adopt default type2 for nonCollocatedTypeNR-CA-r18.

- RAN2 agree to remove Editor’s note for MTTD RAN4 spec reference for Type1.

- the new RRC signalling would not be applied to the FDD-FDD inter-band EN-DC with overlapping or partially overlapping bands.

In short, requirement clarification should consider the following cases:

- Legacy UE supporting FDD-FDD inter-band EN-DC with overlapping or partially overlapping bands from R16.

  - Such UE is not capable of requirementTypeIndication-r18.

- UE supporting TDD-TDD inter-band EN-DC with overlapping or partially overlapping bands from R18.

  - Such UE is capable of requirementTypeIndication-r18.

  - Requirement applicability for such UE should consider whether nonCollocatedTypeMRDC-r18 is provided or not.

The current requirement applicability clarification does not well accommodate the legacy UE which do not support requirementTypeIndication-r18. Power imbalance requirement should be clarified such that it is applicable to UE not supporting requirementTypeIndication-r18 indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16 and UE supporting requirementTypeIndication-r18 indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16 but not provided with nonCollocatedTypeMRDC-r18.

Proposal 2: Power imbalance requirement should be clarified such that it is applicable to UE not supporting requirementTypeIndication-r18 indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16 and UE supporting requirementTypeIndication-r18 indicates that is capable of interBandMRDC-WithOverlapDL-Bands-r16 but not provided with nonCollocatedTypeMRDC-r18.

Corresponding CRs are also provide in [4-7] to address the above proposals 1&2.

# 3. Conclusion

This contribution presented our consideration on further clarification of the requirement for intra-band non-collocated NR CA/EN-DC. The following proposals are concluded.

Proposal 1: it is proposed to change “power imbalance” to “power spectral density imbalance”.

Proposal 2: Power imbalance requirement should be clarified such that it is applicable to UE not supporting requirementTypeIndication-r18 indicates that it is capable of interBandMRDC-WithOverlapDL-Bands-r16 and UE supporting requirementTypeIndication-r18 indicates that is capable of interBandMRDC-WithOverlapDL-Bands-r16 but not provided with nonCollocatedTypeMRDC-r18.

# 4. Reference

R4-2321995, Rel-18 CR for 38.101-3 New BS signalling implementation for non-collocated scenario.

R4-2322004, Rel-18 CR for 38.101-1 New BS signalling implementation for non-collocated scenario.

R2-2313889, LS to RAN4 on Intra-band non-collocated NR-CA. EN-DC

R4-2400410, Clarification on RF requirement for intra-band non-collocated EN-DC (R16), Apple

R4-2400411, Clarification on RF requirement for intra-band non-collocated EN-DC (R17), Apple

R4-2400412, Clarification on RF requirement for intra-band non-collocated EN-DC (R18), Apple

R4-2400413, Modification on power imbalance requirements, Apple