3GPP TSG-RAN WG4 Meeting #110															R4-2400436

Athens, GR, 26 Feb – 01 Mar, 2024

Agenda item:	8.3.2.6

Source:	Apple

Title:   	On Receive timing difference between different directions for NR FR2 multi-Rx chain DL reception

Document for:	Discussion

# 1	Introduction

At RAN4 meeting#109, the following two issues as captured in the WF [1] remained open:

Issue 3-1-1: Whether L1-RSRP GBBR requirements should be extended to RTD >CP if the UE supports RTD>CP capability

- Way forward: Companies are requested to bring further analysis

Issue 3-1-1: How to capture RTD applicability conditions in multi-RX WI

- Way forward: Further discuss in next meeting.

  - Option 1: This condition is mentioned for each relevant multi-RX RRM requirement such as scheduling/measurement restriction, dual TCI state switching, L1-RSRP for GBBR.

  - Option 2: In clause 3.6	“Applicability of requirements in this specification version,” it is stated that RTD < CP as a condition for the relevant requirements to apply

In addition, there were two proposals as to how capture the RTD < CP condition in 38.133 [2][3] although there was no agreement and thus the big CR [4] does not include this condition. However, without the RTD < CP condition, there could be misunderstanding about the specification. In this contribution, we continue to share our views.

# 2	Discussion

How to capture RTD applicability conditions in multi-RX WI

In the agreed big CR [4], there is no mention of RTD < CP as a condition for the requirements to apply. However, there is a clear need to mention it because of the implications on UE implementation and network deployment.

Proposal 1: RTD < CP should be captured in 38.133 as a condition for the relevant multi-RX RRM requirements to apply.

As for the two options, Option 1 requires modification of every such mention and is prone to errors while Option 2 provides an once-and-for-all modification. Therefore, we prefer Option 2. Also, the proposal in [3] is to capture the condition in Clause 7.6 Maximum Receive Timing Difference. However, we think Clause 7.6 is for CA/DC and should not be used to capture RTD for single carrier mode, which is the focus in R18. In terms of futureproofness when CA/DC case with multi-RX is added, we can further modify Clause 3.6.

Proposal 2: It is proposed to state in clause 3.6	“Applicability of requirements in this specification version.” that RTD < CP is the condition for the relevant multi-RX RRM requirements to apply.

A corresponding R17 CR is provided in [5].

Whether L1-RSRP GBBR requirements should be extended to RTD >CP if the UE supports RTD>CP capability

As we commented at the last meeting, any requirement that would require UE to support RTD>CP would increase UE implementation. That is why RAN4 reached the following conclusion in RAN4#108:

  - The requirements defined for multi-Rx are only applicable for receive timing difference (RTD) less than CP in Rel-18 multi-Rx WI.

    - RAN4 understands it is important to support RTD>CP and recommend to consider in future release.

As a result, any agreement on whether L1-RSRP GBBR requirements should be extended to RTD >CP if the UE supports RTD>CP capability should be made in future release.

Proposal 3: L1-RSRP GBBR requirements should not be extended to RTD >CP because of UE implementation impact. UE support of RTD>CP can be discussed in R19.

# 3	Conclusions

In this contribution, we make the following proposals.

Proposal 1: RTD < CP should be captured in 38.133 as a condition for the relevant multi-RX RRM requirements to apply.

Proposal 2: It is proposed to state in clause 3.6	“Applicability of requirements in this specification version.” that RTD < CP is the condition for the relevant multi-RX RRM requirements to apply.

Proposal 3: L1-RSRP GBBR requirements should not be extended to RTD >CP because of UE implementation impact. UE support of RTD>CP can be discussed in R19.

# 4	References

R4-2317427, "WF on FR2_multiRx_part2," Ericsson

R4-2318697, "draft CR on capturing RTD < CP as a condition for multi-RX RRM requirements," Apple

R4-2320725, " Draft CR MRTD requirements for Multi Rx," Nokia

R4-2321636, " Big CR to TS 38.133 for RRM requirements for NR FR2 multi-Rx chain DL reception," vivo, Ericsson

R4-2400437, "draft CR on capturing RTD < CP as a condition for multi-RX RRM requirements," Apple