3GPP TSG-RAN WG4 Meeting # 110	R4-2400460

Athens, GR, 26 Feb – 01 Mar, 2024

Agenda item:	8.13.2.2

Source:	Apple

Title:	Simulation results for MU-MIMO with advanced receiver

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#108 RAN4 concluded the Phase 1/ study on advanced receiver for MU-MIMO and the details of the study were captured in TR 38.878. In RAN4#109 the UE capability signaling and parameters for defining requirements in phase 2 were discussed and WF [1] was agreed. In this contribution we present our simulation results for Phase 2 of this WI.

# 2. Simulation Results

In [1] we agreed on some simulation assumptions for further evaluation for defining requirements for MU-MIMO with advanced receiver.

## Simulation results when MO is signaled (DCI index 1~5)

For the agreed simulation assumptions in [1], in Tables 1 and 2 we provide the simulation results with co-scheduled UE modulation order signaled for FDD and TDD modes respectively.

Table 1: Results with MO signaled for FDD

| Rank for target + Co-UE | Precoder selection | Channel Model | Ant Config/Corr | MCS | MO of co- UE | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | R-ML | MMSE-IRC |
| 1+1 | Random | TDLC300-100 | 2x2 ULA Med | 13 | QPSK | 16.9 | 26.0 |
|  |  |  | 2x2 ULA Low |  |  | 14.8 | 15.0 |
|  |  | TDLA30-10 | 2x4 ULA Low |  |  | 6.9 | 6.9 |
|  |  | TDLC300-100 | 2x4 ULA Med |  |  | 16.9 | 24.9 |
|  | Orthogonal | TDLC300-100 | 2x2 ULA Med |  |  | 16.7 | 20.6 |
|  |  |  | 2x4 ULA Med |  |  | 16.6 | 19.9 |
| 2+2 | Orthogonal | TDLA30-10 | 4x4 ULA Low |  |  | 10.9 | 13.0 |
|  |  |  | 4x4 XP Med |  |  | 28.7 | N/A |
|  |  |  | 4x4 ULA Low | 17 | 16QAM | 16.6 | 17.6 |
|  |  |  | 4x4 XP Med |  |  | N/A | N/A |

Table 2: Results with MO signaled for TDD

| Rank for target + Co-UE | Precoder selection | Channel Model | Ant Config/Corr | MCS | MO of co- UE | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | R-ML | MMSE-IRC |
| 1+1 | Random | TDLC300-100 | 2x2 ULA Med | 13 | QPSK | 16.9 | 31.1 |
|  |  |  | 2x2 ULA Low |  |  | 15.1 | 15.3 |
|  |  | TDLA30-10 | 2x4 ULA Low |  |  | 7.0 | 7.0 |
|  |  | TDLC300-100 | 2x4 ULA Med |  |  | 16.8 | 29.3 |
|  | Orthogonal | TDLC300-100 | 2x2 ULA Med |  |  | 16.7 | 21.6 |
|  |  |  | 2x4 ULA Med |  |  | 16.5 | 21.5 |
| 2+2 | Orthogonal | TDLA30-10 | 4x4 ULA Low |  |  | 11.0 | 13.5 |
|  |  |  | 4x4 XP Med |  |  | 28.7 | N/A |
|  |  |  | 4x4 ULA Low | 17 | 16QAM | 16.9 | 18.2 |
|  |  |  | 4x4 XP Med |  |  | N/A | N/A |

Observations based on the results:

- For 1+1 with Low antenna correlation, with 2RX or 4RX the performance with R-ML is very close to MMSE-IRC

- For 2+2 with Medium correlation, with 13QAM+QPSK – the SNR is very high with R-ML receiver

- For 2+2 with medium correlation with MCS17 +16QAM 70% of max TP is not achievable with R-ML receiver.

- The performance is slightly improved with orthogonal precoder compared to random precoder for 1+1 with medium ant correlation.

For 1+1 with Low antenna correlation, with 2RX or 4RX the performance with R-ML is very close to MMSE-IRC.

For 2+2 with Medium correlation, with 13QAM+QPSK – the SNR is very high with R-ML receiver

For 2+2 with medium correlation with MCS17 +16QAM 70% of max TP is not achievable with R-ML receiver.

The performance is slightly improved with orthogonal precoder compared to random precoder for 1+1 with medium ant correlation.

## Simulation results when MO is not signaled (DCI index 6)

For the simulation assumptions agreed in [1] we provide results with blind modulation order detection for FDD and TDD in tables below.

Table 3: Results with blind detection of MO for FDD

| Rank for target + Co-UE | Precoder selection | Channel Model | Ant Config/Corr | MCS | MO of co- UE | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | R-ML | MMSE-IRC |
| 1+1 | Random | TDLC300-100 | 2x2 ULA Med | MCS 13 | QPSK | 20.2 | 26.0 |
|  |  |  | 2x2 ULA Low |  |  | 14.9 | 15.0 |
|  |  | TDLA30-10 | 2x4 ULA Low |  |  | 6.9 | 6.9 |
|  |  | TDLC300-100 | 2x4 ULA Med |  |  | 19.9 | 24.9 |
|  |  | TDLC300-100 | 2x2 ULA Med | MCS 17 | 16QAM | 25.1 | N/A |
|  |  |  | 2x2 ULA Low |  |  | 18.5 | N/A |
|  |  | TDLA30-10 | 2x4 ULA Low |  |  | 10.5 | 10.5 |
|  |  | TDLC300-100 | 2x4 ULA Med |  |  | 24.4 | N/A |
|  | Orthogonal | TDLC300-100 | 2x2 ULA Med | MCS 13 | QPSK | 18.5 | 20.6 |
|  |  |  | 2x4 ULA Med |  |  | 18.1 | 19.9 |
|  |  |  | 2x2 ULA Med | MCS 17 | 16QAM | 22.8 | N/A |
|  |  |  | 2x4 ULA Med |  |  | 22.1 | N/A |
| 2+2 |  | TDLA30-10 | 4x4 ULA Low | MCS 13 | QPSK | 12.9 | 13.0 |
|  |  |  | 4x4 XP Med |  |  | N/A | N/A |
|  |  |  | 4x4 ULA Low | MCS 17 | 16QAM | 17.4 | 17.6 |
|  |  |  | 4x4 XP Med |  |  | N/A | N/A |

Table 4: Results with blind detection of MO for TDD

| Rank for target + Co-UE | Precoder selection | Channel Model | Ant Config/Corr | MCS | MO of co- UE | SNR @ 70% Max TP |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | R-ML | MMSE-IRC |
| 1+1 | Random | TDLC300-100 | 2x2 ULA Med | MCS 13 | QPSK | 20.3 | 31.1 |
|  |  |  | 2x2 ULA Low |  |  | 15.2 | 15.3 |
|  |  | TDLA30-10 | 2x4 ULA Low |  |  | 7.0 | 7.0 |
|  |  | TDLC300-100 | 2x4 ULA Med |  |  | 20.2 | 29.3 |
|  |  | TDLC300-100 | 2x2 ULA Med | MCS 17 | 16QAM | 25.4 | N/A |
|  |  |  | 2x2 ULA Low |  |  | 18.8 | N/A |
|  |  | TDLA30-10 | 2x4 ULA Low |  |  | 10.7 | 10.7 |
|  |  | TDLC300-100 | 2x4 ULA Med |  |  | 25.3 | N/A |
|  | Orthogonal | TDLC300-100 | 2x2 ULA Med | MCS 13 | QPSK | 18.5 | 21.6 |
|  |  |  | 2x4 ULA Med |  |  | 18.5 | 21.5 |
|  |  |  | 2x2 ULA Med | MCS 17 | 16QAM | 22.9 | N/A |
|  |  |  | 2x4 ULA Med |  |  | 23.2 | N/A |
| 2+2 |  | TDLA30-10 | 4x4 ULA Low | MCS 13 | QPSK | 13.3 | 13.5 |
|  |  |  | 4x4 XP Med |  |  | N/A | N/A |
|  |  |  | 4x4 ULA Low | MCS 17 | 16QAM | 17.8 | 18.2 |
|  |  |  | 4x4 XP Med |  |  | N/A | N/A |

Observations:

- For 1+1 with Low antenna correlation, with 2RX or 4RX the performance with R-ML is very close to MMSE-IRC

- Performance with orthogonal precoder is better than random precoder

- For 2+2 with low correlation, performance with R-ML with BD-MO is not significantly better than MMSE-IRC

- For 2+2 with Medium correlation, R-ML receiver with BD MO cannot reach max TP and there is significant performance degradation

For 1+1 with Low antenna correlation, with 2RX or 4RX the performance with R-ML is very close to MMSE-IRC.

Performance with orthogonal precoder is better than random precoder

For 2+2 with low correlation, performance with R-ML with BD-MO is not significantly better than MMSE-IRC

For 2+2 with Medium correlation, R-ML receiver with BD MO cannot reach max TP and there is significant performance degradation

# 3. Conclusion

In this paper, we provide simulation results for requirements with advanced receiver for MU-MIMO. Our observations are captured below:

- For 1+1 with Low antenna correlation, with 2RX or 4RX the performance with R-ML is very close to MMSE-IRC.

For 2+2 with Medium correlation, with 13QAM+QPSK – the SNR is very high with R-ML receiver

For 2+2 with medium correlation with MCS17 +16QAM 70% of max TP is not achievable with R-ML receiver.

The performance is slightly improved with orthogonal precoder compared to random precoder for 1+1 with medium ant correlation.

For 1+1 with Low antenna correlation, with 2RX or 4RX the performance with R-ML is very close to MMSE-IRC.

Performance with orthogonal precoder is better than random precoder

For 2+2 with low correlation, performance with R-ML with BD-MO is not significantly better than MMSE-IRC

For 2+2 with Medium correlation, R-ML receiver with BD MO cannot reach max TP and there is significant performance degradation

# Reference

- R4-2321114, “WF on the advanced receiver for MU-MIMO scenario”, China Telecom, Apple.