3GPP TSG-RAN4 Meeting #110	R4-2400513

# Athens, Greece, 26th February – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0697 | rev | - | Current version: | 16.18.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_newRAT-Core) Correction to CA A-MPR requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2024-02-15 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In the A-MPR requirements the CA NS values are referencing the single carrier clauses for the emission requirements. This is incorrect and it needs to be referred to the CA requirements instead. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update requirements clauses to the CA clauses instead of single carrier. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Requirement remains incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.521-2 CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### < Unchanged sections omitted >

### 6.2A.3	UE maximum output power with additional requirements for CA

#### 6.2A.3.1	General

Additional emission requirements can be signalled by the network with network signalling value indicated by the field additionalSpectrumEmission. To meet these additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in clause 6.2A.1. Unless stated otherwise, an A-MPR of 0 dB shall be used. Unless otherwise stated, the allowed total back off is maximum of A-MPR and MPR specified in clause 6.2A.2.

For intra-band contiguous aggregation with the UE configured for transmissions on two serving cells, the maximum output power reduction specified in Table 6.2A.3.1-1 is allowed for all serving cells of the applicable uplink contiguous CA configurations.

Table 6.2A.3.1-1 specifies the additional requirements and allowed A-MPR with corresponding network signalling label and operating band. The mapping between network signalling labels and the additionalSpectrumEmission IE defined in TS 38.331 [13] is specified in Table 6.2A.3.1-2. Unless otherwise stated, the allowed total back off is maximum of A-MPR and MPR specified in clause 6.2A.2.

Table 6.2A.3.1-1: Additional maximum power reduction (A-MPR)

| Network Signalling value | Requirements (clause) | NR Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| CA_NS_200 |  |  |  |  | N/A |
| CA_NS_201 | 6.5A.3.2.2 | n258 |  |  | 6.2A.3.2 |
| CA_NS_202 | 6.5A.3.2.3 | n257, n258 |  |  | 6.2A.3.3 |
| CA_NS_203 | 6.5A.3.2.4 | n258 |  |  | 6.2A.3.4 |
| NOTE: CA_NS_201 is obsolete, the associated additional spurious emission requirements are not applicable. |  |  |  |  |  |

Table 6.2A.3.1-2: Value of additionalSpectrumEmission

| NR Band | Value of additionalSpectrumEmission / NS number |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n257 | CA_NS_200 | CA_NS_202 |  |  |  |  |  |  |
| n258 | CA_NS_200 | CA_NS_201 | CA_NS_202 | CA_NS_203 |  |  |  |  |
| n259 | CA_NS_200 |  |  |  |  |  |  |  |
| n260 | CA_NS_200 |  |  |  |  |  |  |  |
| n261 | CA_NS_200 |  |  |  |  |  |  |  |
| NOTE 1: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [13]. NOTE 2: CA_NS_201 is obsolete, the associated additional spurious emission requirements are not applicable. |  |  |  |  |  |  |  |  |

#### < End of changes >