3GPP TSG-RAN WG4 Meeting # 110	R4-2400533

Athens, GR, 26 Feb – 01 Mar, 2024

Agenda item:	9.4.5

Source:	MediaTek Inc.

Title:	Discussion on UE requirements for IoT-NTN enhancement

Document for:	Discussion

- Introduction

In RAN4#109 meeting, a WF for IoT-NTN enhancement requirements was agreed [1]. In this contribution, we provide our views on remaining issues.

- Discussion

RAN4 agreed not to introduce PUSCH requirements with HARQ disabled in the last meeting.

Issue 3-1: Whether to define PUSCH requirements with HARQ disabled - Agreement: - Do not define PUSCH requirements with HARQ disabled.

However, there is no conclusion for PDSCH requirements with HARQ disabled.

Issue 2-1: Whether to define PDSCH requirements with HARQ disabled? - Agreement: - Continue the discussion in the next meeting.

For IoT-NTN enhancement, RAN1/RAN2 did not increase the HARQ process number but only introduced disable HARQ functionality to mitigate impact of HARQ stalling on UE data rates. In legacy PDSCH tests, the test metric is throughput which is calculated based on HARQ feedback. The maximum HARQ process is 1 (mandatory) or 2 (with optional capability) for NB-IoT and 2 for eMTC CE Mode B. With disabled HARQ feedback for downlink transmission, it is difficult to have statistics for throughput since there is no HARQ ACK/NACK feedback. TE has no idea whether UE correctly decode PDSCH or not, which means that the UE performance cannot be verified. Also, the operation of disabled HARQ feedback is to turn off the ACK/NACK for HARQ process and not related to the UE demodulation performance. Therefore, we propose not to introduce UE demodulation performance requirement for the operation of disabled HARQ feedback.

Observation 1: The maximum HARQ process is 1 (mandatory) or 2 (with optional capability) for NB-IoT and 2 for eMTC CE Mode B. If there is no ACK/NACK feedback from UE with disabled HARQ feedback, TE cannot have throughput statistics to verify the UE demodulation performance.

Observation 2: The operation of disabled HARQ feedback is to turn off the ACK/NACK for HARQ process. It is kind of some functionality and not related to the UE demodulation performance.

Proposal: Do not introduce demodulation requirements for operation with disabled HARQ feedback for NB-IoT/eMTC.

- Conclusion

In this contribution, we provide our views about UE demodulation requirements for operation with disabled HARQ feedback for NB-IoT/eMTC.

Observation 1: The maximum HARQ process is 1 (mandatory) or 2 (with optional capability) for NB-IoT and 2 for eMTC CE Mode B. If there is no ACK/NACK feedback from UE with disabled HARQ feedback, TE cannot have throughput statistics to verify the UE demodulation performance.

Observation 2: The operation of disabled HARQ feedback is to turn off the ACK/NACK for HARQ process. It is kind of some functionality and not related to the UE demodulation performance.

Proposal: Do not introduce demodulation requirements for operation with disabled HARQ feedback for NB-IoT/eMTC.

- Reference

- R4-2321145, “WF on [109][332] IoT_NTN_Demod”, MediaTek