3GPP TSG-RAN WG4 Meeting # 110	R4-2400576

# Athens, Greece, February 26 – March 1, 2024

Source:	LG Electronics

Title:	Simulation results for SL-U PSSCH demodulation

Agenda item:	8.22.4

Document for:	Information

Introduction

In the last RAN4#109 meeting, companies discussed whether to define the PSSCH demodulation performance for SL-U. In this contribution, some initial simulation results are delivered.

Discussion

  - Simulation assumptions

- Table 1: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Active cell(s) |  |  | None |
| SL transmission model |  |  | As specified in B.5 |
| SL model parameters | SL transmission duration values | Slots | {2,4,6,7} |
|  | Occupied OFDM symbols in slot other than the first slot of the transmission duration | Symbols | 12 (Except AGC, TxRx switching symbols) |
|  | Occupied OFDM symbols in the first slot of the transmission duration | Symbols | {6,7, …,12} |
|  | SL transmission period | ms | 5 |
|  | LBT failure probability (pLBT) |  | 0.25 |
| Sidelink UE 1 | Sidelink transmissions |  | PSCCH + PSSCH |
|  | PSSCH DMRS pattern (Note 1) |  | {2,3} |
|  | Number of interlace |  | 1 with RB index 0, 5, 10, …, 50 |
|  | Timing offset (Note 2) | s | CP/2-12*64*Tc |
|  | Frequency offset (Note 3) | Hz | +650 |
|  | Synchronization |  | GNSS or GNSS-equivalent |
|  | Antenna configuration |  | 1x2 Low |
| PSFCH resource period |  | Slot | 4 |
| MinTimeGapPSFCH |  | Slot | 3 |
| Note 1: {x, y}: x and y means the number of DMRS symbols for slot with PSFCH transmission and without PSFCH transmission, respectively. Note 2: Time offset of transmitted Sidelink UE signal with respect to GNSS referring timing. Note 3: Frequency offset of transmitted Sidelink UE signal with respect to GNSS reference frequency. |  |  |  |

- Table 2: PSSCH Reference Channel for SL-U

| Parameters |  | Unit | Value |
| --- | --- | --- | --- |
| Reference channel |  |  | R.PSSCH.2-2.1 |
| Channel bandwidth |  | MHz | 20 |
| Subcarrier spacing |  | kHz | 30 |
| Allocated resource blocks |  | RB | 11 |
| CP-OFDM symbols for slot with PSFCH (Note 1) |  |  | 9 |
| CP-OFDM symbols for slot without PSFCH (Note 2) |  |  | 12 |
| Modulation order |  |  | 16QAM |
| MCS index |  |  | 11 |
| Number of MIMO layers |  |  | 1 |
| Number of DMRS REs |  |  | 15 |
| Number of REs for SCI format 1-A |  |  | 240 |
| 2nd stage SCI format 2-A configuration | Payloads | Bits | 68 |
|  | α |  | 1 |
|  | βoffset |  | 2 |
| Overhaed for TBS determination |  |  | 0 |
| Transport Block Size for slot with PSFCH |  | Bits | 808 |
| Transport Block Size for slot without PSFCH |  | Bits | 1416 |
| Transport block CRC |  |  | 24 |
| Maximum number of HARQ transmissions |  |  | 1 |
| Binary Channel Bits for slots with PSFCH |  |  | 2232 |
| Binary Channel Bits for slots without PSFCH |  | Bits | 3792 |
| Note 1: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC), PSFCH symbols, and guard symbols. Note 2: OFDM symbols is for PSCCH/PSSCH transmission not including first symbol (AGC) and guard symbols. |  |  |  |

Table 3: Minimum performance for SL-U

| Test num. | Reference channel | Bandwidth (MHz)/ Subcarrier spacing(kHz) | Modulation format and code rate | Propagation condition | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | PSSCH BLER (%) | SNR(dB) of PSSCH |
| 1 | R.PSSCH.2-2.1 | 20 / 30 | 16QAM, 0.37 | TDLA30-1500 | 10 % | TBD |

  - Simulation results

Summary

In this contribution, we shared initial simulation results for SL-U PSSCH demodulation.

Reference

[1] R4-2321131, “WF on UE demodulation performance for NR SL evolution”, LG Electronics