3GPP TSG-RAN WG4#110	R4-2400642

Athens, Greece, February 26th – March 1st, 2024

Agenda item:	7.1.1.1

Source: 	Qualcomm France

Title: 	MSD for UL CA_n3B

Document for:	Approval

# Introduction

MSD analysis for UL CA n3B is provided in this contribution.

- Background

This UL CA combination with BCS4/5 was added into RAN4 work in RAN#102 [1]:

- Discussion

The following test point was used in the analysis. The BW’s are chosen so that PCC is the narrowest possible yet fulfilling entire passband together with SCC. The Total UL RB allocation is chosen as MIN(Number of UL RB’s for wider CC in REFSENS config, Max applicable number of UL RB’s which can be split exactly according to BW’s of PCC and SCC). In this case 50MHZ CC has 50RB’s in REFSENS config, and the closest applicable UL RB number below 50 which can be split for PCC/SCC is 48RB. Channels are spaced according to nominal channel spacing in CA, and UL RB positions are chosen so that IMD3 fits inside PCC DL, and IMD5 fits inside SCC DL.

| CA configuration | SCS (PCC/SCC) (kHz) | FC PCC/SCC UL | FC PCC/SCC DL | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3BX | 15/15 | 1722.5/ 1759.7 | 1817.5/ 1854.7 | 25MHz + 50MHz | 16 (RBSTART = 60) | 32 (RBSTART = 165) | TBD | TBD | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier center frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE X: Applicable only to BCS 4-5 |  |  |  |  |  |  |  |  |  |

- Conclusion

MSD analysis For UL CA_n3B was provided with the following proposals for MSD.

| CA configuration | SCS (PCC/SCC) (kHz) | FC PCC/SCC UL | FC PCC/SCC DL | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n3BX | 15/15 | 1722.5/ 1759.7 | 1817.5/ 1854.7 | 25MHz + 50MHz | 16 (RBSTART = 60) | 32 (RBSTART = 165) | 41.2 | 18.9 | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier center frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE X: Applicable only to BCS 4-5 |  |  |  |  |  |  |  |  |  |

- Reference

[1] RP-233223, “Wi Exception sheet for NR_CA_R18_Intra combo list”, Ericsson