3GPP TSG-RAN WG1 Meeting #110															R4-2400767

Athens, Greece, February 26th – March 1st,2024

Agenda item:			8.25.7

Source:	Moderator (Qualcomm)

Title:	Topic summary for [110][231] NR_mobile_IAB

Document for:	Information

# Introduction

This email discussion contains the summary of the proposals on the RRM core requirements maintenance and RRM performance requirements/tests to be defined for mobile IAB.

# Topic #1: RRM core maintenance and RRM performance

This sections discusses the proposals submitted to this meeting.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2401335 | Huawei, HiSilicon | Maintenance CR with some corrections |
| R4-2401606 | Qualcomm Incorporated | Proposal 1: The measurement performance requirements for mIAB-MT should be introduced in Clause 12.5B of TS 38.174. The requirements to be reused from TS 38.133 can be found in the list below: Clause 10.1.2 – FR1 RSRP Clause 10.1.3 – FR2 RSRP Clause 10.1.6 – RSRP reporting mapping Clause 10.1.7 – FR1 RSRQ Clause 10.1.8 – FR2 RSRQ Clause 10.1.11 – RSRQ reporting mapping Clause 10.1.12 – FR1 SINR Clause 10.1.13 – FR2 SINR Clause 10.1.16 – SINR report mapping Clause 10.1.19 – FR1 L1-RSRP Clause 10.1.20 – FR2 L1-RSRP Proposal 2: Introduce the following measurement accuracy test cases based on the corresponding tests from TS 38.133: - FR1 SS-RSRP accuracy test : Section A.6.7.1.1 - FR1 SS-RSRQ accuracy test: Section A.6.7.2.1 - FR1 SS-SINR accuracy test: Section A.6.7.3.1 - FR1 L1-SINR accuracy test: Section A.6.7.4.1 - FR2 SS-RSRP accuracy test : Section A.7.7.1.1 - FR2 SS-RSRQ accuracy test: Section A.7.7.2.1 - FR2 SS-SINR accuracy test: Section A.7.7.3.1 - FR2 L1-SINR accuracy test: Section A.7.7.4.1 Proposal 3: Reuse the test configurations in Annex G.1 for the mIAB-MT RRM tests. Proposal 4: The following CR work split should be discussed and agreed: Requirements/Tests; Company. Measurement Accuracy requirements. Measurement Accuracy test cases. Handover tests. Active TCI switch tests. Measurement procedures – intra-frequency measurement tests. Transmit timing tests. Signaling characteristics. |
| R4-2402054 | Dell Technologies | ## Observation 1: For mobile IAB, the 4-step RACH and 2-step RACH shall be supported. - Observation 2: The 2-step RACH not only reduces the latency involved in the overall RA procedure but also reduces the control signaling overhead. Proposal 1: It is suggested to reuse UE test requirements based on 38.133 for mobile IAB NR standard alone. Where the clause A6.3 for both 4-step RACH and the 2-step RACH type with contention and non-contention requirements can be used for FR1 and the clause A 7.3 can be used for FR2-1. |
| R4-2402561 | Nokia, Nokia Shanghai Bell | In the paper we identified an unclarity in the applicability of existing IAB-MT requirements to mIAB-MT. We also found out that Timer accuracy and Uplink spatial relation switch requirements are missing. The following Observations and Proposals were made: Observation 1: Some of the IAB-MT RRM Core requirements were not updated for mIAB-MTs (e.g., RACH, Timing) but it is not specified that they are still applicable to mIAB-MTs. Proposal 1: Add new clause 4.7.3 in TS 38.174 to clarify the applicability of legacy IAB-MT RRM requirements to mIAB-MT. Proposal 2: Random access requirements in Clause 12.1.1.2, TS 38.174 are applicable to mIAB-MT. Proposal 3: Existing timing requirements in Section 12.2, TS 38.174 are applicable to mIAB-MT. Proposal 4: RAN4 to define mIAB-MT timer accuracy requirements by reusing UE requirements from Clause 7.2 of TS 38.133. Proposal 5: RAN4 to introduce Uplink spatial relation switch delay requirements for mIAB-MTs by reusing UE requirements in Section 8.12 from TS 38.133. |
| R4-2402562 | Nokia, Nokia Shanghai Bell | - Draft CR with corrections based on the proposals in R4-2402561 Changes: - Applicability of RRM requirements for mIAB-MT is introduced - mIAB-MT timer accuracy requirement is introduced - Uplink spatial relation switch delay requirement for IAB.MT is introduced |
| R4-2402563 | Nokia, Nokia Shanghai Bell | Observation 1: mIAB-MT requirements are expected to follow legacy IAB-MT requirements form TS 38.174. Transmit timing test cases are already defined for legacy IAB-MTs. Proposal 1: Legacy NR IAB-MT Transmit Timing Tests for FR1/FR2-1 (Clauses G.2.2.1.1/2 of TS 38.174) shall be applicable and reused for mIAB-MTs. Observation 2: Core requirements on Uplink spatial relation switch are missing for IAB-MT but shall be introduced. Proposal 2: RAN4 to define MAC-CE based uplink spatial relation switch and RRC based spatial relation switch test cases by reusing UE test cased defined in A.5.5.9.1 and A.5.5.9.1 of TS 38.133. Proposal 3: Define mIAB-MT test cases only for 4-step RA contention based RA for FR1/FR2-1 for NR standalone by reusing corresponding UE test cases from TS 38.133. |
| R4-2402824 | Ericsson | This draft CR defines all the HO test cases for mIAB-MT in annex G of TS 38.174 as agreed in the WF in R4-2321534. |

## Open issues summary

The open issues are discussing some RRM maintenance CRs and continuing the mIAB-MT RRM performance discussion based on the WF (R4-2321534) agreed in RAN4#109

- RRM core maintenance CR

- Applicability of legacy IAB-MT RRM requirements to mIAB-MT

- Random access requirements applicability

- Timing requirements applicability

- Timer requirements applicability

- UL spatial relation switching requirements

- RRM core maintenance CR

- 2-step RACH requirements

- Measurement performance requirements

- Measurement accuracy test cases

- Transmit timing test handling

- UL spatial relation switching test

- Test configurations

- HO test cases

- CR split

### Sub-topic 1-1

RRM core requirements maintenance

The CR in R4-2401335 contains the following proposals:

| Reason for change: | - According to the speicfication suffix information in 4.12, mIAB needs to support both local area IAB-MT requirements and additional requirements with suffixes B. However, according to agreements reached in R4-2310085, No requirements for inter-frequency RRC Re-establishment and RRC release with re-direction. Thus, clarifications about applicability are needed. - There are typos in the spec. |
| --- | --- |
| Summary of change: | - Clarify in 12.1.1.1 (with mIAB dedicated section 12.1.1.1B) and 12.1.1.3 (No requirements for mIAB) that requirements do not apply to mIAB-MT. - Typo correction. |

Issue 1-1: Correction CR in R4-2401335

- Proposals

  - Option 1: CR is agreeable

  - Option 2: CR should be revised

  - Option 3: CR is not agreeable, the changes are not needed

- Recommended WF

  - To be discussed

Please provide any comments for any necessary changes

### Sub-topic 1-2

RRM Maintenance -

Issue 1-2: Clarficiations to applicability of legacy requirements

- Proposals

  - Option 1: Add new clause 4.7.3 in TS 38.174 to clarify the applicability of legacy IAB-MT RRM requirements to mIAB-MT.

- Recommended WF

  - Option 1

Please provide any comments for any other changes

### Sub-topic 1-3

RRM maintenance

Issue 1-3: Random access requirements applicability

- Proposals

  - Option 1: Random access requirements in Clause 12.1.1.2, TS 38.174 are applicable to mIAB-MT.

- Recommended WF

  - Agree Option 1

Please provide any comments for any other changes

### Sub-topic 1-4

RRM maintenance

Issue 1-4: Timing requirements

- Proposals

  - Option 1: Existing timing requirements in Section 12.2, TS 38.174  are applicable to mIAB-M

- Recommended WF

  - Option 1

### Sub-topic 1-5

RRM maintenance

Issue 1-5: Timer accuracy requirements

- Proposals

  - Option 1: RAN4 to define mIAB-MT timer accuracy requirements by reusing UE requirements from Clause 7.2 of TS 38.133

  - Option 2: Do not define timer accuracy requirements

- Recommended WF

  - To be discussed

### Sub-topic 1-6

RRM maintenance

Issue 1-6: UL spatial relation switch requirements

- Proposals

  - Option 1: RAN4 to introduce Uplink spatial relation switch delay requirements for mIAB-MTs by reusing UE requirements in Section 8.12 from TS 38.133

  - Option 2: others

- Recommended WF

  - To be discussed

### Sub-topic 1-7

Draft maintenance CR

A draft CR based on the proposals in Sub-topics 1-2 to 1-6 is proposed in R4-2402562

Issue 1-7: Draft CR for maintenance

- Proposals

  - Option 1: Agree R4-2402562

  - Option 2: CR needs changes

  - Option 3: CR is not agreeable

- Recommended WF

  - To be discussed

Please provide concrete comments if CR needs to be revised

### Sub-topic 1-8

2-step RACH requirements

It is proposed to introduce 2-step RACH test requirements for mIAB-MT

Issue 1-8: 2-step RACH

- Proposals

  - Option 1: Introduce 2-step RACH test requirements based on the corresponding UE test requirements in 38.133 (Clause A6.3 for FR1 and clause A7.3 for FR2-1)

  - Option 2: Only introduce 4-step contention based RA test based on the corresponding UE test in TS 38.133

- Recommended WF

  - To be discussed

### Sub-topic 1-9

Measurement performance requirements

Issue 1-9: Measurement performance requirements

- Proposals

  - Option 1: The measurement performance requirements for mIAB-MT should be introduced in Clause 12.5B of TS 38.174. The requirements to be reused from TS 38.133 can be found in the list below:

    - Clause 10.1.2 – FR1 RSRP

    - Clause 10.1.3 – FR2 RSRP

    - Clause 10.1.6 – RSRP reporting mapping

    - Clause 10.1.7 – FR1 RSRQ

    - Clause 10.1.8 – FR2 RSRQ

    - Clause 10.1.11 – RSRQ reporting mapping

    - Clause 10.1.12 – FR1 SINR

    - Clause 10.1.13 – FR2 SINR

    - Clause 10.1.16 – SINR report mapping

    - Clause 10.1.19 – FR1 L1-RSRP

    - Clause 10.1.20 – FR2 L1-RSRP

  - Option 2: others

- Recommended WF

  - Option 1

Please provide any comments if Option 1 should be modified

### Sub-topic 1-10

Measurement accuracy test cases

Issue 1-10: Measurement accuracy test cases

- Proposals

  - Option 1:

Introduce the following measurement accuracy test cases based on the corresponding tests from TS 38.133:

- FR1 SS-RSRP accuracy test : Section A.6.7.1.1

- FR1 SS-RSRQ accuracy test: Section A.6.7.2.1

- FR1 SS-SINR accuracy test: Section A.6.7.3.1

- FR1 L1-SINR accuracy test: Section A.6.7.4.1

- FR2 SS-RSRP accuracy test : Section A.7.7.1.1

- FR2 SS-RSRQ accuracy test: Section A.7.7.2.1

- FR2 SS-SINR accuracy test: Section A.7.7.3.1

- FR2 L1-SINR accuracy test: Section A.7.7.4.1

  - Option 2: others

- Recommended WF

  - Option 1

Please provide any comments if Option 1 should be modified

### Sub-topic 1-11

Transmit timing test

Issue 1-11: Transmit timing test handling

- Proposals

  - Option 1: Legacy NR IAB-MT Transmit Timing Tests for FR1/FR2-1 (Clauses G.2.2.1.1/2 of TS 38.174) shall be applicable and reused for mIAB-MTs.

  - Option 2: others

- Recommended WF

  - To be discussed

### Sub-topic 1-12

UL spatial relation switching

Issue 1-12:  UL spatial relation switching tests

- Proposals

  - Option 1: RAN4 to define MAC-CE based uplink spatial relation switch and RRC based spatial relation switch test cases by reusing UE test cased defined in A.5.5.9.1 and A.5.5.9.1 of TS 38.133.

  - Option 2: others

- Recommended WF

  - To be discussed

### Sub-topic 1-13

Test configurations

Issue 1-13: Test configurations

- Proposals

  - Option 1: Reuse the test configurations in Annex G.1 for the mIAB-MT RRM tests.

  - Option 2: changes/additions are needed to the test configuations

  - Option 3: discuss needed changes based on the draft CRs

  - Option 4: others

- Recommended WF

  - Take Option 1 as baseline, discuss changes/additions based on the draft CRs

### Sub-topic 1-14

HO test cases

Draft CR in R4-2402824 proposes all the HO test cases

Issue 1-14: HO test cases

- Proposals

  - Option 1: Draft CR is agreeable

  - Option 2: changes are needed

  - Option 3: postpone discussion to next meeting and consider all draft CRs at the same time

  - Option 4: others

- Recommended WF

  - To be discussed

### Sub-topic 1-15

CR split

A possible CR split is proposed in R4-2401606

Issue 1-11: CR split

- Proposals

  - Option 1: Agree the CR split in the table below

| Requirements/Tests | Company |
| --- | --- |
| Measurement Accuracy requirements |  |
| Measurement Accuracy test cases |  |
| Handover tests | Ericsson? |
| Active TCI switch tests |  |
| Measurement procedures – intra-frequency measurement tests |  |
| Transmit timing tests |  |
| Signaling characteristics |  |

  - Option 2: others

- Recommended WF

  - Option 1

Companies are invited to volunteer on drafting some of the CRs.