3GPP TSG-RAN4 Meeting #110	R4-2400844

# Athens, GR, Feb 26 - Mar 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-1 | CR | 0414 | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_ATG-Perf) BigCR to TS 38.141-1 Introduction of demodulation performance requirements for NR ATG BS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-Perf |  |  |  |  |  | Date: |  |  | 2024-03-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For ATG demodulation, Introduce BS manufacturer, applicability rule, performance requirements and FRC definition based on the endorsed CRs in RAN4#110, including: Tdoc; Title; Source; Note. R4-2401569; Draft CR on manufacturer and applicability rule of BS demodulation requirements for Rel-18 ATG; Samsung; 3.3, 4.6, 8.1.2.x1, 8.1.2.x2, 8.1.2.x3. R4-2402976; (NR_ATG-Perf) Draft CR for 38.141-1 on ATG PUSCH demodulation requirements and FRC table; Ericsson; 8.2.X, A.Y, A.Z. |
| Summary of change: |  | - For ATG demodulation, Introduce BS manufacturer, applicability rule, performance requirements and FRC definition |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Demod perf requirements for ATG BS will missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.3, 4.6, New clauses: 8.1.2.x1, 8.1.2.x2, 8.1.2.x3, 8.2.X, A.Y, A.Z |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of change>

3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AAS	Active Antenna System

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

ATG 		Air-To-Ground

AWGN	Additive White Gaussian Noise

<unchanged parts skipped>

## <Next change>

4.6	Manufacturer declarations

The following BS declarations listed in table 4.6-1, when applicable to the BS under test, are required to be provided by the manufacturer for the conducted requirements testing of the BS type 1-C and BS type 1-H.

For the BS type 1-H declarations required for the radiated requirements testing, refer to TS 38.141-2 [3].

Table 4.6-1 Manufacturer declarations for BS type 1-C and BS type 1-H conducted test requirements

| Declaration identifier | Declaration | Description | Applicability |  |
| --- | --- | --- | --- | --- |
|  |  |  | BS type 1-C | BS type 1-H |
| D.122 | PUSCH repetition type A | Declaration of support PUSCH repetition type A | x | x |
| D.1xx | Air-to-ground scenario | Declaration of air-to-ground scenario support, i.e., ATG support or no ATG support | x | x |

## <Next change>

8.1.2.1	Applicability of PUSCH performance requirements

8.1.2.x1	Applicability of PUSCH performance requirements for ATG scenario

Unless otherwise stated, PUSCH requirement tests in clause 8.2.1, 8.2.2 and 8.2.3 with 1Tx test cases shall apply only for the BS declared to be supported (see D.1XX in table 4.6-1).

Unless otherwise stated, PUSCH requirement tests in clause [8.2.x] shall apply only for the BS declared to be supported (see D.1XX in table 4.6-1).

##### 8.1.2.x1.1	Applicability of requirements with different UL-DL patterns

Unless otherwise stated, for each subcarrier spacing declared to be supported, if BS supports multiple TDD UL-DL patterns including new TDD pattern (e.g., 30D4S6U, S= 14G) for ATG scenario, only one of the supported TDD UL-DL patterns shall be used for all tests.

Note: For PUSCH Performance test cases, FRCs are not expected to be defined for the special slots, unless otherwise stated.

##### 8.1.2.x1.2	Applicability of requirements for different MCSs

Unless otherwise stated, PUSCH requirement tests with 256QAM in clause 8.2.x shall apply only for the BS declared to be supported (see D.1XX in table 4.6-1).  A BS that declares to support 256QAM, and passes the test with 256QAM and the test with 64QAM in clause 8.2.1, the PUSCH requirement tests with 64QAM in clause 8.2.x can be skipped.

8.1.2.x2	Applicability of PUCCH performance requirements for ATG scenario

Unless otherwise stated, PUCCH requirement tests in clauses 8.3.1 to 8.3.6 shall apply only for the BS declared to be supported (see D.1XX in table 4.6-1).

8.1.2.x3	Applicability of PRACH performance requirements for ATG scenario

Unless otherwise stated, PRACH requirement tests in clauses 8.4.1.5 shall apply only for the BS declared to be supported (see D.1XX in table 4.6-1).

## <Next change>

8.2.X	Performance requirements for PUSCH for ATG

8.2.X.1	Definition and applicability

The performance requirement of PUSCH is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex A. The performance requirements assume HARQ re-transmissions. The performance requirements for 1Tx defined in 8.2.1, 8.2.2 and 8.2.3 are mandatory for a BS declaring support ATG scenario.

Which specific test(s) are applicable to BS is based on the test applicability rules defined in clause 8.1.2.x and clause 8.1.2.y.

The performance requirements for PUSCH are only applied for ATG Base Stations (Subject to declaration).

8.2.X.2	Minimum Requirement

The minimum requirement is in TS 38.104 [2] clause [8.2.X].

8.2.X.3	Test Purpose

The test shall verify the receiver’s ability to achieve throughput under ATG conditions for a given SNR.

8.2.X.4	Method of test

8.2.X.4.1	Initial Conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested:	M; see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

8.2.X.4.2	Procedure

1)	Connect the BS tester generating the wanted signal, channel simulators and AWGN generators to all BS antenna connectors (depending on ATG scenario) for diversity reception via a combining network as shown in annex D.5 and D.6 for BS type 1-C and type 1-H respectively.

2)	Adjust the AWGN generator, according to the channel bandwidth, defined in table 8.2.X.4.2-1.

Table 8.2.X.4.2-1: AWGN power level at the BS input

| Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- |
| 15 kHz | 5 | -86.5dBm / 4.5MHz |
| 30 kHz | 10 | -83.6dBm / 8.64MHz |
| NOTE: The AWGN power level contains an AWGN offset of 16dB by default. If needed for test purposes, the AWGN level can be reduced from the default by any value in the range 0dB to 16dB. Changing the AWGN level does not impact the validity of the test, as it reduces the effective base band SNR level. |  |  |

3)	The characteristics of the wanted signal shall be configured according to the corresponding UL reference measurement channel defined in annex A and the test parameters in table 8.2.X.4.2-2.

Table 8.2.X.4.2-2: Test parameters for testing air-to-ground PUSCH

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Uplink-downlink allocation for TDD (Note 1) |  | 15 kHz SCS: FDD 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL patterns, e.g., 30D4S6U, S=14G for 30kHz SCS. |  |  |

4)	The channel simulators shall be configured according to the corresponding channel model defined in annex G.

5)	Adjust the equipment so that required SNR specified in table 8.2.x.5-1 to 8.2.x.5-2 is achieved at the BS input.

6)	For each of the reference channels in table 8.2.x.5-1 to 8.2.x.5-2 applicable for the base station, measure the throughput.

8.2.X.5	Test Requirement

The throughput measured according to clause 8.2.X.4.2 shall not be below the limits for the SNR levels specified in table 8.2.X.5-1 to 8.2.X.5-2. Unless stated otherwise, the MIMO correlation matrices for the gNB are defined in annex G for low correlation.

Table 8.2.X.5-1: Test requirements for PUSCH with 70% of maximum throughput, Type A, 5 MHz channel bandwidth, 15 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Frequency offset [Hz] | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | AWGN | 200 | 70 % | G-FR1-AY-1 | pos1 | [19.7] |
|  |  | Normal | AWGN | 200 | 70 % | G-FR1-AZ-1 | pos1 | [20.0] |

Table 8.2.X.5-2: Test requirements for PUSCH with 70% of maximum throughput, Type A, 10 MHz channel bandwidth, 30 kHz SCS

| Number of TX antennas | Number of RX antennas | Cyclic prefix | Propagation conditions and correlation matrix (annex G) | Frequency offset [Hz] | Fraction of maximum throughput | FRC (annex A) | Additional DM-RS position | SNR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | Normal | AWGN | 500 | 70 % | G-FR1-AY-2 | pos1 | [19.3] |
|  |  | Normal | AWGN | 500 | 70 % | G-FR1-AZ-2 | pos1 | [20.3] |

## <Next change>

A.Y	Fixed Reference Channels for performance requirements (64QAM, R=948/1024)

The parameters for the reference measurement channels are specified in table A.Y-1 for FR1 PUSCH performance requirements:

-	FRC parameters are specified in table A.Y-1 for FR1 PUSCH with transform precoding disabled, Additional DM-RS position = pos1 and 1 transmission layers.

Table A.Y-1: FRC parameters for FR1 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos1 and 1 transmission layers (64QAM, R=948/1024)

| Reference channel | G-FR1-AY-1 |  | G-FR1-AY-2 |
| --- | --- | --- | --- |
| Subcarrier spacing [kHz] |  | 15 | 30 |
| Allocated resource blocks |  | 25 | 24 |
| CP-OFDM Symbols per slot (Note 1) |  | 12 | 12 |
| Modulation |  | 64 QAM | 64 QAM |
| Code rate (Note 2) |  | 948/1024 | 948/1024 |
| Payload size (bits) |  | 19968 | 18960 |
| Transport block CRC (bits) |  | 24 | 24 |
| Code block CRC size (bits) |  | 24 | 24 |
| Number of code blocks - C |  | 3 | 3 |
| Code block size including CRC (bits) (Note 2) |  | 6688 | 6352 |
| Total number of bits per slot |  | 21600 | 20736 |
| Total symbols per slot |  | 3600 | 3456 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos1, l0 = 2 and I = 11 for PUSCH mapping type A as per table 6.4.1.1.3-3 of TS 38.211 [9]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [15]. |  |  |  |

A.Z	Fixed Reference Channels for performance requirements (256QAM, R=754/1024)

The parameters for the reference measurement channels are specified in table A.Z-1 for FR1 PUSCH performance requirements:

-	FRC parameters are specified in table A.Z-1 for FR1 PUSCH with transform precoding disabled, Additional DM-RS position = pos1 and 1 transmission layers.

Table A.Z-1: FRC parameters for FR1 PUSCH performance requirements, transform precoding disabled, Additional DM-RS position = pos1 and 1 transmission layers (256QAM, R=754/1024)

| Reference channel | G-FR1-AZ-1 | G-FR1-AZ-2 |
| --- | --- | --- |
| Subcarrier spacing [kHz] | 15 | 30 |
| Allocated resource blocks | 25 | 24 |
| CP-OFDM Symbols per slot (Note 1) | 12 | 12 |
| Modulation | 256 QAM | 256 QAM |
| Code rate (Note 2) | 754/1024 | 754/1024 |
| Payload size (bits) | 21000 | 20496 |
| Transport block CRC (bits) | 24 | 24 |
| Code block CRC size (bits) | 24 | 24 |
| Number of code blocks - C | 3 | 3 |
| Code block size including CRC (bits) (Note 2) | 7032 | 6864 |
| Total number of bits per slot | 28800 | 27648 |
| Total symbols per slot | 3600 | 3456 |
| NOTE 1: DM-RS configuration type = 1 with DM-RS duration = single-symbol DM-RS and the number of DM-RS CDM groups without data is 2, Additional DM-RS position = pos1, l0 = 2 and I = 11 for PUSCH mapping type A as per table 6.4.1.1.3-3 of TS 38.211 [9]. NOTE 2: Code block size including CRC (bits) equals to K' in clause 5.2.2 of TS 38.212 [15]. |  |  |

Annex B (normative):
Error Vector Magnitude (FR1)

## <End of change>