3GPP TSG RAN WG4 Meeting #110	R4-2400975

Athens, Greece, February 26- March 01, 2024

Agenda Item:	8.6.1

Source: 	Ericsson

Title:	Analysis of RAN2 LS on BWP operation without restriction

Document for:	Discussion

# Introduction

In this paper we analyze the reply to RAN2 LS to RAN4 regarding the applicability of the NCD-SSB based L1 and L3 measurements on PSCell for the UE supporting option C [1].

# Analysis of applicability of PSCell

The RAN2 LS is reply to an earlier LS sent by RAN4 to RAN2 [2]. In this LS RAN4 informed RAN2 that:

- For UE supporting option C and configured with CA, NCD-SSB based L1 and L3 intra-frequency measurement requirements are only applicable for the PCell.

RAN4 original LS has left ambiguity in RAN2 regarding whether the NCD-SSB based L1 and L3 intra-frequency measurement requirements are also applicable to the PSCell. Therefore, RAN2 is asking RAN4 to clarify in case RAN4 has missed this information in the original LS.

Based on RAN4 discussion and agreements on this issue, it is our understanding that the NCD-SSB based L1 and L3 intra-frequency measurement requirements for the UE supporting option C are also applicable to any NR SpCell. These measurements are therefore also applicable to the PSCell in EN-DC or NR-DC scenarios.

# Summary

This paper analyzes the question raised by RAN2 in their reply LS to RAN2 regarding the applicability of the NCD-SSB based L1 and L3 intra-frequency measurement requirements to the PSCell. The following are the proposals:

- Proposal #1: The NCD-SSB based L1 and L3 intra-frequency measurement requirements for the UE supporting option C are applicable to any NR SpCell.

- Proposal #2: The NCD-SSB based L1 and L3 intra-frequency measurement requirements for the UE supporting option C are therefore also applicable to PSCell in EN-DC or NR-DC scenario.

- Proposal #3: Send reply LS to RAN2 informing that the NCD-SSB based L1 and L3 intra-frequency measurement requirements for the UE supporting option C are also applicable to PSCell in EN-DC or NR-DC scenario.

# References

- R2-2313951, Reply LS to RAN4 on BWP operation without restriction, RAN2 reply LS to RAN4

- R4-2317430, LS on conclusion on BWP operation without restriction, RAN4 LS to RAN2