3GPP TSG-RAN WG4 Meeting #110              	R4-2400978

# Athens, GR, 26 Feb – 01 Mar, 2024

Agenda item:	8.10.6.1

Source:	Samsung

Title:	Discussion on UE demodulation requirements for dedicated spectrum less than 5MHz

Document for:	Discussion

- Introduction

In RAN#101 meeting, the WID[1] on NR supporting dedicated spectrum less than 5MHz for FR1 was approved with the following objective as

Core requirement - Specify necessary RAN4 requirements to support deploying NR in spectrum allocations from approximately 3 MHz up to below 5 MHz [RAN4], including in bands n100, n106, n26, n28 and n85: - Specify system parameters (including channel and sync rasters) for the associated dedicated spectrum. - Minimize impact on RF requirements: - Reuse 5 MHz channel bandwidth at least for FRMCS use case (assuming co-located NR and GSM-R with same operator). - Specify the required RF requirements for optional 3 MHz channel bandwidth in bands n100, n106, n26, n28 and n85. - Specify RRM requirements while minimizing specification impact to support operation in dedicated spectrum allocations from approximately 3 MHz up to below 5 MHz. Performance requirement Specify necessary UE/BS performance requirements for NR operation in dedicated FDD FR1 spectrum allocations from approximately 3MHz up to below 5MHz, corresponding to the core requirements: - Specify necessary RRM performance requirements (RAN4) - Specify necessary UE demodulation performance and CSI reporting requirements (RAN4) - Specify necessary BS demodulation performance requirements (RAN4) - Specify necessary BS conformance tests (RAN4)

In this contribution, our view on the test scope and simulation parameters of UE demodulation requirement for dedicated spectrum less than 5MHz are provided.

# 2	Discussion

## 2.1	PDSCH

In RAN4#109 meeting, the WF[2] was approved with the PDSCH as:

Issue 1-1-1: Will UE exist that supports ONLY 5MHz CBW and less? Agreement (online session): - For the purpose of demodulation test coverage in Rel-18, assume there are no UEs that ONLY support maximum channel bandwidth of 5 MHz. Issue 1-2-1: Introduction of PDSCH requirements Way forward: Introduction of PDSCH requirements for less then 5MHz requires further discussion: - Option 1: Do not introduce new PDSCH requirements for 3MHz CBW - Option 2: Introduce new requirements for PDSCH for 3MHz CBW - Option 2-a: in non-HST conditions - Option 2-b: with HST channel - Option 2-c: Option 2-a and Option 2-b.

Introduction of new requirements

Regarding PDSCH, companies had concerns that if a UE supports only 3MHz channel bandwidth, current requirements may not be applied to test cases for the UE. Meanwhile, in RAN4#109, the agreement has been achieved to assume that there are no UEs that only support maximum channel bandwidth of 5 MHz. It means that current requirements can be applied to test cases for the UE that supports both 3MHz and wider channel bandwidth. In our view the issue has been addressed, and we think new PDSCH requirements for 3MHz channel bandwidth is not needed.

Proposal 1: Do not introduce new PDSCH requirements for 3MHz CBW

## 2.2	PDCCH

In RAN4#109 meeting, the WF[2] was approved with the PDCCH as:

Issue 1-3-1: Requirements for non-punctured PDCCH Agreement: - Not to introduce new demodulation performance requirements for non-punctured PDCCH with CBW below 5MHz in normal conditions. Issue 1-3-2: Requirements for punctured PDCCH Way forward: Further discussion of requirements for punctured PDCCH is needed: - Option 1: Define punctured PDCCH demodulation requirements with 15PRBs for UE supporting NR_FR1_lessthan_5MHz_BW considering the following parameters: - 15PRBs, 3 symbols, non-interleaved, AL4, DCI 1_0 (35 bits for 15 PRBs); Use CCEs #4, #5, #6, and #7 to transmit PDCCH with DCI 1_0. - Option 2: Introduce requirements, if testability issue is resolved. - Option 3: Do not introduce new requirements for punctured PDCCH with focus on CORESET#0 puncturing. Issue 1-3-3: PDCCH requirements in HST conditions Way forward: The Issues requires further discussion: - Option 1: Introduce PDCCH requirements at 3MHz CBW in HST conditions. - Option 2: Not to introduce HST scenario for PDCCH requirements.

Requirements for punctured PDCCH

According to RAN1#115 UE feature [3], the UE should support reception of 15PRB CORESET#0, if the UE declared capability to support 3MHz channel bandwidth (FG51-1). In our understanding, if the UE can receive 15PRB CORESET#0, the UE should support receiving punctured PDCCH as well as non-punctured PDCCH. Furthermore, performance loss is expected for punctured PRBs, which may need requirements to verify the UE supporting 3MHz channel bandwidth. Therefore we think new requirement is needed for punctured PDCCH.

For the testability issue, since CORESET#0 can be configured both CSS and USS. The contents of data scheduled by CORESET#0 would be different according to CSS or USS. For example CORSET#0 configured by CSS schedules SIB1, and configured by USS schedules other data, which has HARQ-ACK feedback. Thus, we think CORESET#0 can be tested by USS.

Proposal 2: Introduce PDCCH requirements with punctured PRB

For information, simulation result of option1 is provided as follows:

Table 1 PDCCH simulation results of option1

| Bandwidth (MHz) | CORESET RB | CORESET duration | Aggregation level | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 3 | 15 | 3 | 4 | TDLC300-100 | 2x2 Low | 1 | 0.9 |
| 3 | 15 | 3 | 4 | TDLC300-100 | 2x4 Low | 1 | -3.3 |

PDCCH requirements in HST conditions

HST scenario was not introduced in PDCCH requirements, since it is not the bottleneck of the channel. If the UE has passed PDSCH test with HST scenario, it can be assumed that PDCCH is also feasible. In the same sense, we think it is not necessary to introduce HST scenario for PDCCH requirements with 3MHz CBW.

Proposal 3: Not to introduce HST scenario for PDCCH requirements

## 2.3	PBCH

In RAN4#109 meeting, the WF[2] was approved with the PBCH as:

Issue 1-4-1: PBCH requirement in non-HST conditions Agreement: Use the following parameters for PBCH requirement in normal conditions. Interested companies are encouraged to bring simulation results. Duplex; Bandwidth (MHz) / Subcarrier spacing (kHz); SSB/PBCH index; Propagation condition; Antenna configuration and correlation matrix; Reference value. Pm-bch (%); SNR (dB). FDD; 3 / 15; Unknown; TDLC300-100; 1 x 2 Low,; 1; TBD. 1x4 Low; 1; TBD. Issue 1-4-2: PBCH requirement in HST conditions Way forward: Further discuss whether to introduce PBCH requirements in HST conditions: - Option 1: Define PBCH requirements in HST conditions considering the following parameters: Duplex; Bandwidth (MHz) / Subcarrier spacing (kHz); SSB/PBCH index; Propagation condition; Antenna configuration and correlation matrix; Reference value. Pm-bch (%); SNR (dB). FDD; 3 / 15; Unknown; [HST-417]; 1Tx/2Rx Low; 1; TBD. FDD; 3 / 15; Unknown; [HST-417]; 1Tx/4Rx Low; 1; TBD. - Option 2: Not to introduce HST scenario for PBCH requirements.

PBCH requirement in non-HST conditions

Following is simulation results for PBCH demodulation in non-HST condition.

Table 2 PBCH simulation results

| Duplex | Bandwidth (MHz) / Subcarrier spacing (kHz) | SSB/PBCH index | Propagation condition | Antenna configuration and correlation matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Pm-bch (%) | SNR (dB) |
| FDD | 3 / 15 | Unknown | TDLC300-100 | 1 x 2 Low, | 1 | -6 |
|  |  |  |  | 1x4 Low | 1 | -8.7 |

Whether to consider HST conditions for PBCH

In general, there are no conformance test for PBCH performance test requirement due to test feasibility issue. RAN4 PBCH requirement is just for reference. So we think introducing PBCH requirement in non-HST condition is enough.

Proposal 4: Not to introduce HST scenario for PBCH requirements

# 3	Conclusion

In this contribution, our view on UE demodulation requirement is provided.

Proposal 1: Do not introduce new PDSCH requirements for 3MHz CBW

Proposal 2: Introduce PDCCH requirements with punctured PRB

Proposal 3: Not to introduce HST scenario for PDCCH requirements

Proposal 4: Not to introduce HST scenario for PBCH requirements

# References

RP- 231713 Revised WID: NR support for dedicated spectrum less than 5MHz for FR1, Nokia, Nokia Shanghai Bell

R4- 2321064 WF on NR_FR1_lessthan_5MHz_BW_demod, Nokia, Nokia Shanghai Bell

R1-2312572 Updated RAN1 UE features list for Rel-18 NR after RAN1#115, AT&T, NTT DOCOMO, INC.