# 3GPP TSG-RAN WG4 Meeting #110	R4-2400992

# Athens, GR, 26 Feb – 01 Mar, 2024

Agenda Item:	8.3.2.1

Source: 	OPPO

Title: 	On remaining issues for FR2_multiRX_DL

Document for:	Approval

1	Introduction

A WF on Rel-18 FR2_multiRX_DL was approved in RAN4#109 meeting [1].

In this contribution, we further discuss the remaining issues for FR2-1 multi-Rx reception.

2	Discussion

Applicability and conditions

#### Issue 1-3-5: UE behaviour when a condition becomes violated during a measurement For the requirement defined in the WI excepting the requirement for beam sweeping factor reduction: - When the side conditions are not fulfilled, the corresponding multi-Rx requirement is not applicable. - Companies may bring concrete proposal on UE requirement in the transition period for each requirement/side condition in the maintenance part. - Option 1a: - When the side conditions are changed with a transition between multi-Rx operation and no multi-Rx operation, the corresponding multi-Rx requirement is not applicable, and no UE behavior requirement needs to be defined. - Option 2: - When the condition of multi-Rx becomes violated during measurement, at least UE can continue the on-going L1 measurement. - When the condition of multi-Rx becomes violated during measurement, the measurement/scheduling restriction relaxation is not allowed any more. - When the condition of multi-Rx becomes violated during measurement, whether the requirements of L1 measurement period should be relaxed, depending on the final identified L1 measurement period requirements. If fast beam sweeping is applied during L1 measurement, then the condition and implementation of fast beam sweeping would impact the decision.

The set of conditions for the L1 measurement requirements have been discussed for several meetings. We don’t see it is necessary to have general agreements on the set of conditions to be considered in the L1 measurement requirements. Different applicable conditions may need to be specified for different requirements. They can be discussed in the corresponding requirements and drafting CR. When side conditions for multi-Rx operation are violated, UE is not expected to meet the requirements of multi-Rx operation. The legacy requirement can apply when UE restart the measurement.

Proposal 1: When the side conditions are violated, the corresponding requirement is not applicable, and no UE requirements need to be defined.

3	Conclusion

Proposal 1: When the side conditions are violated, the corresponding requirement is not applicable, and no UE requirements need to be defined.

4	Reference

- R4-2321602, WF on FR2 multi-RX operation RRM requirements, vivo, Ericsson