3GPP TSG-RAN WG4 Meeting # 110	R4-2401009

# Athens, GR, 26 Feb – 01 Mar, 2024

Agenda Item:	8.17.1.2

Source: 	OPPO

Title: 	Discussion on MUSIM gap collisions handling

Document for:	Approval

1	Introduction

MUSIM gaps were discussed and introduced in Rel-17 and the corresponding requirements will be discussed in Rel-18. MUSIM gap collision is one main issue to be discussed and the related achievements are captured in [1]. We will discuss the remaining open issues and provide our considerations.

2	Discussion

Issue 2-2-2: Clarification on “priority base solution” and “keep solution” Proposals - P1: A UE supporting MUSIM gaps shall at least support priority based solution. A UE supporting MUSIM gaps may support keep solution. (Nokia)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_DualTxRx_MUSIM | 43-1 | Collision handling and signalling solution for handling collisions between MUSIM gaps and between measurement gaps and MUSIM gaps | UE supports indication of MUSIM gap priority; collision handling between MUSIM gaps and between measurement gaps and MUSIM gaps | musim-GapPreference-r17 | YES |  | UE is not capable to meet Rel-18 MUSIM requirement | Per UE |

Based on the UE capability agreed in the last meeting, a single UE feature 43-1 is used to indicate the support of collision handling and related signaling. In our understanding, both priority-based solution and keep solution are included in this feature. If this understanding is correct, no separate UE capabilities are needed for these two solutions and the above P1 should be revised as:

Proposal 1: A UE supporting feature 43-1 shall support both priority-based solution and keep solution.

3	Conclusion

This contribution gave our general views on how to MUSIM gap collision issue and the following proposals:

Proposal 1: A UE supporting feature 43-1 shall support both priority-based solution and keep solution.

4	Reference

- R4-2321613, WF on NR Dual TxRx Multi-SIM, vivo