3GPP TSG-RAN WG4 Meeting #110	R4-2401109

Athens, GR, 26 Feb – 01 Mar, 2024

Source:	Samsung

Title:	discussion on 8Rx PDSCH requirements

Agenda item:	8.1.3.1.2

Document for:	Discussion

- Introduction

In last RAN4 #109 meeting, general aspects, PDSCH requirements for FR1 8Rx single CC and CA cases have some agreements in WF [1]. Generally, the general aspects, SDR and CQI requirements parts do not have open issues any more, while several open issues on PDSCH demodulation part still need further discussion.

Issue 2-1: Antenna correlation for 8Rx Rank 4 and Rank 8 - Agreement - No further discussion on the Medium antenna correlation selection for Rank 4 and Rank 8 for 8Rx tests Issue 2-2: How to align the ideal results alignment - Proposals - Option 1: Remove the farthest outlier from the average results (the methodology used from NR BS Rel-15) - Option 2: Set the max allowed span to 3dB Issue 2-3: Margin to be added on top of the averaged impairment results for requirements derivation - Proposal - Rank2 and Rank8 - Add 0.8dB margin for 64QAM (the methodology used from NR UE Rel-15) - Rank4 - Option 1: Add 0.8dB margin for 64QAM (the methodology used from NR UE Rel-15) - Option 2: Add 1.5dB margin for 64QAM - Option 3: Define two sets of requirements applicable to UE support of maxNumberMIMO-LayersPDSCH = 8 and maxNumberMIMO-LayersPDSCH = 4 respectively - Set A (maxNumberMIMO-LayersPDSCH=8): Add 0.8dB margin (the methodology used from NR UE Rel-15) - Set B (maxNumberMIMO-LayersPDSCH=4): Add an additional margin on top of Set A results (e.g., 1.2dB); i.e., When UE supports maxNumberMIMO-LayersPDSCH=4, an additional x dB margin is added to the SNR requirement Issue 3-1: Antenna correlation for carrier with Rank 2 in 8Rx CA test - Proposal - Option 1: Revisit Rank 2 to TDLA30-10 Low - Option 2: Keep TDLC300-100 ULA Medium B ( = 0.3, = 0.005154) that is same as Rank 2 single carrier test - Note: Interested companies can provide simulation results with both channel conditions TDLA30-10 Low and TDLC300-100 ULA Medium B ( = 0.3, = 0.005154) for the next RAN4#110 meeting Issue 3-2: Number of HARQ process for 8Rx CA test - Proposal HARQ process number; CCs with the same duplex mode & SCS with Pcell; CCs with different duplex mode / SCS with Pcell. FDD 15 kHz + TDD 30 kHz CA; FDD PCell; 4; 8. TDD PCell; Option 1: 8 Option 2: For CC with Rank 2: 10 For CC with Rank 8: 8 Other options not precluded; 8. FDD 15 kHz + FDD 15 kHz CA; FDD PCell; 4; N/A. TDD 30 kHz + TDD 30 kHz CA; TDD PCell; Option 1: 8 Option 2: For CC with Rank 2: 10 For CC with Rank 8: 8 Other options not precluded; N/A. Issue 3-3: K1 value for 8Rx CA test - Proposal The number of slots between PDSCH and corresponding HARQ-ACK information; CCs with the same duplex mode and SCS with Pcell; CCs with different duplex mode and/or SCS with Pcell. FDD 15 kHz + TDD 30 kHz CA; FDD PCell; {2}; {2}. TDD PCell; Option 1: For CC with Rank 2: {8,7,6,5,5,4,3,2} For CC with Rank 8: {8,7,6,5,5,4,3} Option 2: For CC with Rank 2: {8,7,6,5,5,4,3,11} For CC with Rank 8: {8,7,6,5,5,4,3} Other options not precluded; {7,5,4,11,9}. FDD 15 kHz + FDD 15 kHz CA; FDD PCell; {2}; N/A. TDD 30 kHz + TDD 30 kHz CA; TDD PCell; Option 1: For CC with Rank 2: {8,7,6,5,5,4,3,2} For CC with Rank 8: {8,7,6,5,5,4,3} Option 2: For CC with Rank2: {8,7,6,5,5,4,3,11} For CC with Rank 8: {8,7,6,5,5,4,3} Other options not precluded; N/A.

- Discussion on 8Rx PDSCH requirements

  - Single carrier requirements

In last RAN4#109 meeting, due to current simulation results from companies have large span, how to do the results alignment and the value of margin to be added on top of the averaged impairment results are discussed, but no consensus agreements are achieved. If we check current simulation results which updated in RAN4 #109 meeting [2] carefully, we could find out the results with impairments have much more span than the ideal results, as different impairment values from 1dB to 3dB are added. Therefore, we suggest companies to double check the results with impairments firstly and remove the farthest outlier from the average results.

Observation 1: The results with impairments have much more span than the ideal results, as different impairment values from 1dB to 3dB are added.

Proposal 1: Double check the results with impairments firstly and remove the farthest outlier from the average results.

For the margin to be added on top of the averaged impairments results, we prefer to use option 3, and the value of x should be decided according to simulation results.

Proposal 2: For 8Rx with rank 4 case, the margin to be added on top of the averaged impairments results, define two sets of requirements applicable to UE support of maxNumberMIMO-LayersPDSCH = 8 and maxNumberMIMO-LayersPDSCH = 4 respectively

      - Set A (maxNumberMIMO-LayersPDSCH=8): Add 0.8dB margin (the methodology used from NR UE Rel-15)

      - Set B (maxNumberMIMO-LayersPDSCH=4): Add an additional margin on top of Set A results (e.g., 1.2dB); i.e., When UE supports maxNumberMIMO-LayersPDSCH=4, an additional x dB margin is added to the SNR requirement.

  - CA requirements

Antenna correlation for carrier with Rank 2 in 8Rx CA test

For the CA case, as we agreed to use the same assumptions with the single CC case before, and considering 8Rx case are targeting for CPE/FWA/vehicle/industrial devices, we don’t see any necessary to revert to TDLA30-10 Low antenna correlation.

Proposal 3: For 8Rx CA test with Rank 2, keep TDLC300-100 ULA Medium B  ( = 0.3,  = 0.005154) that is same as Rank 2 single carrier test.

Number of HARQ process for 8Rx CA test

Before version V17.7.0, the definition in 38.101-4 about number of HARQ process for CA test is as Table 2-1. The correction CR [3] update the HARQ process number of FDD + TDD case from 8 to 10 in Table 2-2, since the dl-DataToUL-ACK has up to 8 entries.

Table 2-1 HARQ process number before version V17.7.0

| HARQ process number |  | CCs with the same duplex mode & SCS with Pcell | CCs with different duplex mode / SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | 4 | 8 |
|  | TDD PCell | 8 | 8 |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | 4 | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 8 | N/A |

Table 2-2 HARQ process number since version V17.7.0

| HARQ process number |  | CCs with the same duplex mode & SCS with Pcell | CCs with different duplex mode / SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | 4 | 8 |
|  | TDD PCell | 10 | 8 |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | 4 | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 8 | N/A |

For 8Rx with rank 8, considering there is no PDSCH scheduling on special slot, the HARQ process number defined as 8 is enough. For 8Rx with rank 2, there is PDSCH scheduling on special slot, hence the HARQ process number should still be defined as 10.

Proposal 4: For 8Rx CA test, define test parameters for number of HARQ processes as below

| HARQ process number |  | CCs with the same duplex mode & SCS with Pcell | CCs with different duplex mode / SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | 4 | 8 |
|  | TDD PCell | For CC with Rank 2: 10 For CC with Rank 8: 8 | 8 |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | 4 | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 8 | N/A |

K1 value for 8Rx CA test

Accordingly with the number of HARQ processes, the test parameters for K1 values should be defined separately for CC with Rank 2 and CC with Rank 8.

Proposal 5: For 8Rx CA test, define test parameters for K1 values as below

| The number of slots between PDSCH and corresponding HARQ-ACK information |  | CCs with the same duplex mode and SCS with Pcell | CCs with different duplex mode and/or SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | {2} | {2} |
|  | TDD PCell | For CC with Rank 2: {8,7,6,5,5,4,3,11} For CC with Rank 8: {8,7,6,5,5,4,3} | {7,5,4,11,9} |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | {2} | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | For CC with Rank 2: {8,7,6,5,5,4,3,2} For CC with Rank 8: {8,7,6,5,5,4,3} | N/A |

  - Simulation results for CA tests

Below tables list our simulation results with impairments for CA tests.

Table 2-3: Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-X.Y1 FDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.63 |
| 10 | R.PDSCH.1-3.1 FDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.63 |
| 15 | R.PDSCH.1-X.Y2 FDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.75 |
| 20 | R.PDSCH.1-X.Y3 FDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 11.86 |
| 25 | R.PDSCH.1-X.Y4 FDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.07 |
| 30 | R.PDSCH.1-X.Y5 FDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.09 |
| 35 | R.PDSCH.1-X.Y6 FDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.11 |
| 40 | R.PDSCH.1-X.Y7 FDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.39 |
| 45 | R.PDSCH.1-X.Y8 FDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.56 |
| 50 | R.PDSCH.1-X.Y9 FDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.56 |

Table 2-4: Single carrier performance for TDD 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-X.Y1 TDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.31 |
| 10 | R.PDSCH.2-X.Y2 TDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.31 |
| 15 | R.PDSCH.2-X.Y3 TDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.44 |
| 20 | R.PDSCH.2-X.Y4 TDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.45 |
| 25 | R.PDSCH.2-X.Y5 TDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.51 |
| 30 | R.PDSCH.2-X.Y6 TDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.62 |
| 40 | R.PDSCH.2-3.1 TDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.67 |
| 50 | R.PDSCH.2-X.Y7 TDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.82 |
| 60 | R.PDSCH.2-X.Y8 TDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 12.93 |
| 80 | R.PDSCH.2-X.Y9 TDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.10 |
| 90 | R.PDSCH.2-X.Y10 TDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.21 |
| 100 | R.PDSCH.2-X.Y11 TDD | 64QAM, 0.50 | TDLC300-100 | 2x8, ULA Medium B | 70 | 13.24 |

Table 2-5: Single carrier performance for FDD 15 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.1-Z.Y1 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.21 |
| 10 | R.PDSCH.1-3.7 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.24 |
| 15 | R.PDSCH.1-Z.Y2 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.37 |
| 20 | R.PDSCH.1-Z.Y3 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.42 |
| 25 | R.PDSCH.1-Z.Y4 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.59 |
| 30 | R.PDSCH.1-Z.Y5 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.61 |
| 35 | R.PDSCH.1-Z.Y6 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.78 |
| 40 | R.PDSCH.1-Z.Y7 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.92 |
| 45 | R.PDSCH.1-Z.Y8 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.98 |
| 50 | R.PDSCH.1-Z.Y9 FDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 22.06 |

Table 2-6: Single carrier performance for TDD 30 kHz SCS for CA configurations

| Bandwidth (MHz) | Reference channel | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | R.PDSCH.2-Z.Y1 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.69 |
| 10 | R.PDSCH.2-Z.Y2 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.69 |
| 15 | R.PDSCH.2-Z.Y3 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.72 |
| 20 | R.PDSCH.2-Z.Y4 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.77 |
| 25 | R.PDSCH.2-Z.Y5 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.85 |
| 30 | R.PDSCH.2-Z.Y6 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.83 |
| 40 | R.PDSCH.2-3.7 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 20.85 |
| 50 | R.PDSCH.2-Z.Y7 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.13 |
| 60 | R.PDSCH.2-Z.Y8 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.36 |
| 80 | R.PDSCH.2-Z.Y9 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.57 |
| 90 | R.PDSCH.2-Z.Y10 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.61 |
| 100 | R.PDSCH.2-Z.Y11 TDD | 64QAM, 0.43 | TDLA30-10 | 8x8, ULA Low | 70 | 21.62 |

Observation 2: The SNR differences between the minimum and maximum bandwidths are no more than 1dB for both rank 2 and rank 8 scenarios.

- Conclusion

In this contribution, we provide our analysis for open issues and simulation results for rank 2 and rank 8 cases for 8Rx CA scenarios, both FDD and TDD. The observations and proposals could be summarized as:

Observation 1: The results with impairments have much more span than the ideal results, as different impairment values from 1dB to 3dB are added.

Proposal 1: Double check the results with impairments firstly and remove the farthest outlier from the average results.

Proposal 2: For 8Rx with rank 4 case, the margin to be added on top of the averaged impairments results, define two sets of requirements applicable to UE support of maxNumberMIMO-LayersPDSCH = 8 and maxNumberMIMO-LayersPDSCH = 4 respectively

      - Set A (maxNumberMIMO-LayersPDSCH=8): Add 0.8dB margin (the methodology used from NR UE Rel-15)

      - Set B (maxNumberMIMO-LayersPDSCH=4): Add an additional margin on top of Set A results (e.g., 1.2dB); i.e., When UE supports maxNumberMIMO-LayersPDSCH=4, an additional x dB margin is added to the SNR requirement.

Proposal 3: For 8Rx CA test with Rank 2, keep TDLC300-100 ULA Medium B  ( = 0.3,  = 0.005154) that is same as Rank 2 single carrier test.

Proposal 4: For 8Rx CA test, define test parameters for number of HARQ processes as below

| HARQ process number |  | CCs with the same duplex mode & SCS with Pcell | CCs with different duplex mode / SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | 4 | 8 |
|  | TDD PCell | For CC with Rank 2: 10 For CC with Rank 8: 8 | 8 |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | 4 | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | 8 | N/A |

Proposal 5: For 8Rx CA test, define test parameters for K1 values as below

| The number of slots between PDSCH and corresponding HARQ-ACK information |  | CCs with the same duplex mode and SCS with Pcell | CCs with different duplex mode and/or SCS with Pcell |
| --- | --- | --- | --- |
| FDD 15 kHz + TDD 30 kHz CA | FDD PCell | {2} | {2} |
|  | TDD PCell | For CC with Rank 2: {8,7,6,5,5,4,3,11} For CC with Rank 8: {8,7,6,5,5,4,3} | {7,5,4,11,9} |
| FDD 15 kHz + FDD 15 kHz CA | FDD PCell | {2} | N/A |
| TDD 30 kHz + TDD 30 kHz CA | TDD PCell | For CC with Rank 2: {8,7,6,5,5,4,3,2} For CC with Rank 8: {8,7,6,5,5,4,3} | N/A |

Observation 2: The SNR differences between the minimum and maximum bandwidths are no more than 1dB for both rank 2 and rank 8 scenarios.

- Reference

[1] R4-2321199, “WF for UE 8Rx performance requirements”, Huawei, HiSilicon, MediaTek, 3GPP RAN4 #109 meeting

[2] R4-2319226, “Simulation results collection for 8Rx UE demodulation requirements”, Ericsson, 3GPP RAN4 #109 meeting

[3] R4-2218214, “CR to dl-DataToUL-ACK for PDSCH demod CA TCs”, Anritsu Corporation, 3GPP RAN4 #105 meeting