3GPP TSG-RAN4 Meeting #110	R4-2401157

# Athens, Greece, February 26 – March 1, 2024

Source:	LG Electronics

Title:	Maintenance of SL CA

Agenda item:	8.22.1.3

Document for:	Discussion

Introduction

In this paper, we provide our views on the maintenance issues of SL CA, i.e., MPR and A-MPR for intra-band SL CA in Band n47.

Discussion

PSSCH/PSCCH MPR for SLCA with non-contiguous RB allocation

In the last RAN4 meeting, both MPRs for SLCA with contiguous RB allocation and non-contiguous RB allocation were discussed, however, MPR with contiguous RB allocation was specified and MPR with non-contiguous RB allocation was not specified. The MPR with non-contiguous RB allocation was captured in TR.

Based on the RAN1 current specification, the both RB allocation should be supported in SL intra-band contiguous CA. Therefore, MPR with non-contiguous RB should be specified.

Based on [1] and [2], the following SL C-CA MPR with non-contiguous RB allocation is proposed.

Proposal 1: Specify PSSCH/PSCCH MPR for SL Contiguous CA with non-contiguous RB allocation in Table 2-1.

Table 2-1 PSSCH/PSCCH MPR for SL Contiguous CA with Non-contiguous RB allocations

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 | ≤ 9.5 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 | ≤ 9.5 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 | ≤ 9.5 |
|  | 256QAM | ≤ 7.0 | ≤ 7.0 | ≤ 9.5 |

A-MPR for SLCA

Based on WF [1], NS_52 was agreed to be considered for SL CA A-MPR. A-MPR for NS_52 was already specified in LTE V2X and NR V2X based on the current FCC regulation because of when the update of FCC regulation is officially announced is not clear. The same principle needs to be considered in NR SL CA.

Proposal 2: Specify ’SLCA_NS_52’ for SL intra-band C-CA A-MPR as Table 2-2.

Table 2-2: Mapping of network signaling label

| Sidelink CA operating band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| SL_n47 | SLCA_NS_01 |  | SLCA_NS_52 |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 |  |  |  |  |  |  |  |  |

A-MPR for SLCA PSSCH/PSCCH

Based on WF[1] and the simulation results [3],  we propose as follow.

Proposal 3: For SL C-CA PSSCH/PSCCH A-MPR for contiguous RB allocation and non-contiguous RB allocation, consider Table 2-3 and Table 2-4  respectively.

Table 2-3 SL CA NS_52 PSSCH/PSCCH A-MPR for SL Contiguous CA with Contiguous RB allocations

| Modulation |  | A-MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 7.0 | ≤ 8.5 |
|  | 16QAM | ≤ 7.0 | ≤ 8.5 |
|  | 64QAM | ≤ 7.0 | ≤ 8.5 |
|  | 256QAM | ≤ 7.0 | ≤ 8.5 |

Table 2-4 SL CA NS_52 PSSCH/PSCCH A-MPR for SL Contiguous CA with Non-contiguous RB allocations

| Modulation |  | A-MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 8.0 | ≤ 13.5 |
|  | 16QAM | ≤ 3.0 | ≤ 8.0 | ≤ 13.5 |
|  | 64QAM | ≤ 4.5 | ≤ 8.0 | ≤ 13.5 |
|  | 256QAM | ≤ 7.0 | ≤ 8.0 | ≤ 13.5 |

A-MPR for SLCA PSFCH

Based on WF[1] and the simulation results [3],  we propose as follow.

Proposal 4: For SL-CA NS_52 PSFCH A-MPR, consider Table 2-5.

Table 2-5 SL CA NS_52 PSFCH A-MPR for SL Contiguous CA

| Modulation |  | A-MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | R ≤0. 1 | 0.1 < R ≤ 0. 55 | 0.55 < R ≤ 1.0 |
| CP-OFDM | QPSK | ≤4.0 | ≤17.0 | ≤19.0 |
| Here, R = NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2) |  |  |  |  |

A-MPR for SLCA S-SSB

Based on WF[1] and the simulation results [3],  we propose as follow.

Proposal 5: For SL-CA NS_52 S-SSB A-MPR, consider Table 2-6.

Here, Inner/Outer1/Outer2 RB allocation is refered with NR C-CA with non-contiguous RB allocation

Table 2-6 SL CA NS_52 S-SSB A-MPR for SL Contiguous CA

| A-MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- |
| Inner | Outer1 | Outer2 |
| ≤ 9.0 | ≤ 13.0 | ≤ 16.5 |

Conclusion

In this contribution, we proposed MPR requirements for SL intra-band contiguous CA with non-contiguous RB allocation, and A-MPR requirements for SL intra-band contiguous CA based on WF[1] and simulation results [3].

- MPR for SLCA PSSCH/PSCCH with non-contiguous RB allocation

Proposal 1: Specify PSSCH/PSCCH MPR for SL Contiguous CA with non-contiguous RB allocation in Table 2-1.

Table 2-1 PSSCH/PSCCH MPR for SL Contiguous CA with Non-contiguous RB allocations

| Modulation |  | MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 5.0 | ≤ 9.5 |
|  | 16QAM | ≤ 3.0 | ≤ 5.0 | ≤ 9.5 |
|  | 64QAM | ≤ 4.5 | ≤ 5.0 | ≤ 9.5 |
|  | 256QAM | ≤ 7.0 | ≤ 7.0 | ≤ 9.5 |

- A-MPR for SLCA

Proposal 2: Specify ’SLCA_NS_52’ for SL intra-band C-CA A-MPR as Table 2-2.

Table 2-2: Mapping of network signaling label

| Sidelink CA operating band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| SL_n47 | SLCA_NS_01 |  | SLCA_NS_52 |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 |  |  |  |  |  |  |  |  |

- A-MPR for SLCA PSSCH/PSCCH

Proposal 3: For SL C-CA PSSCH/PSCCH A-MPR for contiguous RB allocation and non-contiguous RB allocation, consider Table 2-3 and Table 2-4  respectively.

Table 2-3 SL CA NS_52 PSSCH/PSCCH A-MPR for SL Contiguous CA with Contiguous RB allocations

| Modulation |  | A-MPR for bandwidth class B(dB) |  |
| --- | --- | --- | --- |
|  |  | inner | outer |
| CP-OFDM | QPSK | ≤ 7.0 | ≤ 8.5 |
|  | 16QAM | ≤ 7.0 | ≤ 8.5 |
|  | 64QAM | ≤ 7.0 | ≤ 8.5 |
|  | 256QAM | ≤ 7.0 | ≤ 8.5 |

Table 2-4 SL CA NS_52 PSSCH/PSCCH A-MPR for SL Contiguous CA with Non-contiguous RB allocations

| Modulation |  | A-MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Inner | Outer1 | Outer2 |
| CP-OFDM | QPSK | ≤ 3.0 | ≤ 8.0 | ≤ 13.5 |
|  | 16QAM | ≤ 3.0 | ≤ 8.0 | ≤ 13.5 |
|  | 64QAM | ≤ 4.5 | ≤ 8.0 | ≤ 13.5 |
|  | 256QAM | ≤ 7.0 | ≤ 8.0 | ≤ 13.5 |

- A-MPR for SLCA PSFCH

Proposal 4: For SL-CA NS_52 PSFCH A-MPR, consider Table 2-5.

Table 2-5 SL CA NS_52 PSFCH A-MPR for SL Contiguous CA

| Modulation |  | A-MPR for ratio (R) in bandwidth class B(dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | R ≤0. 1 | 0.1 < R ≤ 0. 55 | 0.55 < R ≤ 1.0 |
| CP-OFDM | QPSK | ≤4.0 | ≤17.0 | ≤19.0 |
| Here, R = NGap/(NRB1+NRB2+ NGBchannel_CC1+ NGBchannel_CC2) |  |  |  |  |

- A-MPR for SLCA S-SSB

Proposal 5: For SL-CA NS_52 S-SSB A-MPR, consider Table 2-6.

Here, Inner/Outer1/Outer2 RB allocation is refered with NR C-CA with non-contiguous RB allocation

Table 2-6 SL CA NS_52 S-SSB A-MPR for SL Contiguous CA

| A-MPR for bandwidth class B(dB) |  |  |
| --- | --- | --- |
| Inner | Outer1 | Outer2 |
| ≤ 9.0 | ≤ 13.0 | ≤ 16.5 |

- References

[1] R4-2317731, “WF on NR_SL_enh2_UERF_part3”, Huawei

[2] TR 38.786 v18.0.0

[3] R4-2315532, “UE RF requirements of SL CA”, LG Electronics