3GPP TSG-RAN WG4 # 110	R4-2401159

Athens, Greece, February 26 – March 1, 2024

Agenda item:	8.3.4.3

Source:	MediaTek Inc.

Title:	Discussion on PMI requirements of FR2 Multi-Rx DL Demod

Document for:	Discussion

# Introduction

In RAN4#109, the Way Forward document was approved [1]. We provide discussion of PMI requirements with proposals for the Rel-18 work in this contribution.

# Discussion

In this chapter we discuss remaining open items of PMI requirements of FR2 Multi-Rx DL Demod. We have provided simulation results of PMI reporting with proposed parameters to this meeting in [2].

Issue 3-1-4: Performance Metric

- Agreement:

  - X= [90%]

In our PMI reporting simulations [2], we see 90% as feasible test metric. Therefore, we propose to use 90% test metric as it is more commonly used in PMI requirements.

Proposal #1: Set test metric as γ=tue/trnd, where tue is 90% of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRue with random precoding.

Issue 3-1-5: Throughput ratio (γ) value

- Agreement:

  - Consider γ = [1.3] in the next meeting, other values are not precluded. Consider some margin (to be discussed).

In our PMI reporting simulations [2], we see γ value without margins is 1.29. We need to check all companies results and agree margin.

Observation #1: In our simulation γ value without margins is 1.29.

Proposal #2: We propose to use γ value 1.1 for test requirement.

Issue 3-1-6: Reference Channel

- <way forward>

  - Option 1: Use reference channels as presented in Table 2.3-1 for FR2 multi-Rx PMI reporting requirements.

  - Option 2: Reuse the reference channel from TS38.101-4, section 6.3.2.1.7 with adaptation to 4 ports as starting point. Configure the codebook for 1 panel.

In our CR we are implementing new reference channels in single and dual port PT-RS configurations, namely as R.PDSCH.5-7.2 TDD and R.PDSCH.5-7.3 TDD.

Proposal #3: We propose to use R.PDSCH.5-7.2 TDD and R.PDSCH.5-7.3 TDD added into Table A.3.2.2.5-7.

# Conclusion

In this paper we provided the view on the PMI requirements of FR2 multipanel RX downlink demodulation requirements. The following proposals are made:

Proposal #1: Set test metric as γ=tue/trnd, where tue is 90% of the maximum throughput obtained at SNRue using the precoders configured according to the UE reports, and trnd is the throughput measured at SNRue with random precoding.

Observation #1: γ value without margins is 1.29.

Proposal #2: We propose to use γ value 1.1 for test requirement.

Proposal #3: We propose to use R.PDSCH.5-7.2 TDD and R.PDSCH.5-7.3 TDD added into Table A.3.2.2.5-7.

# Reference

- R4-2321140, “WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception”, Qualcomm

- R4-2401160, “Simulation results of PMI requirements of FR2 Multi-Rx DL Demod”, MediaTek inc.