# 3GPP TSG-RAN WG4 Meeting #110								   		                    R4-2401187

# Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR |  | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on interruprion requirements for inter-RAT NR measurement without gap (case a-1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Xiaomi |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are brackets and editor’s note in the interruprion requirements. The Tcycle,i under DRX is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the brackets and editor’s note - Add Tcycle,i definition under DRX |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RRM requirements for inter-RAT NR measurement without gap will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.8.2.22 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

=================================Start of Change #1====================================

#### 7.8.2.22	Interruptions during inter-RAT NR measurements without measurement gap

For a UE indicating ‘no-gap-with-interruption’ with the capability interRAT-NeedForInterruptionNR-r18 for inter-RAT NR measurement, the UE is allowed to cause interruptions on PCell or activated SCell(s) due to inter-RAT NR measurements with interruption ratio no more than the requirements specified below.

For the individual inter-RAT NR measurements performed on a frequency layer that corresponds to a configured measurement object i, Tcycle,i is the measurement cycle specified in Table 7.8.2.22-1, where Nfreq is defined in clause 8.1.2.4.29.1.1.

Table 7.8.2.22-1: Tcycle,i length for inter-RAT NR MOi

| DRX cycle | TCycle,I |
| --- | --- |
| No DRX | max (80ms, SMTC period) x Nfreq |
| DRX cycle ≤ 320ms | max (80ms, SMTC period, DRX cycle) x Nfreq |
| DRX cycle>320ms | DRX cycle x Nfreq |

UE is allowed to cause interruption on a certain MOi up to the interruption ratio of 2LTcycle,i. The allowed maximum total interruption ratio (D) is

D=i=1N2LTcycle,i

Where,

- N is the total number of NR MOs that UE indicates ‘no-gap-with-interruption’ for the band which the NR MO belongs to, and none of the SMTCs of the NR MOs are overlapped by the measurement gap, and

- L is the maximum interruption length for each interruption occasion, which shall not exceed 1 subframe.

The interruptions are allowed for all the serving cells in the same FR as NR SCell being measured if UE supports per-FR measurement gaps, and all the serving cells if UE does not support per-FR measurement gaps.

=================================End of Change #1====================================