3GPP TSG-RAN WG4 Meeting #110				                                                                                                                                                                                                                     R4-2401202

Athens, GR, 26 Feb – 01 Mar, 2024

Agenda item:	8.5.1

Source: 	Xiaomi

Title: 	Rel-18 RAN4 UE feature list for enhancements on NR and MR-DC measurement gaps and measurements without gaps

Document for:	Approval

- Introduction

In this paper we summarized and proposed the UE feature list needed to for NR_MG_enh2-Core WI.

- Discussion

In the last RAN4 meeting, the UE feature list in [1] was agreed as the start point for further discussion.

According to some online in the last meeting, there is still some concerns on [32-7]. In the previous discussion, the motivation to introduce this separated capability is to differentiate Rel17 UE support NeedForNCSG-InfoEUTRAN with Rel18 UE which can support both NeedForNCSG-InfoEUTRAN and EMW. The more detailed interpretation can be found in [2]. However, after some further offline discussion, the harmonized proposal can be given below.

Proposal 1:

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [32-7] | [Inter-RAT EUTRAN measurements without gap and outside active DL BWP] Case B-1 | 1. Support of requirements of inter-RAT EUTRAN measurements outside active DL BWP without gap with or without interruption. FFS: Whether this could be coupled or related to the capability of EMW supporting. |  | Yes | NA | UE behaviour of supporting inter-RAT EUTRAN measurements without gap is known to network | [Per UE] | No | No | N.A |  | Optional with capability signalling |
| [32-8] | Inter-RAT EUTRAN measurement without gap [and within active DL BWP] (Note: Case B-2) | Support of inter-RAT EUTRAN measurements without gap when CRS is contained within UE’s active DL BWP |  | Yes | No | Measurement gap will be needed for inter-RAT EUTRAN measurements | Per UE | No | FR1 only | N.A |  | Optional with capability signalling |
| [32-9] | Effective measurement window for inter-RAT EUTRAN measurements | Support configuration of effective measurement window for inter-RAT EUTRAN measurements, including offset, duration and periodicity. | [eutra-NeedForGapNCSG-Reporting-r17 for case b-1 or 32-8 for case b-2] | Yes | No | Undefined UE measurement behavior and when to allow scheduling restriction | Per UE | No | No | N.A | - A bitmap for 6 effective measurement window (EMW) patterns defined in TS 38.133. TBD which patterns(s) are mandatory supported | Optional with capability signalling |

Based on the harmonized proposal above, some further considerations can be given as below.

Proposal 2: If no explicit 32-7, for Rel18 UE which supports EMW under case b-1 and case b-2, the eutra-NeedForGapNCSG-Reporting-r17 = {supported} and 32-8 shall be prerequisite respectively.

Proposal 3: When UE was further inquired to report its capability of NeedForGapNCSG-InfoEUTRA,  the indication of “nogap-noncsg” in EUTRAN band is also expected.

# Reference

[1] R4-2321604

[2] R4-2318864

[3] R4-2321993, Rel-18 RAN4 UE feature list for NR (version 2)

# Appendix:

NeedForGapNCSG-InfoEUTRA information element -- ASN1START -- TAG-NEEDFORGAPNCSG-INFOEUTRA-START NeedForGapNCSG-InfoEUTRA-r17 ::= SEQUENCE { needForNCSG-EUTRA-r17 SEQUENCE (SIZE (1..maxBandsEUTRA)) OF NeedForNCSG-EUTRA-r17 } NeedForNCSG-EUTRA-r17 ::= SEQUENCE { bandEUTRA-r17 FreqBandIndicatorEUTRA, gapIndication-r17 ENUMERATED {gap, ncsg, nogap-noncsg} } -- TAG-NEEDFORGAPNCSG-INFOEUTRA-STOP -- ASN1STOP

MeasAndMobParametersCommon ::= SEQUENCE { supportedGapPattern BIT STRING (SIZE (22)) OPTIONAL, ssb-RLM ENUMERATED {supported} OPTIONAL, ssb-AndCSI-RS-RLM ENUMERATED {supported} OPTIONAL, ..., [[ eventB-MeasAndReport ENUMERATED {supported} OPTIONAL, handoverFDD-TDD ENUMERATED {supported} OPTIONAL, eutra-CGI-Reporting ENUMERATED {supported} OPTIONAL, nr-CGI-Reporting ENUMERATED {supported} OPTIONAL ]], [[ independentGapConfig ENUMERATED {supported} OPTIONAL, periodicEUTRA-MeasAndReport ENUMERATED {supported} OPTIONAL, handoverFR1-FR2 ENUMERATED {supported} OPTIONAL, maxNumberCSI-RS-RRM-RS-SINR ENUMERATED {n4, n8, n16, n32, n64, n96} OPTIONAL ]], [[ nr-CGI-Reporting-ENDC ENUMERATED {supported} OPTIONAL ]], [[ eutra-CGI-Reporting-NEDC ENUMERATED {supported} OPTIONAL, eutra-CGI-Reporting-NRDC ENUMERATED {supported} OPTIONAL, nr-CGI-Reporting-NEDC ENUMERATED {supported} OPTIONAL, nr-CGI-Reporting-NRDC ENUMERATED {supported} OPTIONAL ]], [[ reportAddNeighMeasForPeriodic-r16 ENUMERATED {supported} OPTIONAL, condHandoverParametersCommon-r16 SEQUENCE { condHandoverFDD-TDD-r16 ENUMERATED {supported} OPTIONAL, condHandoverFR1-FR2-r16 ENUMERATED {supported} OPTIONAL } OPTIONAL, nr-NeedForGap-Reporting-r16 ENUMERATED {supported} OPTIONAL, supportedGapPattern-NRonly-r16 BIT STRING (SIZE (10)) OPTIONAL, supportedGapPattern-NRonly-NEDC-r16 ENUMERATED {supported} OPTIONAL, maxNumberCLI-RSSI-r16 ENUMERATED {n8, n16, n32, n64} OPTIONAL, maxNumberCLI-SRS-RSRP-r16 ENUMERATED {n4, n8, n16, n32} OPTIONAL, maxNumberPerSlotCLI-SRS-RSRP-r16 ENUMERATED {n2, n4, n8} OPTIONAL, mfbi-IAB-r16 ENUMERATED {supported} OPTIONAL, dummy ENUMERATED {supported} OPTIONAL, nr-CGI-Reporting-NPN-r16 ENUMERATED {supported} OPTIONAL, idleInactiveEUTRA-MeasReport-r16 ENUMERATED {supported} OPTIONAL, idleInactive-ValidityArea-r16 ENUMERATED {supported} OPTIONAL, eutra-AutonomousGaps-r16 ENUMERATED {supported} OPTIONAL, eutra-AutonomousGaps-NEDC-r16 ENUMERATED {supported} OPTIONAL, eutra-AutonomousGaps-NRDC-r16 ENUMERATED {supported} OPTIONAL, pcellT312-r16 ENUMERATED {supported} OPTIONAL, supportedGapPattern-r16 BIT STRING (SIZE (2)) OPTIONAL ]], [[ -- R4 19-2 Concurrent measurement gaps concurrentMeasGap-r17 CHOICE { concurrentPerUE-OnlyMeasGap-r17 ENUMERATED {supported}, concurrentPerUE-PerFRCombMeasGap-r17 ENUMERATED {supported} } OPTIONAL, -- R4 19-1 Network controlled small gap (NCSG) nr-NeedForGapNCSG-Reporting-r17 ENUMERATED {supported} OPTIONAL, eutra-NeedForGapNCSG-Reporting-r17 ENUMERATED {supported} OPTIONAL,