3GPP TSG-RAN WG4 Meeting #110 					                  R4-2401233

Athens, GR, 26 Feb - 01 Mar, 2024

Source:	China Telecom

Title:	Requirements/behavior for UE supporting multiple options

Agenda Item:	8.6.1

Document for:	Discussion

- Introduction

The core part of BWP without restriction WI was completed in RAN #102, and the following issues can be further discussed in the maintenance part according to the approved WF in [1]:

Issues may be discussed during maintenance

Companies are encouraged to bring analysis on the following issues.

- Whether and how to define requirements/UE behaviour for UE supporting both option B-1-1 and A

- Whether and how to define requirements/UE behaviour for UE supporting both option C and A

This contribution discusses the requirements/behaviour for UE supporting multiple options.

- Discussion

#### Requirements/UE behaviour for UE supporting both option B-1-1 and A

The following options are listed in the pre-meeting moderator summary in RAN4 #109:

- Proposals

  - Option 1a: (Vodafone, CMCC, Nokia, Apple, MTK, Ericsson)

    - For UE supporting both option B-1-1 and option A, UE shall perform L1 measurement according to network configuration, there is no need to define requirements/ UE behaviour.

  - Option 1b: (Huawei)

    - For UE supporting both option B-1-1 and A, if configured by NW, UE should perform L1 measurement on both CD-SSB outside active BWP and CSI-RS within active BWP. No additional requirements will be defined for such scenario.

  - Option 2: (vivo)

    - For UE supporting both option B-1-1 and A, if CSI-RS is configured within the active BWP for L1 measurements, then UE is NOT expected to perform RLM/BFD/BM based on CD-SSB outside active BWP.

    - It is clarified in the SSB based L1 measurement requirements that if UE supports both option B-1-1 and option A then the UE is not required to measure the SSB if it is outside active BWP.

  - Option 3: (CATT)

    - No need to define requirements for supporting multiple options.

We also agree that the network will configure the reference signal(s) for RLM/BM/BFD measurements. For UE supporting both option B-1-1 and A, UE shall perform L1 measurement according to network configuration, there is no need to define additional requirements/UE behaviour. And this is the same for the Rel-15 scenario where UE supports both CD-SSB within the active BWP and CSI-RS based L1 measurements.

Proposal 1: For UE supporting both option B-1-1 and option A, UE shall perform L1 measurement according to network configuration, there is no need to define additional requirements/UE behaviour.

#### Requirements/UE behaviour for UE supporting both option C and A

The following options are listed in the pre-meeting moderator summary in RAN4 #109:

- Proposals

  - Option 1a: (Vodafone, CMCC, Nokia, Apple, MTK, Ericsson)

    - For UE supporting both option C and option A, UE shall perform L1 measurement according to network configuration, there is no need to define requirements/ UE behaviour.

  - Option 1b: (vivo, Huawei)

    - For UE supporting both option C and A, both CSI-RS and NCD-SSB within the active BWP can be configured for L1 measurements. No additional requirements/UE behaviour is needed.

  - Option 2: (CATT)

    - No need to define requirements for supporting multiple options.

Similar agreement can be made for UE supporting both option C and A, i.e., for UE supporting both option C and option A, UE shall perform L1 measurement according to network configuration, there is no need to define additional requirements/UE behaviour.

Proposal 2: For UE supporting both option C and option A, UE shall perform L1 measurement according to network configuration, there is no need to define additional requirements/UE behaviour.

- Conclusion

The contribution discussed the requirements/behaviour for UE supporting multiple options, with the following proposals:

Proposal 1: For UE supporting both option B-1-1 and option A, UE shall perform L1 measurement according to network configuration, there is no need to define additional requirements/UE behaviour.

Proposal 2: For UE supporting both option C and option A, UE shall perform L1 measurement according to network configuration, there is no need to define additional requirements/UE behaviour.

- References

- R4- 2321607, WF on BWP without restriction, vivo, Vodafone, RAN4 #109, Nov 2023.