3GPP TSG-RAN WG4 Meeting # 110                                   R4-2401240

# Athens, Greece, February 26 – March 01, 2024

Agenda item:	8.26.2.2

Source:	China Telecom

Title:	Discussion on UE capabilities for NES

Document for:	Discussion

- Introduction

In RAN4 #109 meeting, UE capabilities for inter-band SSB-less SCell activation for NES was discussed and the WF [1] was approved. In this paper, we present our views about UE capabilities for NR network energy saving.

- Discussion

UE capabilities

Status in the WF [1]:

Issue 1-4-1: UE capability indication for inter-band SSB-less - Proposals - Option 1: Introduce per BC UE capability. - Option 2: Introduce per FS UE capability. - Option 3: Introduce per FSPC UE capability. Issue 1-4-2: UE capability indication for A-TRS based inter-band SSB-less SCell activation - Proposals - Option 1: Define a per band UE capability: whether ATRS based inter-band SSB-less SCell activation is supported.

In our views, compared with per BC/per FS/per FSPC UE capability, it’s more reasonable to define per band pair per BC capability. Per band pair per BC capability can indicate which band pair could support inter-band SSB-less operation when the operating band for inter-band CA is larger than two bands, because not all band pairs in the band combination with larger than two bands could support inter-band SSB-less operation, e.g., band pair with large frequency separation cannot support inter-band SSB-less operation.

Observation 1: Not all band pairs in the band combination with larger than two bands could support inter-band SSB-less operation, e.g., band pair with large frequency separation cannot support inter-band SSB-less operation.

Observation 2: Per band pair per BC capability can indicate which band pair could support inter-band SSB-less operation when the operating band for inter-band CA is larger than two bands.

Proposal 1: For FR1 inter-band SSB-less SCell activation, introduce per band pair per BC UE capability.

A-TRS can be used in inter-band SSB-less SCell activation if A-TRS is configured by network, for A-TRS based inter-band SSB-less SCell activation, it’s preferred to define a per band UE capability to indicate whether A-TRS based inter-band SSB-less SCell activation is supported.

Proposal 2: For A-TRS based inter-band SSB-less SCell activation, it’s preferred to define a per band UE capability to indicate whether A-TRS based inter-band SSB-less SCell activation is supported.

- Conclusion

The contribution presents our views on RRM performance requirements for NES, with the following observations and proposals:

Observation 1: Not all band pairs in the band combination with larger than two bands could support inter-band SSB-less operation, e.g., band pair with large frequency separation cannot support inter-band SSB-less operation.

Observation 2: Per band pair per BC capability can indicate which band pair could support inter-band SSB-less operation when the operating band for inter-band CA is larger than two bands.

Proposal 1: For FR1 inter-band SSB-less SCell activation, introduce per band pair per BC UE capability.

Proposal 2: For A-TRS based inter-band SSB-less SCell activation, it’s preferred to define a per band UE capability to indicate whether A-TRS based inter-band SSB-less SCell activation is supported.

- References

- R4-2321562, WF on RRM requirements for NR network energy saving, Huawei, HiSilicon, RAN WG4 Meeting #109, November 2023.