3GPP TSG- RAN4 Meeting # 110	R4-2401339

# Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections on L1-RSRP measurement on candidate cells |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-1-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | - Adding one condition for LTM intra-frequency measurement As per the agreement in [R4-2321621] In RAN4#109 meeting, the LTM intra-frequency layer to be measured on shall be configured as one of L3 MO. “In Rel-18, for LTM L1 measurement, RAN4 RRM requirements are applicable only if L1 measurement layer is configured on the same frequency as one of current L3 MO.” - For L1-RSRP measurement on neighbour cell, UE measures only one intra-frequency layer on each FR2 band in CA scenario[R4-2321621]. The principle shall be captured in spec. - Correction on inter-frequency L1-RSRP measurement without gap: For FR1 UE capable of [capability of measurement with RTD>CP], the LTM inter-frequency L1-RSRP measurement without gap period shall scale Nlayer. For FR1 UE incapable of [capability of measurement with RTD>CP], no need to scale Nlayer, where NLayer = the number of intra-frequency layers configured for L1-RSRP measurement + the number of inter-frequency layers without measurement gaps. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Adding one condition for LTM intra-frequency measurement As per the agreement in [R4-2321621] In RAN4#109 meeting, the LTM intra-frequency layer to be measured on shall be configured as one of L3 MO. “In Rel-18, for LTM L1 measurement, RAN4 RRM requirements are applicable only if L1 measurement layer is configured on the same frequency as one of current L3 MO.” - For L1-RSRP measurement on neighbour cell, UE measures only one intra-frequency layer on each FR2 band in CA scenario[R4-2321621]. The principle shall be captured in spec. - Correction on inter-frequency L1-RSRP measurement without gap: For FR1 UE capable of [capability of measurement with RTD>CP], the LTM inter-frequency L1-RSRP measurement without gap period shall scale Nlayer. For FR1 UE incapable of [capability of measurement with RTD>CP], no need to scale Nlayer, where NLayer = the number of intra-frequency layers configured for L1-RSRP measurement + the number of inter-frequency layers without measurement gaps. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 9.14.2, 9.14.4, 9.15.6.1.1 |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

9.14.2	Requirements Applicability

The requirements in the clause 9.14 are applicable to FR1 and FR2-1 for LTM.

The requirements in clause 9.14 apply, provided for the SSB from the neighbor cell configured for intra-frequency L1-RSRP, the following conditions are met:

-	The SSB configured for intra-frequency L1-RSRP measurement is on the same carrier frequency of SSB configured for RRM measurement in one of the measurement objects configured in the measObjectNR.

-	The cell is known

-	The SSB resources configured for L1-RSRP measurements are measurable.

An SSB resource configured for L1-RSRP for neigbor cell shall be considered measurable when for each relevant SSB the following conditions are met:

-	L1-RSRP related side conditions given in clause [10.1.19] for FR1 and [10.1.20] for FR2-1, respectively, for a corresponding band,

-	SSB_RP and SSB Ês/Iot according to Annex [B.2.4.1] for a corresponding band.

The cell is considered as known if the following conditions are met in this requirement:

-	The UE has performed L3 measurement on the target cell -during the last [X] seconds-, and

-	The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements specified in clause 9.2 and 9.3.

Otherwise, the cell is unknown.

<End of Change 1>

<Start of Change 2>

9.14.4	Number of cells and number of SSB

For each intra-frequency layer, the UE shall be capable of performing L1-RSRP measurements for at least [[TBD] cells/SSBs based on UE capability].

For L1-RSRP measurement on neighbour cell, UE measures only one intra-frequency layer on each FR2 band in CA scenario, where this single intra-frequency layer shall be:

  - PCC when UE is configured with SA NR operation mode with PCC in the band; or

  - PSCC when UE is configured with EN-DC with PSCC in the band; or

  - PSCC when UE is configured with NR-DC with PSCC in the band; or

  - One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation.

<End of Change 2>

<Start of Change 3>

9.15.6	Inter frequency L1-RSRP measurement without measurement gaps

9.15.6.1	Inter frequency L1-RSRP measurement requirements

9.15.6.1.1	Inter-frequency SSB based L1-RSRP measurement

When configured by the network and the inter-frequency cell’s SSB is completely contained in the DL active BWP, the UE shall be able to perform L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_inter_withoutGap.

[If a neighbor cell is known according 9.x.2, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_inter_withoutGap if deriveSSB-IndexFromCellInter-r17 is enabled or UE has reported SSB index in L3 measurement report of the same cell. [Otherwise, UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_inter_withoutGap+TSSB_time_index_inter, where TSSB_time_index_inter is the time period used to acquire the index of the SSB being measured given in Table 9.3.9.1-3 for FR1 and in Table 9.3.9.1-4 for FR2].

The value of TL1-RSRP_Measurement_Period_SSB_inter_withoutGap is defined for FR1 in Table 9.15.6.1.1-1.  The value of TL1-RSRP_Measurement_Period_SSB_inter_withoutGap  is defined for FR2 in Table 9.15.6.1.1-2, where

-	[M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise]

-	N= 8.

-	[P value for SSB resource to be measured is defined as

-	Ntotal / Noutside_MG in FR1

-	Psharing factor * Ntotal / Noutside_MG in FR2 with Navailable = 0

-	Ntotal / Navailable in FR2 with Navailable > 0

For a window W of duration max(TL1,  MGRP_max), where MGRP max is the maximum MGRP across all configured per-UE measurement gaps and per-FR measurement gaps within the same FR as serving cell, and starting at the beginning of any SSB resource occasion:

-	Ntotal is the total number of SSB resource occasions within the window, including those overlapped with measurement gap occasions or SMTC occasions within the window, and

-	Noutside_MG is the number of SSB resource occasions that are not overlapped with any measurement gap occasion within the window W

-	Navailable is the number of SSB resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion within the window W

-	TL1 is periodicity of the target SSB.

-	Psharing factor = 1, if the SSB configured for L1-RSRP measurement outside measurement gap is

-	not overlapped with the SSB symbols indicated by SSB-ToMeasure and 1 data symbol before each consecutive SSB symbols indicated by SSB-ToMeasure and 1 data symbol after each consecutive SSB symbols indicated by SSB-ToMeasure, given that SSB-ToMeasure is configured, where the SSB-ToMeasure is the union set of SSB-ToMeasure from all the configured measurement objects merged on the same serving carrier, and,

-	not overlapped with the RSSI symbols indicated by ss-RSSI-Measurement and 1data symbol before each RSSI symbol indicated by ss-RSSI-Measurement and 1 data symbol after each RSSI symbol indicated by ss-RSSI-Measurement, given that ss-RSSI-Measurement is configured,

-	Psharing factor = 3, otherwise.

-	PL1_sharing is defined as

-	When number of neighboring cells to be measured is 1

-	PL1_sharing = 2, if any symbol of the SSBs from serving cell and neighbor cell are overlapping or adjacent (in time domain)

-	PL1_sharing = 1, otherwise

-	When number of neighboring cells to be measured is more than 1

-	When TCI state of all the intra-frequency neighbor cells are not in the active TCI state list

-	PL1_sharing = 3*NNeighbor_Cell, where NNeighbor_Cell is the number of neighbor cells to measure on intra-frequency and inter-frequency without gap

-	Otherwise

-	PL1_sharing = 3*NNeighbor_Cell_in_list, where NNeighbor_Cell_in_list is the number of neigbour cells (including intra-frequency neighbor cells and inter-frequency without gap neighbor cells) whose TCI state(s) are in the active TCI state list

Note: The above principle and requirements apply when the NW Network is supposed to activate only one TCI state(s) from only one neighbor cell.

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, for cells indicated in the pci-List parameter in smtc2, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.

Longer measurement period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet previous conditions.

For either an FR1 or FR2 cell, longer measurement period would be expected during the period Tidentify_CGI when the UE is requested to decode an NR CGI.

For either an FR1 or FR2 cell, longer L1 RSRP measurement period would be expected during the period Tidentify_CGI,E-UTRAN when the UE is requested to decode an LTE CGI.]

Table 9.15.6.1.1-1: Measurement delay for inter frequency L1-RSRP measurement without measurement gaps in FR1 for UE capable of [capability of measurement with RTD]

| Configuration | TL1-RSRP_Measurement_Period_SSB_inter_withoutGap (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC*NLayer) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC)*NLayer) |
| DRX cycle > 320ms | ceil(M*P)*TDRX*NLayer |
| Note 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. [NLayer = the number of inter-frequency layers which are configured for L1-RSRP measurement with [LTM-CSI-ResourceConfig-r18] and measured without measurement gaps + the number of intra-frequency layers configured for L1-RSRP measurement with [LTM-CSI-ResourceConfig-r18]]. Note 2: K = 1.5. |  |

Table 9.15.6.1.1-2: Measurement delay for inter frequency L1-RSRP measurement without measurement gaps in FR2 for UE capable of [capability of measurement with RTD]

| Configuration | TL1-RSRP_Measurement_Period_SSB_inter_withoutGap (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*PL1_sharing*N)*TSSB_NBC) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*PL1_sharing *N)*max(TDRX,TSSB_NBC) ) |
| DRX cycle > 320ms | ceil(1.5*M*P*PL1_sharing*N)*TDRX |
| Note 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: If the actual RTD of serving cell and neighbor cell is larger than CP, for UE incapable of [capability of measurement with RTD>CP],[accuracy degradation is allowed or no requirements]. Note3: When the number of neighbor cells to be measured on intra-frequency and inter-frequency without gap is more than 1 and TCI state(s) of one of the neighbor cells is in the active TCI state, no requirements for other intra-frequency and inter-frequency without gap neighbor cells except the one whose TCI state(s) is in the active TCI state. |  |

Table 9.15.6.1.1-3: Measurement delay for inter frequency L1-RSRP measurement without measurement gaps in FR1 for UE incapable of [capability of measurement with RTD]

| Configuration | TL1-RSRP_Measurement_Period_SSB_inter_withoutGap (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB_NBC) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1.5. Note 3: No requirements if the actual RTD of serving cell and neighbor cell is larger than CP. |  |

Table 9.15.6.1.1-4: Measurement delay for inter frequency L1-RSRP measurement without measurement gaps in FR2 for UE incapable of [capability of measurement with RTD]

| Configuration | TL1-RSRP_Measurement_Period_SSB_inter_withoutGap (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*PL1_sharing*N)*TSSB_NBC) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*PL1_sharing *N)*max(TDRX,TSSB_NBC) ) |
| DRX cycle > 320ms | ceil(1.5*M*P*PL1_sharing*N)*TDRX |
| Note 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: If the actual RTD of serving cell and neighbor cell is larger than CP, for UE incapable of [capability of measurement with RTD>CP],[accuracy degradation is allowed or no requirements]. Note3: When the number of neighbor cells to be measured on intra-frequency and inter-frequency without gap is more than 1 and TCI state(s) of one of the neighbor cells is in the active TCI state, no requirements for other intra-frequency and inter-frequency without gap neighbor cells except the one whose TCI state(s) is in the active TCI state. |  |