3GPP TSG-RAN WG4 Meeting #110						R4- 2401430

Athens, Greece, 26th February – 1st March 2024

Agenda item:	8.26.3

Source:	Intel Corporation

Title:	Test cases discussions for network energy saving RRM requirements

Document for:	Discussion

# Introduction

Performance part of the work item starts in this meeting. In this paper we provide our views on the test cases list of NES RRM requirements.

# Discussion

## Test cases list for SSB-less SCell activation

Table 1 Test cases list for SSB-less SCell activation

| No. | Item of core requirements | Type of test cases | Frequency range of serving cell | Subclause | Responsibility | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | Inter-band SSB-less SCell activation | TRS based inter-band SSB-less SCell activation delay | FR1 | A.6.5.3.X |  |  |
| 1-2 |  | A-TRS based inter-band SSB-less SCell activation delay | FR1 | A.6.5.3.X |  |  |

In SCell activation delay test cases, there is no difference between inter-band and intra-band non-contiguous CA cases. So we don’t need to test intra-band cases for the same RRM requirements.

## Test cases list for new CHO requirements

Table 2 Test cases list for new CHO requirements

| No. | Item of core requirements | Type of test cases | Frequency range of serving cell | Subclause | Responsibility | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 2-1 | NES triggering intra-frequency target CHO delay | CHO from FR1 to FR1 | FR1 | A.6.3.3.X |  |  |
| 2-2 |  | CHO from FR2 to FR2 | FR2 | A.7.3.3.X |  |  |
| 2-3 | NES triggering inter-frequency target CHO delay within FR | CHO from FR1 to FR1 | FR1 | A.6.3.3.X |  |  |
| 2-4 |  | CHO from FR2 to FR2 | FR2 | A.7.3.3.X |  |  |
| 2-5 | NES triggering inter-frequency target CHO delay across FR | CHO from FR1 to FR2 | FR1 and FR2 | A.7.3.3.X |  |  |

Note that we have never had handover test cases from FR2 to FR1 due to test equipment limitations. We think it is not crucial at this moment that we introduce CHO test case from FR2 to FR1 in the spec.

## Measurement test cases

We don’t think it is necessary to test measurement requirements since so far no new measurement requirements are introduced. However, if there is measurement accuracy requirements specifically introduced for NES in Rel-18 performance part, we can consider to define measurement accuracy test cases.

# Conclusion

In this paper we provide our initial views on RRM aspects that are related to the discussions within WI scope.

Proposal 1: endorse the test case lists in table 1 and table 2.

Table 1 Test cases list for SSB-less SCell activation

| No. | Item of core requirements | Type of test cases | Frequency range of serving cell | Subclause | Responsibility | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 1-1 | Inter-band SSB-less SCell activation | TRS based inter-band SSB-less SCell activation delay | FR1 | A.6.5.3.X |  |  |
| 1-2 |  | A-TRS based inter-band SSB-less SCell activation delay | FR1 | A.6.5.3.X |  |  |

Table 2 Test cases list for new CHO requirements

| No. | Item of core requirements | Type of test cases | Frequency range of serving cell | Subclause | Responsibility | Note |
| --- | --- | --- | --- | --- | --- | --- |
| 2-1 | NES triggering intra-frequency target CHO delay | CHO from FR1 to FR1 | FR1 | A.6.3.3.X |  |  |
| 2-2 |  | CHO from FR2 to FR2 | FR2 | A.7.3.3.X |  |  |
| 2-3 | NES triggering inter-frequency target CHO delay within FR | CHO from FR1 to FR1 | FR1 | A.6.3.3.X |  |  |
| 2-4 |  | CHO from FR2 to FR2 | FR2 | A.7.3.3.X |  |  |
| 2-5 | NES triggering inter-frequency target CHO delay across FR | CHO from FR1 to FR2 | FR1 and FR2 | A.7.3.3.X |  |  |

# References

- RP-223540, New WID: Network energy savings for NR, Huawei