3GPP TSG-RAN WG4 Meeting # 110						R4-2401432

# Athens, Greece, February 26 – March 1, 2024

Source:	LG Electronics

Title:	On feature list of SL-U

Agenda item:	8.22.1.1

Document for:	Approval

# Introduction

In this paper, we provide our views on the default power class and the feature list related to Rel-18 SL-U power class based on the core requirements.

# Discussion

## Issues related RF requirements in 38.101

SL-U UE RF requirements were specified in Rel-18 with the power class 5 being the only defined power class as shown in  Table 6.2E.1F-1 in TS 38.101-1.

Table 6.2E.1F-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n46 |  |  |  |  |  |  | 20 | +2/-3 |
| n96 |  |  |  |  |  |  | 20 | +2/-3 |
| n102 |  |  |  |  |  |  | 20 | +2/-3 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance |  |  |  |  |  |  |  |  |

During the Rel-18 Work Item following agreements were made to ensure that WI can be completed in the given time frame without compromising the quality of the requirements:

- RAN4#106 agreement: Use power class 5 as the default power class for SL-U.

Similar decision was done during the Rel-17 WI for NR-U i.e. complete first the work for power class 5 and use power class 5 as the default power class for the feature, and this was also captured as a note into the specification. For SL-U the note that captures the agreement on default power class is missing and should therefore be added following the same approach that is already used for NR-U.

#### Proposal 1: Add Note 2: Power class 5 is default power class unless otherwise stated into the Table 6.2E.1F-1.

## Issues related to radio resource control (RRC) in 38.331

Currently the power class 5 is not supported for SL in TS 38.331 [2], but power class 2 and power class 3 are supported as follows.

#### SidelinkParameters

The IE SidelinkParameters is used to convey capabilities related to NR and V2X sidelink communications.

……

BandSidelink-r16 ::=  SEQUENCE {

freqBandSidelink-r16                          FreqBandIndicatorNR,

…

[[

ue-PowerClassSidelink-r16                         ENUMERATED {pc2, pc3, spare6, spare5, spare4, spare3, spare2, spare1}

OPTIONAL

]],

…

In order to support the power class 5 for the NR SL-U, a capability of power class per band needs to be defined, for example, ‘ue-PowerClassSidelink-r18’. At least power class 5 should be supported with it. Power class 5 can be default.

BandSidelink Parameters

Table 2-1: Capability of SL power class

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| ue-PowerClassSidelink-r18 This parameter indicates the supported power class for this band used for sidelink. If the field is absent, the UE supports the default power class in TS 38.101-1 [2], Table 6.2E.1F-1. | Band | No | N/A | N/A |

This capability needs to be listed also in Rel-18 UE feature list to ask RAN2 to implement this feature. Proposed addition to NR_SL_enh2 features is shown in Table 2-2.

#### Proposal 2: Add the power class per band capability for Rel-18 SL as in Table 2-1 into UE feature list.

# Conclusion

In this contribution, we propose the default power class and the feature list related to SL-U power class based on the core requirements.

#### Proposal 1: Add Note 2 : Power   class 5 is default power class unless otherwise stated in Table 6.2E.1F-1.

Table 6.2E.1F-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n46 |  |  |  |  |  |  | 20 | +2/-3 |
| n96 |  |  |  |  |  |  | 20 | +2/-3 |
| n102 |  |  |  |  |  |  | 20 | +2/-3 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: Power class 5 is default power class unless otherwise stated |  |  |  |  |  |  |  |  |

#### Proposal 2: Add the power class per band capability for Rel-18 SL as in Table 2-1 into Table 2-2 UE feature list.

Table 2-1: Capability of SL power class

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| ue-PowerClassSidelink-r18 This parameter indicates the supported power class for this band used for sidelink. If the field is absent, the UE supports the default power class in TS 38.101-1 [2], Table 6.2E.1F-1. | Band | No | N/A | N/A |

# NR_SL_enh2

Table 2-2: UE feature list

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 45. # NR_SL_enh2 | 45-3 | Power class for sidelink | This parameter indicates the supported power class for this band used for sidelink. If the field is absent, the UE supports the default power class in TS 38.101-1 [2], Table 6.2E.1F-1. |  | Yes | Yes | UE cannot transmit in proper power class as specified in 38.101-1, e.g., power class 5 | Per Band | No | FR1 only | N/A |  | Optional with capability signalling |

# References

[1] TS 38.101-1 v 18.4.0

[2] TS 38.331-1 v 18.0.0