3GPP TSG-RAN WG4 Meeting #110	R4-2401434

# Athens, Greece, February 26th – March 1st, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	On multi-Rx TRP specific BFD requirements

Agenda item:	8.3.2.3

Document for:	Discussion

Introduction

In this contribution we discuss the conditions for Ptrp-factor in the requirements for TRP-specific BFD.

Discussion

RLM and BFD/CBD requirements are still pending on agreements regarding conditions for scheduling restrictions, measurement restrictions and faster beam sweeping. We have discussed those requirements in our companion contribution under the corresponding agenda item.

What is specific for BFD is the conditions for Ptrp factor. Here we think similar approach can be used as for measurement restrictions. The currently agreed conditions are:

For FR2-1, for PCell, for UE supporting [TBD - multi-rx capability], the value of PTRP in table 8.18.3.2-2 is defined as 1, when: - CSI-RS resources in the two sets q0,0 and q0,1 are not overlapped, or - CSI-RS resources in the two sets q0,0 and q0,1 are overlapped and the following conditions are met: - Both CSI-RSs are not in any CSI-RS resource set with repetition ON - The two CSI-RS resources in the two sets q0,0 and q0,1 for beam failure detection [and both PDSCH] are overlapped on the same OFDM symbol. - [The CSI-RS in set q0,0 has same QCL source as the active TCI state of one PDSCH, and the CSI-RS in set q0,1 has same QCL source as the active TCI state of the other PDSCH] - Resources of the active TCI states for the two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report. - [FFS how to capture UE is activated with multi-Rx operation] - else, the value of PTRP is 2.

We do not think the condition where CSI-RS has to be QCLed to the same beam as PDSCH is needed. We understand that this condition is necessary for scheduling restrictions, but for measurements, it is just important that the CSI-RS to be measured have been reported as a resource pair in group-based report.

Similar as we have proposed in our other contributions, we do not see a need for the condition “UE is activated with multi-Rx operation”. In our view the other conditions already cover the case when the UE is expected to be able to receive with two Rx, and if some of the other conditions does not apply, the requirement stops applying as well. Hence, we propose to remove this condition.

Ptrp=1 under the following conditions:

-	Both CSI-RSs are not in any CSI-RS resource set with repetition ON

-	The two CSI-RS resources in the two sets q0,0 and q0,1  for beam failure detection are overlapped on the same OFDM symbol.

-	The two CSI-RS have been reported as a resource group in Rel-17 group-based RSRP report.

Conclusion

The following observations and proposals were made in this contribution:

Proposal 1: Ptrp=1 under the following conditions:

-	Both CSI-RSs are not in any CSI-RS resource set with repetition ON

-	The two CSI-RS resources in the two sets q0,0 and q0,1  for beam failure detection are overlapped on the same OFDM symbol.

-	The two CSI-RS have been reported as a resource group in Rel-17 group-based RSRP report.

- References

- R4-2317332, WF on core maintenance and performance part for multi-Rx WI, Vivo, Ericsson, RAN4#109 meeting, Chicago, US, 13 - 17 Nov 2023.