3GPP TSG-RAN4 Meeting #110	R4-2401465

# Athens, Greece, February 26 – March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.786 | CR | 0001 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to TR 38.786 NS_28 and NS_30 A-MPR for SL-U |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SL_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Only 5 NS values are specified in Rel-18 SL-U. The remaining NS values needed to be defined to ensure compatibility with the regulative requirements around the world. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add NS_28 and NS_30 A-MPR requirements based on R4-2401464. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The NS_28 and NS_30 values are missing for SL-U. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3.1, 6.1.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< START OF CHANGE #1 >

<<<<<<<<<<< Start of changes in section 6 >>>>>>>>>>

### 6.1.3	UE additional maximum output power reduction for NR SL-U

Based on the NR-U agreement for the additional spectrum emission mask in clause 6.5F.2.3 in TS38.101-1, RAN4 do not consider the additional spectrum emission mask.

For the additional spurious emission requirements for SL-U, RAN4 will consider the NR-U additional spurious emission requirements in clause 6.5F.3.3 in TS38.101-1.

To derive A-MPR requirements for SL-U operation in single CC, RAN4 will assume the basic simulation parameters and assumptions in section 6.1.2 and the additional spurious emission requirements in clause 6.5F.3.3 in TS38.101-1 will be considered.

#### 6.1.3.1	A-MPR for SL-U with NS_28

##### 6.1.3.1.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.1.1.1	Qualcomm’s simulation results (R4-2316791)

Table 6.1.3.1.1.1-1: Simulation results for NS_28 Allocation Full and partial allocations.

| NS_28 A-MPR | 20M | 20M | 40M | 40M | 60M | 60M | 80M | 80M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial | Full | Partial | Full | Partial |
| QPSK | ≤8.0 | ≤8.1 | ≤7.7 | ≤8.0 | ≤7.1 | ≤7.8 | ≤6.9 | ≤7.8 |
| 16 QAM | ≤8.0 | ≤8.3 | ≤7.4 | ≤8.3 | ≤7.3 | ≤8.1 | ≤6.9 | ≤8.1 |
| 64 QAM | ≤8.1 | ≤8.7 | ≤7.4 | ≤8.7 | ≤7.3 | ≤8.2 | ≤7.0 | ≤8.2 |
| 256 QAM | ≤8.2 | ≤8.8 | ≤7.4 | ≤8.8 | ≤7.3 | ≤8.2 | ≤7.1 | ≤8.1 |

The power class 5 PSSCH A-MPR in the tables should be considered in the discussion of MPR requirements.

6.1.3.1.1.2	LG Electronics’ simulation results (R4-2401464)

Table 6.1.3.1.1.2-1 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 6.1.3.1.1.2-1: NS_28-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' (5480) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 5.47 | 5.89 | 5.48 | 5.90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 5.47 | 5.89 | 5.48 | 5.90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 5.47 | 5.90 | 5.47 | 5.90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.90 | 5.48 | 5.90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '20MHz' (5500) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.16 | 2.16 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.16 | 2.16 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.16 | 3.17 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.48 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5490) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 5.06 | 5.47 | 5.47 | 5.89 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 5.06 | 5.47 | 5.47 | 5.89 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 5.06 | 5.48 | 5.47 | 5.90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.48 | 5.47 | 5.89 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5510) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 3.53 | 3.91 | 2.81 | 0.39 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 3.53 | 3.90 | 2.81 | 0.40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.53 | 3.91 | 2.81 | 2.15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5550) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.81 | 0.00 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.81 | 0.40 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.48 | 2.81 | 2.15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '60MHz' (5500) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |
|  | 'QPSK' | 4.67 | 5.07 | 5.47 | 5.90 | 5.06 | 5.48 | 2.13 | 0.11 |
|  | '16QAM' | 4.67 | 5.07 | 5.47 | 5.89 | 5.06 | 5.47 | 2.13 | 0.10 |
|  | '64QAM' | 4.67 | 5.07 | 5.47 | 5.90 | 5.06 | 5.47 | 3.15 | 2.12 |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.89 | 5.47 | 5.47 | 5.47 | 5.05 |
| '60MHz' (5520) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 3.90 | 4.67 | 2.14 | 0.40 | 3.53 | 4.28 | 2.13 | 0.35 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 3.90 | 4.29 | 2.14 | 0.40 | 3.53 | 4.28 | 2.13 | 0.10 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.90 | 4.67 | 3.17 | 2.15 | 3.53 | 4.28 | 3.15 | 2.12 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.88 | 5.05 |  |  |  |  |  |  |  |  |  |  |
| '60MHz' (5540) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.82 | 2.14 | 0.40 | 2.47 | 0.66 | 2.13 | 0.35 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.82 | 2.14 | 0.40 | 2.47 | 0.66 | 2.13 | 0.36 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.17 | 2.82 | 3.17 | 2.15 | 3.17 | 2.48 | 3.16 | 2.12 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.06 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.05 |  |  |  |  |  |  |  |  |  |  |
| '60MHz' (5600) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 0.39 | 2.47 | 0.66 | 2.13 | 0.10 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.14 | 0.40 | 2.47 | 0.66 | 2.13 | 0.36 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 3.17 | 2.48 | 3.17 | 2.15 | 3.16 | 2.47 | 3.16 | 2.12 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.88 | 5.05 |  |  |  |  |  |  |  |  |  |  |
| '80MHz' (5510) | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 4.28 | 4.67 | 5.47 | 5.47 | 5.06 | 5.47 | 4.67 | 5.06 | 2.46 | 0.10 | 2.46 | 3.16 |  |  |  |  |  |  |
|  | '16QAM' | 4.28 | 4.67 | 5.47 | 5.89 | 5.06 | 5.47 | 4.67 | 5.07 | 2.46 | 0.09 | 2.46 | 3.16 |  |  |  |  |  |  |
|  | '64QAM' | 4.28 | 4.67 | 5.47 | 5.47 | 5.06 | 5.07 | 4.67 | 5.06 | 3.16 | 2.12 | 3.16 | 3.15 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.90 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.05 | 5.47 | 5.06 |  |  |  |  |  |  |
| '80MHz' (5530)' | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 3.90 | 4.28 | 2.14 | 0.40 | 3.53 | 4.28 | 3.90 | 4.28 | 2.46 | 0.09 | 2.13 | 0.37 |  |  |  |  |  |  |
|  | '16QAM' | 3.53 | 4.28 | 2.15 | 0.39 | 3.53 | 4.28 | 3.90 | 4.28 | 2.46 | 0.36 | 2.13 | 0.63 |  |  |  |  |  |  |
|  | '64QAM' | 3.53 | 4.28 | 3.16 | 2.14 | 3.53 | 4.28 | 3.90 | 4.67 | 3.15 | 2.12 | 3.16 | 2.46 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.06 | 5.47 | 5.08 | 5.47 | 5.07 | 5.47 | 5.06 | 5.46 | 5.05 | 5.47 | 5.06 |  |  |  |  |  |  |
| '80MHz' (5610)' | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 0.15 | 2.47 | 0.39 | 2.47 | 0.40 | 2.46 | 0.10 | 2.13 | 0.36 |
|  | '16QAM' | 2.48 | 2.48 | 2.14 | 0.40 | 2.47 | 0.39 | 2.47 | 0.66 | 2.46 | 0.09 | 2.13 | 0.64 |
|  | '64QAM' | 2.82 | 2.48 | 3.16 | 2.15 | 2.81 | 2.48 | 3.16 | 2.48 | 3.15 | 2.12 | 3.16 | 2.46 |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.08 | 5.47 | 5.07 | 5.47 | 5.06 | 5.46 | 5.06 | 5.46 | 5.06 |

Table 6.1.3.1.1.2-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 6.1.3.1.1.2-2: NS_28-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | RB Allocation /Centre frequency of CBW (MHz) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz : 5480 40MHz : 5490/5510 60MHz : 5500/5520 80MHz : 5510/5530 |  |  |  | 20MHz : 5500 40MHz : 5550 60MHz : 5540/5600 80MHz : 5610 |  |  |  |
|  |  | Outer RB set configuration |  | Inner RB set configuration |  | Outer RB set configuration |  | Inner RB set configuration |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | 5.48 | 5.90 | 2.46 | 3.16 | 2.81 | 2.82 | 2.46 | 0.36 |
|  | 16 QAM | 5.48 | 5.90 | 2.46 | 3.16 | 2.81 | 2.82 | 2.46 | 0.64 |
|  | 64 QAM | 5.47 | 5.90 | 3.16 | 3.15 | 3.17 | 2.82 | 3.16 | 2.46 |
|  | 256 QAM | 5.48 | 5.90 | 5.47 | 5.06 | 5.48 | 5.08 | 5.88 | 5.06 |

The following centre frequencies for each CBW can be assumed to have same A-MPR in Table 6.1.3.1.1.2-2 (left side A-MPR in Table 6.1.3.1.1.2-2) because the distance from the boundary of regulation ‘-30dBm/MHz’ is effectively the same.

- 20MHz

: A-MPR of 5480 MHz = A-MPR of 5160/5340/5700 MHz

- 40MHz

: A-MPR of 5490 MHz = A-MPR of 5170/5330 MHz

: A-MPR of 5510 MHz = A-MPR of 5190/5310 MHz

- 60MHz

: A-MPR of 5500 MHz = A-MPR of 5180/5320 MHz

: A-MPR of 5520 MHz = A-MPR of 5200/5300/5680 MHz

- 80MHz

: A-MPR of 5510 MHz = A-MPR of 5190/5310 MHz

: A-MPR of 5530 MHz = A-MPR of 5210/5290 MHz

Considering implementation margin and the centre frequencies on the right side of A-MPR in Table Table 6.1.3.1.1.2-2, such as, 5500MHz for CBW 20MHz, 5550MHz for CBW 40MHz, 5540/5600MHz for CBW 60MHz, and 5610MHz for CBW 80MHz, the SL-U PSSCH/PSCCH MPR requirement in Table 6.2E.2F-1 of TS 38.101-1 can be reused as A-MPR requirement.

As a result, considering implementation margin and the centre frequency, Table 6.1.3.1.1.2-3 can be proposed for SL-U NS_28 PSSCH/PSCCH A-MPR.

Table 6.1.3.1.1.2-3. NS_28 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation (Note 3) |  |  |  | RB Allocation (Note 4) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration5 |  | Inner RB set configuration5 |  |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 7.0 | ≤ 4.0 | ≤ 5.0 | Table 6.2E.2F-1 |
|  | 16 QAM | ≤ 6.0 | ≤ 7.5 | ≤ 4.0 | ≤ 5.0 |  |
|  | 64 QAM | ≤ 6.5 | ≤ 7.5 | ≤ 4.5 | ≤ 5.5 |  |
|  | 256 QAM | ≤ 7.5 | ≤ 8.0 | ≤ 7.5 | ≤ 7.5 |  |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, 5480, and 5700 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, 5490, and 5510 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5300, 5320, 5500, 5520, 5680 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 4: Applicable for all valid channels other than those enumerated under NOTE 3. NOTE 5: Contiguous outer sub-band configuration and contiguous inner sub-band configuration in Table 6.1.2.1.1.1-5 apply. NOTE 6: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |  |

##### 6.1.3.1.2	A-MPR for S-SSB transmission

6.1.3.1.2.1	LG Electronics’ simulation results (R4-2401464)

Table 6.1.3.1.2.1-1 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 6.1.3.1.2.1-1: NS_28-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ (5160) | 10.07 | 8.52 | 7.33 | 7.83 | 6.90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’ (5180) | 8.75 | 7.32 | 5.53 | 6.45 | 5.71 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’ (5200) | 8.82 | 7.14 | 5.60 | 6.34 | 5.62 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’ (5700) | 8.80 | 7.17 | 7.81 | 6.46 | 5.54 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5170) | 10.98 | 9.88 | 7.77 | 9.86 | 8.73 | 7.43 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5190) | 9.35 | 8.21 | 6.09 | 8.70 | 7.23 | 5.53 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5230) | 9.17 | 8.31 | 6.01 | 8.80 | 7.21 | 5.70 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5670) | 9.11 | 8.13 | 5.99 | 8.58 | 7.28 | 5.63 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5180) | 9.55 | 8.22 | 8.97 | 7.11 | 8.06 | 6.83 | 6.37 | 5.05 | 8.24 | 7.43 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5200) | 8.30 | 7.10 | 7.27 | 6.23 | 6.52 | 5.53 | 6.26 | 5.10 | 8.33 | 7.36 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5220) | 7.95 | 7.03 | 7.28 | 6.17 | 6.51 | 5.65 | 6.46 | 5.04 | 8.46 | 7.15 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5680) | 8.95 | 8.16 | 7.09 | 6.14 | 6.34 | 5.55 | 6.41 | 5.07 | 8.13 | 7.27 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' (5190) | 10.09 | 7.97 | 9.31 | 7.64 | 8.65 | 7.20 | 7.63 | 6.92 | 7.34 | 5.79 | 6.31 | 4.97 | 8.96 | 7.35 | 8.23 | 7.20 | 8.15 | 7.52 |  |  |
| '80MHz' (5210) | 8.69 | 7.40 | 7.31 | 6.27 | 7.14 | 6.18 | 6.36 | 5.40 | 7.48 | 5.90 | 6.35 | 5.03 | 8.40 | 7.35 | 8.12 | 7.09 | 8.33 | 6.98 |  |  |
| '80MHz' (5610) | 7.48 | 6.82 | 7.47 | 6.35 | 7.14 | 6.19 | 6.30 | 5.52 | 7.51 | 6.02 | 6.48 | 4.84 | 8.66 | 7.36 | 8.28 | 6.96 | 7.13 | 6.85 |  |  |

Table 6.1.3.1.2.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 6.1.3.1.2.1-2: NS_28-S-SSB A-MPR simulation results for SL-U power class 5

|  | RB Allocation /Centre frequency of CBW (MHz)/ (dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz : 5160 40MHz : 5170/5190/5310 60MHz :5180/5200/5220/5520/ 5540/ 5680 80MHz : 5190/5210/5530/5610 |  |  |  | 20MHz : 5180/5200/5700 40MHz : 5230/5510/5670 |  |  |  |
|  | Outer RB set configuration |  | Inner RB set configuration |  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/ Non-contiguous sub-band RB sets | 10.98 | 8.22 | 7.51 | 6.02 | 9.17 | 7.81 | - | - |

The following centre frequencies for each CBW can be assumed to have same A-MPR in Table 6.1.3.1.2.1-2 (left side A-MPR in Table 6.1.3.1.2.1-2) because the distance from the boundary of the regulation ‘-30dBm/MHz’ is effectively the same.

- 20MHz

: A-MPR of 5160 MHz = A-MPR of 5340/5480 MHz

: A-MPR of 5180 MHz = A-MPR of 5320 MHz

- 40MHz

: A-MPR of 5170 MHz = A-MPR of 5330/5490 MHz

: A-MPR of 5190MHz = A-MPR of 5310/5510 MHz

- 60MHz

: A-MPR of 5180 MHz = A-MPR of 5320/5500 MHz

: A-MPR of 5200 MHz = A-MPR of 5300/5520MHz

: A-MPR of 5220 MHz = A-MPR of 5280/5540MHz

- 80MHz

: A-MPR of 5190 MHz = A-MPR of 5310/5510 MHz

: A-MPR of 5210 MHz = A-MPR of 5290/5530 MHz

Considering implementation margin, Table 6.1.3.1.2.1-3 shows proposal for SL-U NS_28 S-SSB A-MPR.

Table 6.1.3.1.2.1-3: NS_28 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation2 |  |  |  | RB Allocation3 |
| --- | --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  | Outer/Inner RB sets |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 |  |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 13.5 | ≤ 10.0 | ≤ 10.0 | ≤ 8.5 | Table 6.2E.2F-5(TS38.101-1) |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5965 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5975 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5985 MHz. NOTE 3: Applicable for all valid channels and bandwidths other than those enumerated under NOTE 2. NOTE 5: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |

##### 6.1.3.1.3	A-MPR for PSFCH transmission

6.1.3.1.3.1	LG Electronics’ simulation results (R4-2401464)

Table 6.1.3.1.3.1-1 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 6.1.3.1.3.1-1: NS_28-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’(5160) | 9.39 | 9.11 |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’(5180) | 6.62 | 7.50 |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’(5200) | 6.62 | 7.50 |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’(5700) | 6.62 | 7.50 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5170) | 10.59 | 9.37 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5190) | 9.08 | 6.53 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5230) | 7.85 | 6.53 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5510) | 8.00 | 6.53 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5670) | 7.85 | 6.53 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |
| ‘60MHz’(5180) | 11.17 | 10.29 | 9.10 | 7.00 | 9.71 |  |  |  |  |  |  |  |
| ‘60MHz’(5200) | 9.83 | 8.65 | 7.30 | 7.00 | 9.71 |  |  |  |  |  |  |  |
| ‘60MHz’(5220) | 9.69 | 8.65 | 7.30 | 7.00 | 9.40 |  |  |  |  |  |  |  |
| ‘60MHz’(5520) | 9.69 | 8.65 | 7.30 | 7.00 | 9.71 |  |  |  |  |  |  |  |
| ‘60MHz’(5540) | 9.69 | 8.65 | 7.30 | 7.00 | 9.40 |  |  |  |  |  |  |  |
| ‘60MHz’(5680) | 10.53 | 8.65 | 7.30 | 7.00 | 9.73 |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |
| '80MHz'(5190) | 11.61 | 10.91 | 10.25 | 9.00 | 9.96 | 7.02 | 10.59 | 9.69 | 9.66 |  |  |  |
| '80MHz'(5210) | 11.16 | 10.05 | 8.71 | 7.43 | 9.96 | 7.02 | 10.17 | 9.69 | 9.66 |  |  |  |
| '80MHz'(5530) | 10.43 | 10.01 | 8.43 | 7.43 | 9.96 | 7.02 | 10.17 | 9.69 | 9.66 |  |  |  |
| '80MHz'(5610) | 10.43 | 10.01 | 8.43 | 7.43 | 9.96 | 7.02 | 10.17 | 9.26 | 8.73 |  |  |  |

Table 6.1.3.1.3.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 6.1.3.1.3.1-2: NS_28-PSFCH A-MPR simulation results for SL-U power class 5

| RB set configuration | RB Allocation /Centre frequency of CBW (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 20MHz : 5160 40MHz : 5170/5190/5310 60MHz :5180/5200/5220/5520/ 5540/ 5680 80MHz : 5190/5210/5530/5610 |  | 20MHz : 5180/5200 40MHz : 5230/5510/5670 |  |
|  | Outer RB set configuration (Full/Partial) | Inner RB set configuration (Full/Partial) | Outer RB set configuration (Full/Partial) | Inner RB set configuration (Full/Partial) |
| Contiguous/Non-contiguous sub-band RB sets | 12.23 | 9.96 | 10.2 | - |

The following centre frequencies for each CBW can be assumed to have same A-MPR in Table 6.1.3.1.3.1-2 (left side A-MPR in Table 6.1.3.1.3.1-2) because the distance from the boundary of regulation ‘-30dBm/MHz’ is effectively the same.

- 20MHz

: A-MPR of 5160 MHz = A-MPR of 5340/5480 MHz

: A-MPR of 5180 MHz = A-MPR of 5320 MHz

- 40MHz

: A-MPR of 5170 MHz = A-MPR of 5330/5490 MHz

: A-MPR of 5190MHz = A-MPR of 5310/5510 MHz

- 60MHz

: A-MPR of 5180 MHz = A-MPR of 5320/5500 MHz

: A-MPR of 5200 MHz = A-MPR of 5300/5520MHz

: A-MPR of 5220 MHz = A-MPR of 5280/5540MHz

- 80MHz

: A-MPR of 5190 MHz = A-MPR of 5310/5510 MHz

: A-MPR of 5210 MHz = A-MPR of 5290/5530 MHz

Considering implementation margin, Table 6.1.3.1.3.1-3 shows proposal for SL-U NS_28 PSFCH A-MPR.

Table 6.1.3.1.3.1-3: NS_28 PSFCH A-MPR for SL-U UE power class 5

|  | RB Allocation2 |  | RB Allocation3 |
| --- | --- | --- | --- |
|  | Outer RB set configuration | Inner RB set configuration | Outer/Inner RB sets |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 15.0 | ≤ 12.5 | Table 6.2E.2F-4(TS38.101-1) |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, and 5480 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, and 5490 MHz, 60 MHz hannels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5220, 5280, 5300, 5320, 5500, 5520, 5540, 5680 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, 5530, and 5610 MHz. NOTE 3: Applicable for all valid channels other than those enumerated under NOTE 3. NOTE 5: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |

#### 6.1.3.2	A-MPR for SL-U with NS_29

##### 6.1.3.2.1	A-MPR for simultaneous PSSCH/PSCCH transmission

##### 6.1.3.2.2	A-MPR for S-SSB transmission

##### 6.1.3.2.3	A-MPR for PSFCH transmission

#### 6.1.3.3	A-MPR for SL-U with NS_30

##### 6.1.3.3.1	A-MPR for simultaneous PSSCH/PSCCH transmission

6.1.3.3.1.1	Qualcomm’s simulation results (R4-2316791)

Table 6.1.3.3.1.1-1: Simulation results for NS_30 Allocation Note 2 Full and partial allocations.

| NS_30 A-MPR Allocation Note2 | 20M | 20M | 40M | 40M | 60M | 60M | 80M | 80M |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial | Full | Partial | Full | Partial |
| QPSK | ≤10.7 | ≤10.9 | ≤10.4 | ≤11.0 | ≤9.3 | ≤10.3 | ≤9.1 | ≤10.4 |
| 16 QAM | ≤10.8 | ≤9.0 | ≤10.1 | ≤11.1 | ≤9.6 | ≤10.5 | ≤9.2 | ≤10.5 |
| 64 QAM | ≤10.9 | ≤11.4 | ≤10.1 | ≤11.4 | ≤9.6 | ≤10.6 | ≤9.3 | ≤10.6 |
| 256 QAM | ≤11.0 | ≤11.5 | ≤10.0 | ≤11.5 | ≤9.7 | ≤10.6 | ≤9.4 | ≤10.5 |

Table 6.1.3.3.1.1-2: Simulation results for NS_30 Allocation Note 3 Full and partial allocations.

| NS_30 A-MPR Allocation Note3 | 20M | 20M | 40M | 40M |
| --- | --- | --- | --- | --- |
|  | Full | Partial | Full | Partial |
| QPSK | ≤8.3 | ≤8.4 | ≤5.9 | ≤6.2 |
| 16 QAM | ≤8.1 | ≤8.7 | ≤5.8 | ≤6.3 |
| 64 QAM | ≤8.3 | ≤9.0 | ≤5.8 | ≤6.3 |
| 256 QAM | ≤8.4 | ≤8.9 | ≤5.8 | ≤6.3 |

The power class 5 PSSCH A-MPR in the tables should be considered in the discussion of MPR requirements.

6.1.3.3.1.2	LG Electronics’ simulation results (R4-2401464)

Table 6.1.3.3.1.2-1 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 6.1.3.3.1.2-1: NS_30-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| '20MHz' (5160) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 'QPSK' | 8.51 | 8.52 | 8.97 | 9.44 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 8.50 | 8.52 | 8.96 | 8.98 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 8.51 | 8.51 | 8.96 | 9.44 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 8.51 | 8.97 | 8.96 | 9.90 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '20MHz' (5180) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.82 | 2.83 | 3.90 | 4.29 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.82 | 2.82 | 3.54 | 4.29 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.83 | 3.91 | 3.92 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.08 | 5.48 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '20MHz' (5240) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.17 | 2.16 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.16 | 2.16 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.49 | 3.17 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.08 | 5.89 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '20MHz' (5260) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.16 | 2.16 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.16 | 2.16 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.16 | 3.17 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.08 | 5.48 | 5.09 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '20MHz' (5280) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.16 | 2.15 | 2.49 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.16 | 2.16 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.49 | 3.17 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.08 | 5.89 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '20MHz' (5480) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |
|  | 'QPSK' | 7.17 | 7.62 | 8.06 | 8.07 |  |  |  |  |
|  | '16QAM' | 7.17 | 7.63 | 8.06 | 8.07 |  |  |  |  |
|  | '64QAM' | 7.17 | 7.62 | 8.06 | 8.07 |  |  |  |  |
|  | '256QAM' | 7.17 | 7.62 | 8.06 | 8.07 |  |  |  |  |
| '20MHz' (5550) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.16 | 2.16 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.17 | 2.16 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.16 | 3.18 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.89 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '20MHz' (5680) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.17 | 2.16 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.17 | 2.16 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.17 | 3.18 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '20MHz' (5700) | Scenario # | #1 | #7 | #2 | #8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.16 | 2.16 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.16 | 2.16 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.16 | 3.17 | 2.50 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.08 | 5.48 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5170) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 7.60 | 8.06 | 8.50 | 8.51 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 7.60 | 8.06 | 8.50 | 8.52 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 7.60 | 8.06 | 8.50 | 8.51 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 7.60 | 8.06 | 8.50 | 8.51 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5190) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 6.30 | 6.74 | 2.81 | 3.17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 6.30 | 6.73 | 2.81 | 3.17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 6.30 | 6.74 | 2.81 | 3.17 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 6.30 | 6.74 | 5.47 | 5.08 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5230) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.81 | 0.00 |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.81 | 0.40 |  |  |  |  |
|  | '64QAM' | 2.82 | 2.48 | 2.81 | 2.15 |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.08 |  |  |  |  |
| '40MHz' (5490) | Scenario # | #3 | #9 | #13 | #30 |
|  | 'QPSK' | 7.17 | 7.61 | 7.17 | 7.61 |
|  | '16QAM' | 7.16 | 7.61 | 7.17 | 7.61 |
|  | '64QAM' | 7.16 | 7.61 | 7.17 | 7.62 |
|  | '256QAM' | 7.16 | 7.61 | 7.17 | 7.62 |
| '40MHz' (5510) | Scenario # | #3 | #9 | #13 | #30 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 4.66 | 5.07 | 2.47 | 1.22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 4.67 | 5.07 | 2.81 | 1.22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 4.67 | 5.47 | 2.81 | 2.15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| '40MHz' (5550) | Scenario # | #3 | #9 | #13 | #30 |
|  | 'QPSK' | 2.48 | 2.48 | 2.81 | 0.00 |
|  | '16QAM' | 2.48 | 2.48 | 2.81 | 0.40 |
|  | '64QAM' | 2.82 | 2.48 | 2.81 | 2.15 |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 |
| '60MHz' (5180) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |
|  | 'QPSK' | 8.05 | 8.06 | 8.96 | 9.90 | 8.50 | 8.96 | 2.46 | 2.46 |
|  | '16QAM' | 8.05 | 8.05 | 8.96 | 9.90 | 8.50 | 8.97 | 2.46 | 2.79 |
|  | '64QAM' | 8.05 | 8.06 | 8.96 | 9.43 | 8.50 | 9.42 | 3.16 | 2.80 |
|  | '256QAM' | 8.05 | 8.06 | 8.96 | 9.89 | 8.50 | 8.97 | 5.47 | 5.06 |
| '60MHz' (5200) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |
|  | 'QPSK' | 6.73 | 7.17 | 3.90 | 4.28 | 6.73 | 7.17 | 2.13 | 0.10 |
|  | '16QAM' | 6.73 | 7.17 | 3.90 | 4.29 | 6.30 | 7.17 | 2.13 | 0.36 |
|  | '64QAM' | 6.73 | 7.17 | 3.90 | 4.67 | 6.31 | 7.17 | 3.16 | 2.12 |
|  | '256QAM' | 6.73 | 7.17 | 5.47 | 5.07 | 6.30 | 7.17 | 5.88 | 5.05 |
| '60MHz' (5220) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |
|  | 'QPSK' | 5.06 | 5.88 | 2.14 | 2.15 | 2.47 | 2.47 | 2.13 | 0.10 |
|  | '16QAM' | 5.06 | 5.89 | 2.14 | 2.15 | 2.47 | 2.48 | 2.13 | 0.36 |
|  | '64QAM' | 5.06 | 5.89 | 3.16 | 2.15 | 3.17 | 2.48 | 3.16 | 2.13 |
|  | '256QAM' | 5.47 | 5.89 | 5.48 | 5.07 | 5.47 | 5.07 | 5.88 | 5.06 |
| '60MHz' (5280) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |
|  | 'QPSK' | 4.67 | 5.48 | 2.14 | 0.14 | 2.47 | 0.66 | 2.13 | 0.36 |
|  | '16QAM' | 4.67 | 5.48 | 2.14 | 0.40 | 2.47 | 0.65 | 2.13 | 0.35 |
|  | '64QAM' | 4.67 | 5.47 | 3.17 | 2.16 | 2.81 | 2.47 | 3.16 | 2.12 |
|  | '256QAM' | 5.47 | 5.47 | 5.47 | 5.07 | 5.47 | 5.07 | 5.88 | 5.06 |
| '60MHz' (5500) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |
|  | 'QPSK' | 7.17 | 7.61 | 8.05 | 8.51 | 7.61 | 8.06 | 2.13 | 1.19 |
|  | '16QAM' | 7.17 | 7.61 | 8.05 | 8.51 | 7.60 | 8.06 | 2.13 | 1.49 |
|  | '64QAM' | 7.17 | 7.61 | 8.05 | 8.51 | 7.61 | 8.06 | 3.16 | 2.12 |
|  | '256QAM' | 7.17 | 7.61 | 8.05 | 8.51 | 7.61 | 8.06 | 5.47 | 5.05 |
| '60MHz' (5520) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |  |  |  |  |  |  |  |  |  |  |
|  | 'QPSK' | 5.88 | 6.31 | 2.47 | 3.17 | 4.66 | 5.07 | 2.13 | 0.10 |  |  |  |  |  |  |  |  |  |  |
|  | '16QAM' | 5.88 | 6.74 | 2.48 | 3.17 | 4.67 | 5.07 | 2.13 | 0.35 |  |  |  |  |  |  |  |  |  |  |
|  | '64QAM' | 5.88 | 6.74 | 3.17 | 3.17 | 4.66 | 5.07 | 3.16 | 2.12 |  |  |  |  |  |  |  |  |  |  |
|  | '256QAM' | 5.88 | 6.31 | 5.47 | 5.08 | 5.47 | 5.07 | 5.47 | 5.06 |  |  |  |  |  |  |  |  |  |  |
| '60MHz' (5540) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |
|  | 'QPSK' | 3.17 | 3.90 | 2.14 | 0.39 | 2.47 | 1.22 | 2.13 | 0.10 |
|  | '16QAM' | 3.17 | 3.90 | 2.14 | 0.40 | 2.47 | 1.22 | 2.13 | 0.36 |
|  | '64QAM' | 3.17 | 3.90 | 3.17 | 2.14 | 3.17 | 2.47 | 3.16 | 2.12 |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.88 | 5.06 |
| '60MHz' (5680) | Scenario # | #4 | #10 | #14 | #31 | #15 | #32 | #16 | #33 |
|  | 'QPSK' | 3.17 | 3.90 | 2.14 | 0.15 | 2.47 | 0.66 | 2.13 | 0.10 |
|  | '16QAM' | 3.17 | 3.90 | 2.15 | 0.40 | 2.47 | 0.66 | 2.13 | 0.36 |
|  | '64QAM' | 3.17 | 3.91 | 3.16 | 2.15 | 3.17 | 2.48 | 3.16 | 2.13 |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.05 |
| '80MHz' (5190) | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |
|  | 'QPSK' | 7.60 | 7.61 | 8.96 | 8.98 | 8.50 | 8.96 | 8.05 | 8.05 | 2.46 | 3.15 | 5.88 | 6.30 |  |  |  |
|  | '16QAM' | 7.60 | 7.61 | 8.51 | 9.90 | 8.50 | 8.96 | 8.05 | 8.06 | 2.80 | 3.15 | 5.88 | 6.30 |  |  |  |
|  | '64QAM' | 7.60 | 7.61 | 8.96 | 9.89 | 8.96 | 8.96 | 8.05 | 8.05 | 3.16 | 3.15 | 5.88 | 6.30 |  |  |  |
|  | '256QAM' | 7.61 | 7.61 | 8.51 | 9.90 | 8.50 | 8.96 | 8.05 | 8.05 | 5.46 | 5.06 | 5.88 | 6.30 |  |  |  |
| '80MHz' (5210) | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |
|  | 'QPSK' | 6.73 | 7.17 | 3.90 | 4.29 | 6.30 | 7.17 | 6.73 | 7.17 | 2.46 | 0.10 | 2.13 | 2.13 |
|  | '16QAM' | 6.73 | 7.17 | 3.90 | 4.28 | 6.30 | 7.17 | 6.73 | 7.17 | 2.46 | 0.10 | 2.13 | 2.13 |
|  | '64QAM' | 6.73 | 7.17 | 3.53 | 4.29 | 6.31 | 7.17 | 6.73 | 7.17 | 3.15 | 2.12 | 2.81 | 2.47 |
|  | '256QAM' | 6.73 | 7.17 | 5.47 | 5.07 | 6.30 | 7.17 | 6.73 | 7.17 | 5.47 | 5.06 | 5.47 | 5.06 |
| '80MHz' (5510) | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |
|  | 'QPSK' | 7.16 | 7.17 | 8.05 | 8.07 | 7.60 | 8.06 | 7.17 | 7.61 | 2.46 | 1.80 | 4.27 | 4.66 |
|  | '16QAM' | 7.16 | 7.17 | 8.06 | 8.07 | 7.61 | 7.61 | 7.17 | 7.61 | 2.46 | 1.80 | 4.66 | 4.66 |
|  | '64QAM' | 7.17 | 7.17 | 8.06 | 8.07 | 7.61 | 8.05 | 7.16 | 7.61 | 3.15 | 2.12 | 4.27 | 4.66 |
|  | '256QAM' | 7.17 | 7.17 | 8.05 | 8.06 | 7.61 | 8.05 | 7.17 | 7.61 | 5.46 | 5.05 | 5.47 | 5.06 |
| '80MHz' (5530) | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |
|  | 'QPSK' | 6.30 | 6.73 | 2.14 | 2.15 | 4.67 | 5.06 | 5.88 | 6.30 | 2.46 | 0.10 | 2.13 | 0.91 |
|  | '16QAM' | 6.30 | 6.73 | 2.14 | 2.15 | 4.67 | 5.07 | 5.88 | 6.31 | 2.46 | 0.09 | 2.13 | 0.91 |
|  | '64QAM' | 6.30 | 6.73 | 3.17 | 2.15 | 4.67 | 5.07 | 5.88 | 6.31 | 3.15 | 2.13 | 2.80 | 2.46 |
|  | '256QAM' | 6.30 | 6.73 | 5.47 | 5.08 | 5.47 | 5.07 | 5.88 | 6.31 | 5.47 | 5.06 | 5.47 | 5.06 |
| '80MHz' (5610) | Scenario # | #5 | #11 | #17 | #34 | #18 | #35 | #19 | #36 | #20 | #37 | #21 | #38 |  |  |  |  |  |  |
|  | 'QPSK' | 2.48 | 2.48 | 2.14 | 0.14 | 2.47 | 0.40 | 2.48 | 0.40 | 2.46 | 0.10 | 2.13 | 0.37 |  |  |  |  |  |  |
|  | '16QAM' | 2.48 | 2.48 | 2.14 | 0.40 | 2.47 | 0.66 | 2.47 | 0.66 | 2.46 | 0.36 | 2.13 | 0.37 |  |  |  |  |  |  |
|  | '64QAM' | 2.82 | 2.48 | 3.16 | 2.15 | 2.81 | 2.47 | 3.17 | 2.48 | 3.15 | 2.12 | 3.16 | 2.46 |  |  |  |  |  |  |
|  | '256QAM' | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.07 | 5.47 | 5.05 | 5.47 | 5.06 |  |  |  |  |  |  |

Table 6.1.3.3.1.2-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 6.1.3.3.1.2-2 : NS_30-PSSCH/PSCCH A-MPR simulation results for SL-U power class 5

| Pre-coding | Modulation | RB Allocation /Centre frequency of CBW (MHz) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20MHz : 5160/5480 40MHz : 5170/5190/5490 60MHz : 5180/5200/5500/5520 80MHz : 5190/5210/5510/5530 |  |  |  | 20MHz : 5180 40MHz : 5510 60MHz : 5220/5280/5540/5680 80MHz : |  |  |  |
|  |  | Outer RB set configuration |  | Inner RB set configuration |  | Outer RB set configuration |  | Inner RB set configuration |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| CP-OFDM | QPSK | 12.74 | 14.21 | 5.88 | 6.30 | 5.06 | 5.88 | 2.13 | 0.36 |
|  | 16 QAM | 12.74 | 14.21 | 5.88 | 6.30 | 5.06 | 5.89 | 2.13 | 0.36 |
|  | 64 QAM | 12.26 | 14.21 | 5.88 | 6.30 | 5.06 | 5.89 | 3.16 | 2.13 |
|  | 256 QAM | 12.74 | 14.21 | 5.88 | 6.30 | 5.48 | 5.89 | 5.88 | 5.06 |

The following centre frequencies for each CBW can be assumed to have same A-MPR results in Table 6.1.3.3.1.2-1 (left side A-MPR in Table 6.1.3.3.1.2-1) because the distance from ‘-41dBm/MHz’ is effectively the same.

- 20MHz

: A-MPR of 5160 MHz = A-MPR of 5340/5480MHz

- 40MHz

: A-MPR of 5170MHz = A-MPR of 5330/5490 MHz

: A-MPR of 5190 MHz = A-MPR of 5310 MHz

- 60MHz

: A-MPR of 5180MHz = A-MPR of 5320/5500 MHz

: A-MPR of 5200 MHz = A-MPR of 5300/5520MHz

- 80MHz

: A-MPR of 5190 MHz = A-MPR of 5310/5510 MHz

: A-MPR of 5210 MHz = A-MPR of 5290/5530 MHz

The following centre frequencies for each CBW can be assumed to have same A-MPR results in Table 6.1.3.3.1.2-1 (right side A-MPR in Table 6.1.3.3.1.2-1) because the distance from ‘-41dBm/MHz’ is effectively the same.

- 20MHz

: A-MPR of 5180 MHz = A-MPR of 5320 MHz

- 60MHz

: A-MPR of 5220 MHz = A-MPR of 5280/5540MHz

For other centre frequencies from Table 6.1.3.3.1.2-1, considering implementation margin, SL-U PSSCH/PSCCH MPR requirement in Table 6.2E.2F-1 of TS 38.101-1 can be reused as A-MPR requirement.

As a result, considering implementation margin and the centre frequencies, Table 6.1.3.3.1.2-3 shows proposal for SL-U NS_30 PSSCH/PSCCH A-MPR.

Table 6.1.3.3.1.2-3 : NS_30 PSSCH/PSCCH A-MPR for SL-U UE power class 5

| Pre-coding | Modulation | RB Allocation3 |  |  |  | RB Allocation4 |  |  |  | RB Allocation (Note 5) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer RB set configuration |  | Inner RB set configuration |  | Outer RB set configuration |  | Inner RB set configuration |  |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full/Partial |
| CP-OFDM | QPSK | ≤ 10.0 | ≤ 12.0 | ≤ 7.0 | ≤ 7.5 | ≤ 6.0 | ≤ 7.0 | ≤ 3.0 | ≤ 2.0 | Table 6.2E.2F-1 |
|  | 16 QAM | ≤ 10.0 | ≤ 12.5 | ≤ 7.0 | ≤ 7.5 | ≤ 6.0 | ≤ 7.0 | ≤ 3.0 | ≤ 2.0 |  |
|  | 64 QAM | ≤ 10.0 | ≤ 13.0 | ≤ 7.0 | ≤ 7.5 | ≤ 6.0 | ≤ 7.0 | ≤ 4.0 | ≤ 3.0 |  |
|  | 256 QAM | ≤ 10.5 | ≤ 13.5 | ≤ 7.5 | ≤ 8.0 | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |  |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated or when not all transmitted sub-bands for wideband operation are transmitted. NOTE 3: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, and 5480 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, and 5490 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5300, 5320, 5500, and 5520 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 4: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5180 and 5320 MHz, and 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5510 MHz, and 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5220, 5280, 5540, and 5680 MHz. NOTE 5: Applicable for all valid channels other than those enumerated under NOTE 3 and NOTE 4. NOTE 6: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |  |  |  |  |  |

##### 6.1.3.3.2	A-MPR for S-SSB transmission

6.1.3.3.2.1	LG Electronics’ simulation results (R4-2401464)

Table 6.1.3.3.2.1-1 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 6.1.3.3.2.1-1: NS_30-S-SSB A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 | #3 | #4 | #5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’ (5160) | 27.99 | 26.67 | 24.24 | 21.76 | 21.25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’ (5180) | 8.89 | 7.27 | 5.66 | 6.45 | 5.63 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’ (5200) | 8.78 | 7.16 | 5.63 | 6.46 | 5.72 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’ (5320) | 8.94 | 7.33 | 5.53 | 6.27 | 5.75 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’ (5340) | 32.80 | 32.11 | 24.25 | 26.33 | 24.02 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’ (5480) | 18.31 | 20.19 | 24.26 | 9.90 | 9.25 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’ (5700) | 8.74 | 7.25 | 6.72 | 6.58 | 5.59 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #6 | #7 | #8 | #9 | #10 | #11 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5170) | 29.81 | 29.49 | 24.42 | 28.15 | 26.71 | 24.22 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5190) | 20.42 | 11.45 | 8.69 | 8.92 | 7.24 | 5.41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5230) | 9.24 | 8.21 | 6.04 | 8.77 | 7.31 | 5.60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5310) | 14.82 | 11.44 | 8.89 | 8.69 | 7.25 | 5.63 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5330) | 32.39 | 31.37 | 25.18 | 8.77 | 7.34 | 5.82 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5490) | 28.78 | 28.52 | 23.93 | 17.48 | 19.67 | 8.83 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5510) | 9.16 | 8.46 | 8.59 | 8.70 | 7.41 | 5.42 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’ (5670) | 9.20 | 8.22 | 5.98 | 8.87 | 7.32 | 5.56 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #12 | #13 | #14 | #15 | #16 | #17 | #18 | #19 | #20 | #21 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5180) | 27.98 | 24.92 | 25.31 | 21.72 | 21.26 | 21.14 | 6.37 | 4.96 | 22.67 | 21.01 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5200) | 12.28 | 9.44 | 10.61 | 8.26 | 6.71 | 5.64 | 6.38 | 5.15 | 23.63 | 21.62 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5220) | 9.02 | 7.08 | 7.16 | 5.88 | 6.44 | 5.64 | 6.36 | 5.06 | 10.43 | 7.80 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5280) | 7.94 | 6.97 | 7.10 | 6.37 | 6.37 | 5.64 | 6.27 | 5.11 | 9.87 | 7.97 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5500) | 27.47 | 25.22 | 24.72 | 20.96 | 12.48 | 21.35 | 6.56 | 4.94 | 23.84 | 21.79 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5520) | 10.80 | 9.54 | 7.16 | 8.35 | 6.36 | 5.65 | 6.40 | 5.18 | 23.83 | 9.17 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5540) | 7.90 | 7.07 | 7.09 | 6.20 | 6.57 | 5.60 | 6.38 | 4.86 | 8.23 | 8.04 |  |  |  |  |  |  |  |  |  |  |
| ‘60MHz’ (5680) | 8.22 | 7.65 | 7.18 | 6.21 | 6.56 | 5.85 | 6.47 | 4.95 | 8.34 | 7.05 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #22 | #23 | #24 | #25 | #26 | #27 | #28 | #29 | #30 | #31 | #32 | #33 | #34 | #35 | #36 | #37 | #38 | #39 |  |  |
| '80MHz' (5190) | 28.34 | 25.27 | 27.83 | 22.70 | 24.64 | 21.38 | 21.32 | 20.77 | 9.37 | 8.07 | 6.42 | 5.24 | 25.12 | 21.36 | 23.17 | 21.78 | 23.19 | 21.42 |  |  |
| '80MHz' (5210) | 22.59 | 10.27 | 11.81 | 8.91 | 10.80 | 8.06 | 6.39 | 5.50 | 7.53 | 5.90 | 6.21 | 4.95 | 23.46 | 9.53 | 10.81 | 21.13 | 10.80 | 8.99 |  |  |
| '80MHz' (5290) | 22.15 | 10.27 | 7.76 | 6.27 | 7.12 | 6.40 | 6.49 | 5.91 | 7.42 | 5.94 | 6.26 | 4.96 | 11.75 | 8.90 | 9.43 | 7.79 | 10.69 | 9.19 |  |  |
| '80MHz' (5310) | 28.76 | 23.76 | 11.59 | 9.12 | 7.11 | 6.28 | 6.28 | 5.62 | 10.82 | 8.28 | 6.44 | 5.07 | 26.26 | 21.37 | 23.82 | 9.06 | 26.44 | 21.21 |  |  |
| '80MHz' (5510) | 26.13 | 25.05 | 25.56 | 22.97 | 23.55 | 21.27 | 11.81 | 20.54 | 7.55 | 8.02 | 6.30 | 4.95 | 23.95 | 9.98 | 10.83 | 21.44 | 10.49 | 21.20 |  |  |
| '80MHz' (5530) | 11.69 | 10.36 | 10.77 | 8.63 | 7.21 | 8.07 | 6.65 | 5.65 | 7.44 | 5.88 | 6.20 | 4.94 | 23.81 | 9.43 | 10.98 | 20.95 | 10.79 | 9.18 |  |  |
| '80MHz' (5610) | 7.63 | 6.78 | 7.25 | 6.33 | 7.33 | 6.07 | 6.39 | 5.64 | 7.70 | 5.92 | 6.47 | 5.03 | 8.55 | 7.45 | 8.12 | 6.92 | 7.07 | 7.02 |  |  |

Table 6.1.3.3.2.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 6.1.3.3.2.1-2: NS_30-S-SSB A-MPR simulation results for SL-U power class 5

|  | RB Allocation /Centre frequency of CBW (MHz)/ (dB) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 20MHz : 5160/5340/5480 40MHz : 5170/5190/5310/5330/5490 60MHz : 5180/5200/5500/5520 80MHz : 5190/5210/5290/5310/5510/5530 |  |  |  | 20MHz : 5180/5200/5320/5700 40MHz : 5230/5510/5670 60MHz : 5220/5280/5540/5680 80MHz : 5610 |  |  |  |
|  | Outer RB set configuration |  | Inner RB set configuration |  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/ Non-contiguous sub-band RB sets | 32.80 | 25.27 | 10.82 | 8.28 | 10.43 | 8.59 | 7.70 | 5.92 |

Considering implementation margin, Table 6.1.3.3.2.1-3shows proposal for SL-U NS_30 S-SSB A-MPR.

Table 6.1.3.3.2.1-3:  NS_30 S-SSB A-MPR for SL-U UE power class 5

|  | RB Allocation2 |  |  |  | RB Allocation3 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer RB set configuration |  | Inner RB set configuration |  | Outer RB set configuration |  | Inner RB set configuration |  |
| # of S-SSB repetition/RBset | > 2 | 2 | > 2 | 2 | > 2 | 2 | > 2 | 2 |
| Contiguous/ Non-contiguous sub-band RB sets | ≤ 35.5 | ≤ 28.0 | ≤ 13.5 | ≤ 11.0 | ≤ 13.0 | ≤ 11.0 | ≤ 10.5 | ≤ 8.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, and 5480 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, and 5490 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5300, 5320, 5500, and 5520 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 3: Applicable for all valid channels other than those enumerated under NOTE 2. NOTE 4: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |  |  |  |  |

##### 6.1.3.3.3	A-MPR for PSFCH transmission

6.1.3.3.3.1	LG Electronics’ simulation results (R4-2401464)

Table 6.1.3.3.3.1-1 shows the A-MPR simulation results for the agreed scenarios with different center frequencies.

Table 6.1.3.3.3.1-1: NS_30-PSFCH A-MPR simulation results for SL-U power class 5

| Scenario # | #1 | #2 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ‘20MHz’(5160) | 26.72 | 25.66 |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’(5180) | 7.38 | 9.33 |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’(5200) | 6.62 | 7.50 |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’(5320) | 7.43 | 8.82 |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’(5340) | 32.89 | 30.83 |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’(5480) | 21.05 | 16.83 |  |  |  |  |  |  |  |  |  |  |
| ‘20MHz’(5700) | 6.62 | 7.50 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #3 | #4 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5170) | 29.78 | 26.69 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5190) | 12.58 | 7.85 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5230) | 7.85 | 6.53 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5310) | 12.67 | 6.53 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5330) | 33.21 | 8.86 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5490) | 28.26 | 21.27 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5510) | 12.27 | 6.53 |  |  |  |  |  |  |  |  |  |  |
| ‘40MHz’(5670) | 7.85 | 6.53 |  |  |  |  |  |  |  |  |  |  |
| Scenario # | #5 | #6 | #7 | #8 | #9 |  |  |  |  |  |  |  |
| ‘60MHz’(5180) | 31.10 | 28.95 | 25.70 | 7.69 | 26.58 |  |  |  |  |  |  |  |
| ‘60MHz’(5200) | 23.10 | 18.41 | 9.63 | 7.00 | 26.58 |  |  |  |  |  |  |  |
| ‘60MHz’(5220) | 12.51 | 8.65 | 7.33 | 7.00 | 11.24 |  |  |  |  |  |  |  |
| ‘60MHz’(5280) | 12.91 | 8.65 | 7.30 | 7.00 | 11.35 |  |  |  |  |  |  |  |
| ‘60MHz’(5500) | 30.21 | 26.85 | 16.92 | 7.00 | 26.58 |  |  |  |  |  |  |  |
| ‘60MHz’(5520) | 12.98 | 8.65 | 8.45 | 7.00 | 26.58 |  |  |  |  |  |  |  |
| ‘60MHz’(5540) | 9.69 | 8.65 | 7.30 | 7.00 | 9.40 |  |  |  |  |  |  |  |
| ‘60MHz’(5680) | 9.87 | 8.65 | 7.30 | 7.00 | 9.40 |  |  |  |  |  |  |  |
| Scenario # | #10 | #11 | #12 | #13 | #14 | #15 | #16 | #17 | #18 |  |  |  |
| '80MHz'(5190) | 31.31 | 30.67 | 28.58 | 24.90 | 13.18 | 7.92 | 28.39 | 26.47 | 26.30 |  |  |  |
| '80MHz'(5210) | 27.86 | 13.81 | 12.75 | 9.29 | 9.96 | 7.02 | 27.41 | 26.47 | 26.30 |  |  |  |
| '80MHz'(5290) | 27.41 | 12.25 | 8.43 | 7.43 | 9.96 | 7.02 | 24.90 | 11.30 | 26.55 |  |  |  |
| '80MHz'(5310) | 31.77 | 24.59 | 8.90 | 8.27 | 13.20 | 7.02 | 29.01 | 26.41 | 28.29 |  |  |  |
| '80MHz'(5510) | 30.04 | 29.18 | 26.57 | 17.16 | 12.65 | 7.02 | 27.41 | 26.47 | 26.30 |  |  |  |
| '80MHz'(5530) | 26.05 | 13.81 | 10.92 | 7.94 | 9.96 | 7.02 | 27.22 | 26.47 | 26.30 |  |  |  |
| '80MHz'(5610) | 10.43 | 10.01 | 8.43 | 7.43 | 9.96 | 7.02 | 10.17 | 9.26 | 8.73 |  |  |  |

Table 6.1.3.3.3.1-2 shows the maximum value of simulation results considering combinations of Outer/Inner sub-band configuration and Full/Partial RB allocation.

Table 6.1.3.3.3.1-2: NS_30-PSFCH A-MPR simulation results for SL-U power class 5

| RB set configuration | RB Allocation /Centre frequency of CBW (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | 20MHz : 5160/5340/5480 40MHz :5170/5190/5310/5330/ 5490/5510 60MHz :5180/5200/5500/5520 80MHz : 5190/5210/5290/5310/ 5510/5530 |  | 20MHz : 5180/5200/5320/5700 40MHz : 5230/5510/5670 60MHz : 5220/5280/5540/5680 80MHz : 5610 |  |
|  | Outer RB set configuration (Full/Partial) | Inner RB set configuration (Full/Partial) | Outer RB set configuration (Full/Partial) | Inner RB set configuration (Full/Partial) |
| Contiguous/Non-contiguous sub-band RB sets | 33.21 | 13.20 | 12.91 | 9.96 |

Considering implementation margin, Table 6.1.3.3.3.1-3 shows proposal for SL-U NS_30 PSFCH A-MPR.

Table 6.1.3.3.3.1-3 : NS_30 PSFCH A-MPR for SL-U UE power class 5

| RB set configuration | RB Allocation /Centre frequency of CBW (MHz) |  |  |  |
| --- | --- | --- | --- | --- |
|  | RB Allocation2 |  | RB Allocation3 |  |
|  | Outer RB set configuration | Inner RB set configuration | Outer RB set configuration | Inner RB set configuration |
| Contiguous/Non-contiguous sub-band RB sets | ≤ 36.0 | ≤ 16.0 | ≤ 15.5 | ≤ 12.5 |
| NOTE 1: The A-MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously or non-contiguously allocated in the channel. NOTE 2: Applicable for 20 MHz channels centered at the nearest NR-ARFCN corresponding to 5160, 5340, and 5480 MHz, 40 MHz channels centered at the nearest NR-ARFCN corresponding to 5170, 5190, 5310, 5330, 5490, and 5510 MHz, 60 MHz channels centered at the nearest NR-ARFCN corresponding to 5180, 5200, 5300, 5320, 5500, and 5520 MHz, and 80 MHz channels centered at the nearest NR-ARFCN corresponding to 5190, 5210, 5290, 5310, 5510, and 5530 MHz. NOTE 3: Applicable for all valid channels other than those enumerated under NOTE 2. NOTE 4: In current release larger CBW than 80MHz are not applicable for this network signalling. |  |  |  |  |

<<<<<<<<<<< End of changes in section 6 >>>>>>>>>>