3GPP TSG-RAN WG4 Meeting #110	R4-2401484

Athens, Greece, 26th February – 1st March 2024

Source:	Ericsson, T-Mobile US

Title:	TP for 38718-02-01 adding UL CA_n41C-n71A to DL CA_n41C-n71(2A) and CA_n41C-n71B

Agenda item:			7.10.2

Document for:	Approval

# 1. Introduction

This contribution is a text proposal for 38718-02-01 adding UL CA_n41C-n71A to DL CA_n41C-n71(2A) and CA_n41C-n71B

# 2. Text Proposal

# ---Start of changes---

## 5.72		CA_n41-n71

### 5.72.1	Common for 1 band UL and 2 bands UL CA

#### 5.72.1.1 Operating bands for CA

Table 5.72.1.1-1: CA band combination of band n41+n71

| NR Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
| n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

#### 5.72.1.2	Channel bandwidths per operating band for CA

Table 5.72.1.2-1: Supported bandwidths per CA band combination of band n41+n71

| CA operating/channel bandwidth [MHz] |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n41C-n71A | CA_n41C-n71A | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | See n71 channel bandwidths in Table 5.3.5-1 |  |
| CA_n41C-n71(2A) | CA_n41C-n71A | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71(2A)_BCS 4 and 5 |  |
| CA_n41C-n71B | CA_n41C-n71A | n41 | CA_n41C_BCS 4 and 5 | 4 and 5 |
|  |  | n71 | CA_n71B_BCS 4 and 5 |  |

#### 5.72.1.3	Co-existence studies

The co-existence study has been completed for this CA except the triple-beat analysis, that this addition brings. The analysis of the IMD is found in Table 5.72.1.3-1.

Table 5.72.1.3-1: Triple-beat IMD analysis of band n41+n71

| CC location | fU1L | fU2L | fU3L | fU1H |  | Channel BW |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency | 2496 | 2516 | 2666 | 2690 |  | 10 |
| CC location | fSCCL | fSCCH | fU2H | fU3H |  | Min channel separation |
| Frequency | 663 | 698 | 2670 | 2520 |  | 10 |
| 1st order triple beat | I fUL3 -fUL1 - fSCCL ¦ | I fUL2 -fUL1 + fSCCL ¦ | I fUL2 -fUL1 - fSCCH ¦ | I fUL3 -fUL1 + fSCCH ¦ |  | Max channel separation |
| Ranges | 493 | 683 | 678 | 868 |  | 170 |
| 1st order triple beat | I fUL2 +fUL1 - fSCCH ¦ | I fUH1 +fUH2 - fSCCL ¦ | I fUL2 +fUL1 + fSCCL ¦ | I fUH1 +fUH2 + fSCCH ¦ |  |  |
| Ranges | 4314 | 4697 | 5675 | 6058 |  |  |

There is an IMD issue for this combination.

#### 5.72.1.4	∆TIB and ∆RIB values

These values are already part of the specification. There’s nothing new for the proposed bandwidth combination set.

#### 5.72.1.5	REFSENs requirements

Even a triple-beat IMD product of CA_n41C-n71A is identified then according to the agreed WF R4-2220556 the bands belong to different band groups hence no MSD is needed defined.

#### 5.72.1.6	OOB blocking exception requirements

There is no OOB exception for the CA combination.

---End of changes---

# Reference

[1] 	RP-232920, Rel-18 NR Inter-band Carrier Aggregation/Dual Connectivity  for 2 bands DL with x bands UL (x=1,2), RAN#102