3GPP TSG-RAN4 Meeting #110	R4-2401508

Athens, Greece, 26 February-1 March, 2024

Agenda item:			8.19.1.2

Source:	vivo

Title:	Discussion on the revision of MPR reduction and A-MPR part

Document for:	Discussion

- Introduction

In RAN4 #109 meeting, the CR to 38.101 for MPR reduction has been agreed in [1]. However, there are some details and wording that need further discussion. So, we deliver a draft CR [2] based on [1] in this meeting and state the relative reasons and analysis about corresponding specification change in this contribution.

- Discussion on spec change for MPR reduction

From CR[1] it has been agreed that, when the UE indicates support for UE capability [powerBoostRel18] or [powerBoostTSRel18], and IE [powerBoostPi2BPSKRel18] or [powerBoostQPSKRel18] is also configured, power boosting would be available and ΔPPowerBoost would be added after Ppowerclass, so the lower bound of the maximun output power PCMAX,f,c is raised up. However, when A-MPR needs to be applied, the influence of ΔPPowerBoost should be considered prudently and PCMAX_L,f,c needs to be reduced to the original level before powerboosting to prevent A-MPR from losing its effect.

In the current spec, unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Therefore, the purpose of adding the parameter ΔPPowerBoost on the higher one of ‘MPR+∆MPR’ and ‘A-MPR’ is to bring PCMAX_L,f,c in the above situation back down to its original level and to make the A-MPR value effective enough when it is defined and needed. In clause 6.2.3, the A-MPR part has been updated as below and stated in CR[1].

Figure 1. Changes to A-MPR in CR[1] when using ΔPPowerBoost

## 2.1 The problem of the current spec

The related problem introduced by the new definition of A-MPR is as follows:

When UE indicates support for UE capability [powerBoostRel18] and the RB allocation belongs to ‘the inner region but is outside the enhanced power inner region’ as defined in clause 6.2.2 in CR[1], it can be seen that the applicable MPR0 will be increased by ΔPPowerBoost.

Therefore, based on this premise, when A-MPR is smaller than (0+∆MPR), the additional increase of ΔPPowerBoost shouldn’t apply again, or else there will be double ΔPPowerBoost in ‘MPR+∆MPRc’ part. So that it will also be even lower than the original PCMAX_L,f,c by ΔPPowerBoost.

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(0+ ΔPPowerBoost +∆MPRc + ΔPPowerBoost, A-MPR)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

= MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass) – MAX(MAX(0+∆MPRc + ΔPPowerBoost, A-MPR)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

Observation1：In the premise that UE indicates support for UE capability [powerBoostRel18] and the RB allocation belongs to ‘the inner region but is outside the enhanced power inner region’, when A-MPR is smaller than (MPR+∆MPRc), there will be double ΔPPowerBoost in ‘MPR+∆MPRc’ part.

## 2.2 New modification scheme

Based on the above analysis, the new modification scheme is as follows:

Proposal1: When A-MPR takes effect, PCMAX_L,f,c should be pulled back to the original level before powerboost.

And the different cases will be introduced as follows:

- When A-MPR is smaller than MPR+∆MPRc:

It seems that there is no additional power reduction request in this situation, but actually it should be divided into two different cases: (A-MPR+ ΔPPowerBoost ≤MPR+∆MPRc) and (MPR+∆MPRc <A-MPR+ ΔPPowerBoost). Because the conditions for A-MPR to work are not limited to it being higher than MPR+∆MPRc. And to make sure PCMAX_L,f,c can be pulled back to the original level before powerboost when A-MPR should take effect, A-MPR should be combined with ΔPPowerBoost here.

Observation2: When A-MPR is smaller than MPR+∆MPRc, it should be divided into two different cases: (A-MPR+ ΔPPowerBoost ≤MPR+∆MPRc) and (MPR+∆MPRc <A-MPR+ ΔPPowerBoost).

When A-MPR < MPR+∆MPRc, MAX(MPR+∆MPRc, A-MPR) = MPR+∆MPRc, if excluding other factors and only considering PPowerClass, ΔPPowerBoost, MPR+∆MPRc and A-MPR, the PCMAX_L,f,c formula will be simplified to the following format:

PCMAX_L,f,c = (PPowerClass + ΔPPowerBoost) – MAX(MPR+∆MPRc, A-MPR)

- If (A-MPR+ ΔPPowerBoost > MPR+∆MPRc), which is equal to the difference of A-MPR and (MPR+∆MPRc) is smaller than ΔPPowerBoost, so after powerboost the output power will be higher than the orginal output power limited by A-MPR.

For example: MPR+∆MPRc=3, A-MPR=2.8, ΔPPowerBoost=1(PC3), so before powerboost when A-MPR is effectual the PCMAX_L,f,c should be 23-2.8=20.2. After powerboost the PCMAX_L,f,c will be 23+1-MAX(3,2.8)=21. And it is significantly higher than the original output power limited by A-MPR, in which case A-MPR should start to take effect, so in this situation the PCMAX_L,f,c should be pulled back to the original level before powerboost. So there should be ΔPPowerBoost added after MAX(MPR+∆MPRc, A-MPR) and the PCMAX_L,f,c formula will change back to the format without ΔPPowerBoost:

PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(MPRc+∆MPRc, A-MPRc) + ΔPPowerBoost + ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

= MIN {PEMAX,c– ∆TC,c,  (PPowerClass – ΔPPowerClass) – MAX(MAX(MPRc+∆MPRc, A-MPRc) + ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }

Proposal2: If (A-MPR+ΔPPowerBoost>MPR+∆MPRc), there should be ΔPPowerBoost added after MAX(MPR+∆MPRc, A-MPR), otherwise after powerboost the output power will be higher than the orginal output power limited by A-MPR.

- If (A-MPR+ ΔPPowerBoost ≤ MPR+∆MPRc), which is equal to the difference of A-MPR and (MPR+∆MPRc) is higher than ΔPPowerBoost, so after powerboost the output power will still be lower than the orginal output power limited by A-MPR.

For example: MPR+∆MPRc=3, A-MPR=1.8, ΔPPowerBoost=1(PC3), so before powerboost when A-MPR is effectual the PCMAX_L,f,c should be 23-1.8=21.2. After powerboost the PCMAX_L,f,c will be 23+1-MAX(3,1.8)=21, it is still lower than the original output power limited by A-MPR, in which case A-MPR not need to take effect. And PCMAX_L,f,c can be increased by ΔPPowerBoost compared with the orginal format before powerboost.

- When no A-MPR is defined:

It can be merged into the case of : A-MPR< MPR+∆MPRc. And it means that A-MPR is not required here actually, so when A-MPR+ ΔPPowerBoost ≤ MPR+∆MPRc, the PCMAX_L,f,c could be increased by ΔPPowerBoost, except for the inner outer region with UE capability [powerBoostRel18].

Proposal3: When no A-MPR is defined, it can be merged into the case of : A-MPR< MPR+∆MPRc.

- When A-MPR is higher than MPR+∆MPRc:

Which means A-MPR needs to take effect, so there should also be ΔPPowerBoost added after MAX(MPR+∆MPRc, A-MPR) to make sure the PCMAX_L,f,c can be pulled back to the original level before powerboost.

Proposal4: When A-MPR is higher than MPR+∆MPRc, there should also be ΔPPowerBoost added after MAX(MPR+∆MPRc, A-MPR) to make sure PCMAX_L,f,c can be pulled back to the original level before powerboost.

4、Summary:

According to the modification, there is a table summarizing the different cases under different combinations of (A-MPR, MPR+∆MPRc) and RB region:

Table1. PCMAX_L,f,c formula under different combinations of (A-MPR, MPR+∆MPRc) and RB region

|  |  | UE capability [powerBoostRel18] |  | UE capability [powerBoostTSRel18] |
| --- | --- | --- | --- | --- |
|  |  | enhanced power inner allocation region：MPR=0 | inner outer region (belongs to the inner region but is outside the enhanced power inner region): MPR=0+ ΔPPowerBoost (In this region, PCMAX_L,f,c shouldn’t be boosted) | the whole inner region：MPR=0 |
| A-MPR < MPR+∆MPRc, or no A-MPR is defined | A-MPR+ ΔPPowerBoost ≤MPR+∆MPRc (A-MPR will not take effect, PCMAX_L,f,c can be boosted) | PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(0+∆MPRc, A-MPR)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) } [PCMAX_L,f,c boosted] | PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX( 0 + ΔPPowerBoost +∆MPRc, A-MPR)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }， [PCMAX_L,f,c not boosted in the limited region] | PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(0+∆MPRc, A-MPR)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }， [PCMAX_L,f,c boosted] |
|  | MPR+∆MPRc <A-MPR+ ΔPPowerBoost (A-MPR will take effect, PCMAX_L,f,c shouldn’t be boosted) | PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(0+∆MPRc, A-MPR+ ΔPPowerBoost)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }， [PCMAX_L,f,c not boosted] | PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX( 0 + ΔPPowerBoost +∆MPRc, A-MPR+ ΔPPowerBoost)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }， [PCMAX_L,f,c not boosted] | PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(0+∆MPRc, A-MPR+ ΔPPowerBoost)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }， [PCMAX_L,f,c not boosted] |
| A-MPR > MPR+∆MPRc (A-MPR will take effect, PCMAX_L,f,c shouldn’t be boosted) |  | PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(0+∆MPRc, A-MPR+ ΔPPowerBoost)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }， [PCMAX_L,f,c not boosted] | PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX( 0 + ΔPPowerBoost +∆MPRc, A-MPR+ ΔPPowerBoost)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }， [PCMAX_L,f,c not boosted] | PCMAX_L,f,c = MIN {PEMAX,c– ∆TC,c, (PPowerClass – ΔPPowerClass + ΔPPowerBoost) – MAX(MAX(0+∆MPRc, A-MPR+ ΔPPowerBoost)+ ΔTIB,c + ∆TC,c + ∆TRxSRS, P-MPRc) }， [PCMAX_L,f,c not boosted] |

- Conclusion

Observation1：In the premise that UE indicates support for UE capability [powerBoostRel18] and the RB allocation belongs to ‘the inner region but is outside the enhanced power inner region’, when A-MPR is smaller than (MPR+∆MPRc), there will be double ΔPPowerBoost in ‘MPR+∆MPRc’ part.

Proposal1: When A-MPR takes effect, PCMAX_L,f,c should be pulled back to the original level before powerboost.

Observation2: When A-MPR is smaller than MPR+∆MPRc, it should be divided into two different cases: (A-MPR+ ΔPPowerBoost ≤MPR+∆MPRc) and (MPR+∆MPRc <A-MPR+ ΔPPowerBoost).

Proposal2: If (A-MPR+ΔPPowerBoost>MPR+∆MPRc), there should be ΔPPowerBoost added after MAX(MPR+∆MPRc, A-MPR), otherwise after powerboost the output power will be higher than the orginal output power limited by A-MPR.

Proposal3: When no A-MPR is defined, it can be merged into the case of : A-MPR< MPR+∆MPRc.

Proposal4: When A-MPR is higher than MPR+∆MPRc, there should also be ΔPPowerBoost added after MAX(MPR+∆MPRc, A-MPR) to make sure the PCMAX_L,f,c can be pulled back to the original level before powerboost.

- References

- R4-2321826 CR to 38.101 for MPR reduction, RAN4#109.

- R4-2401507 draft CR to 38.101 for revision and supplement of MPR reduction and A-MPR part, RAN4#110.