3GPP TSG-RAN WG4 Meeting #110												R4-2401516

Athens, GR, 26 Feb – 01 Mar, 2024

Source: 	vivo

Title: 	Discussion on maximum aggregated bandwidth for FR1 inter-band CA

Agenda Item:	12.1.2

Document for:	Approval

- Introduction

In [1], the LS from RAN2 is received and the following question is raised:

Question)	Can the new maximum aggregated bandwidth UE capabilities be applied genericly to intra-band FR1 CA as well as inter-band FR1 CA?

It has been a long time since this topic was discussed, and in this contribution, we review the past discussions and try to give an answer to the question above.

- Discussion

In [2], the mismatch between theoretical possible maximum aggregated bandwidth and the maximum aggregated bandwidth in spec was raised, e.g., CA_n7B, and an easy solution is to use multiple feature sets to indicate the real maximum aggregated bandwidth of UE. In [3], the signaling of maximum aggregated bandwidth for intra-band CA was proposed to reduce the signaling overhead.

Observation 1: The signaling of maximum aggregated bandwidth is originally intended to be introduced for intra-band CA.

In [4], the signaling was proposed to be extended to inter-band CA since it can also reduce the overhead. However, the main point of controversy is whether RAN2 really needs such capabilities to reduce signaling overhead since multiple feature sets can also solve the problem, and this is the reason why LS from RAN4 [5] only request RAN2 to check the signaling overhead and let RAN2 to decide whether the capabilities are needed.

Observation 2: RAN2 now decide to introduce new capability for maximum aggregated bandwidth and there is no need to argue about the necessity for this capability.

Considering the capabilities are introduced and NW can also benefit it from intra-band CA case, we think it is reasonable to extend the capabilities to FR1 intra-band CA.

Proposal: The new maximum aggregated bandwidth UE capabilities can also be applied to FR1 intra-band CA.

A draft reply LS is prepared in [6].

- Conclusion

Observation 1: The signaling of maximum aggregated bandwidth is originally intended to be introduced for intra-band CA.

Observation 2: RAN2 now decide to introduce new capabilities for maximum aggregated bandwidth and there is no need to argue about the necessity for this capability.

Proposal: The new maximum aggregated bandwidth UE capabilities can also be applied to FR1 intra-band CA.

- References

- R2-2313745 LS on applicability of maximum aggregated bandwidth UE capabilities to intra-band FR1 CA, Qualcomm, RAN2#124

- R4-2200462 Handling of maximum aggregated channel bandwidth for BCS4/5, Nokia, RAN4#101bis-e

- R4-2204509 Discussion on maximum aggregated channel bandwidth for BCS4/5, Qualcomm Incorporated, RAN4#102-e

- R4-2213176 Maximum aggregated channel bandwidth for FR1 CA, Qualcomm Incorporated, RAN4#104-e

- R4-2303685 LS on UE signalling for the maximum aggregated bandwidth for FR1 CA, Qualcomm Incorporated, RAN4#106

- R4-2401517, draft reply LS on maximum aggregated bandwidth for FR1 inter-band CA, vivo, RAN4#110