3GPP TSG-RAN WG4 Meeting #110                                                       R4-2401554

Athens, Greece, 26th Feb. – 1st Mar. 2024

Agenda Item:	8.1.3.1.2

Source:	Ericsson

Title:	Simulation results for 8Rx PDSCH CA requirement

Document for:	Information

# 1	Introduction

In this contribution, we provided our simulation results for 8Rx PDSCH CA requirement. For rank 2 cases, the MCS, channel and antenna correlation have different options, as specified by the following agreements listed in the WF [1]:

Issue 3-1: Antenna correlation for carrier with Rank 2 in 8Rx CA test - Proposal - Option 1: Revisit Rank 2 to TDLA30-10 Low - Option 2: Keep TDLC300-100 ULA Medium B (a = 0.3, b = 0.005154) that is same as Rank 2 single carrier test - Note: Interested companies can provide simulation results with both channel conditions TDLA30-10 Low and TDLC300-100 ULA Medium B (a = 0.3, b = 0.005154) for the next RAN4#110 meeting

For rank 2 cases, therefore, we provided simulation results for both options.

# 2	Discussion

Table 2-1 Simulation results for 8Rx PDSCH CA with rank 2, ULA medium B, FDD, 15kHz SCS

| Bandwidth (MHz) | MCS | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | 19 | 2 | TDLC300-100 | 2x8, medium B | 70 | 9.6 |
| 10 | 19 | 2 | TDLC300-100 | 2x8, medium B | 70 | 9.5 |
| 15 | 19 | 2 | TDLC300-100 | 2x8, medium B | 70 | 9.6 |
| 20 | 19 | 2 | TDLC300-100 | 2x8, medium B | 70 | 9.8 |
| 25 | 19 | 2 | TDLC300-100 | 2x8, medium B | 70 | 9.7 |
| 30 | 19 | 2 | TDLC300-100 | 2x8, medium B | 70 | 9.9 |
| 35 | 19 | 2 | TDLC300-100 | 2x8, medium B | 70 | 9.8 |
| 40 | 19 | 2 | TDLC300-100 | 2x8, medium B | 70 | 9.8 |
| 45 | 19 | 2 | TDLC300-100 | 2x8, medium B | 70 | 10 |
| 50 | 19 | 2 | TDLC300-100 | 2x8, medium B | 70 | 10.1 |

Table 2-2 Simulation results for 8Rx PDSCH CA with rank 2, ULA low, FDD, 15kHz SCS

| Bandwidth (MHz) | MCS | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | 19 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 5.7 |
| 10 | 19 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 5.7 |
| 15 | 19 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 5.9 |
| 20 | 19 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 6 |
| 25 | 19 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 5.9 |
| 30 | 19 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 6.4 |
| 35 | 19 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 6.1 |
| 40 | 19 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 6.1 |
| 45 | 19 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 6.2 |
| 50 | 19 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 6.4 |

Table 2-3 Simulation results for 8Rx PDSCH CA with rank 2, ULA medium B, FDD, 15kHz SCS

| Bandwidth (MHz) | MCS | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | 17 | 2 | TDLC300-100 | 2x8, medium B | 70 | 7.8 |
| 10 | 17 | 2 | TDLC300-100 | 2x8, medium B | 70 | 7.8 |
| 15 | 17 | 2 | TDLC300-100 | 2x8, medium B | 70 | 7.7 |
| 20 | 17 | 2 | TDLC300-100 | 2x8, medium B | 70 | 8.0 |
| 25 | 17 | 2 | TDLC300-100 | 2x8, medium B | 70 | 8.0 |
| 30 | 17 | 2 | TDLC300-100 | 2x8, medium B | 70 | 8.1 |
| 35 | 17 | 2 | TDLC300-100 | 2x8, medium B | 70 | 8.2 |
| 40 | 17 | 2 | TDLC300-100 | 2x8, medium B | 70 | 8.0 |
| 45 | 17 | 2 | TDLC300-100 | 2x8, medium B | 70 | 8.1 |
| 50 | 17 | 2 | TDLC300-100 | 2x8, medium B | 70 | 8.3 |

Table 2-4 Simulation results for 8Rx PDSCH CA with rank 2, ULA low, FDD, 15kHz SCS

| Bandwidth (MHz) | MCS | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | 17 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 4.3 |
| 10 | 17 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 4.3 |
| 15 | 17 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 4.4 |
| 20 | 17 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 4.5 |
| 25 | 17 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 4.4 |
| 30 | 17 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 4.8 |
| 35 | 17 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 4.7 |
| 40 | 17 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 4.5 |
| 45 | 17 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 4.6 |
| 50 | 17 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 4.8 |

Table 2-5 Simulation results for 8Rx PDSCH CA with rank 8, FDD, 15kHz SCS

| Bandwidth (MHz) | MCS | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.1 |
| 10 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.1 |
| 15 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.5 |
| 20 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.7 |
| 25 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.8 |
| 30 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 20.3 |
| 35 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 20.3 |
| 40 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 20.8 |
| 45 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 20.8 |
| 50 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 20.6 |

Table 2-6 Simulation results for 8Rx PDSCH CA with rank 2, ULA medium B, TDD, 30kHz SCS

| Bandwidth (MHz) / SCS | MCS | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | 19 | 2 | TDLC300-100 | 2x8, medium B | 70 | 9 |
| 10 | 19 | 2 | TDLC300-100 | 2x8, medium B | 70 | 9,3 |
| 15 | 19 | 2 | TDLC300-100 | 2x8, medium B | 70 | 9,3 |
| 20 | 19 | 2 | TDLC300-100 | 2x8, medium B | 70 | 9,3 |
| 25 | 19 | 2 | TDLC300-100 | 2x8, medium B | 70 | 9,4 |
| 30 | 19 | 2 | TDLC300-100 | 2x8, medium B | 70 | 9,4 |
| 40 | 19 | 2 | TDLC300-100 | 2x8, medium B | 70 | 9,6 |
| 50 | 19 | 2 | TDLC300-100 | 2x8, medium B | 70 | 9,5 |
| 60 | 19 | 2 | TDLC300-100 | 2x8, medium B | 70 | 9,5 |
| 80 | 19 | 2 | TDLC300-100 | 2x8, medium B | 70 | 10 |
| 90 | 19 | 2 | TDLC300-100 | 2x8, medium B | 70 | 9,7 |
| 100 | 19 | 2 | TDLC300-100 | 2x8, medium B | 70 | 9,8 |

Table 2-7 Simulation results for 8Rx PDSCH CA with rank 2, ULA low, TDD, 30kHz SCS

| Bandwidth (MHz) / SCS | MCS | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | 19 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 5,9 |
| 10 | 19 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 6 |
| 15 | 19 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 6.1 |
| 20 | 19 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 6.1 |
| 25 | 19 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 6,3 |
| 30 | 19 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 6,4 |
| 40 | 19 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 6,4 |
| 50 | 19 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 6,3 |
| 60 | 19 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 6,4 |
| 80 | 19 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 6,7 |
| 90 | 19 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 6,6 |
| 100 | 19 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 6,7 |

Table 2-8 Simulation results for 8Rx PDSCH CA with rank 2, ULA medium B, TDD, 30kHz SCS

| Bandwidth (MHz) / SCS | MCS | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | 17 | 2 | TDLC300-100 | 2x8, medium B | 70 | 7.5 |
| 10 | 17 | 2 | TDLC300-100 | 2x8, medium B | 70 | 7.5 |
| 15 | 17 | 2 | TDLC300-100 | 2x8, medium B | 70 | 7.6 |
| 20 | 17 | 2 | TDLC300-100 | 2x8, medium B | 70 | 7.5 |
| 25 | 17 | 2 | TDLC300-100 | 2x8, medium B | 70 | 7.6 |
| 30 | 17 | 2 | TDLC300-100 | 2x8, medium B | 70 | 7.7 |
| 40 | 17 | 2 | TDLC300-100 | 2x8, medium B | 70 | 7.8 |
| 50 | 17 | 2 | TDLC300-100 | 2x8, medium B | 70 | 7.7 |
| 60 | 17 | 2 | TDLC300-100 | 2x8, medium B | 70 | 8 |
| 80 | 17 | 2 | TDLC300-100 | 2x8, medium B | 70 | 8.2 |
| 90 | 17 | 2 | TDLC300-100 | 2x8, medium B | 70 | 8.1 |
| 100 | 17 | 2 | TDLC300-100 | 2x8, medium B | 70 | 8.1 |

Table 2-9 Simulation results for 8Rx PDSCH CA with rank 2, ULA low, TDD, 30kHz SCS

| Bandwidth (MHz) / SCS | MCS | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | 17 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 4.5 |
| 10 | 17 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 4.5 |
| 15 | 17 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 4.6 |
| 20 | 17 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 4.6 |
| 25 | 17 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 4.6 |
| 30 | 17 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 4.9 |
| 40 | 17 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 4.8 |
| 50 | 17 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 4.7 |
| 60 | 17 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 5 |
| 80 | 17 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 5.2 |
| 90 | 17 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 5.2 |
| 100 | 17 | 2 | TDLA30-10 | 2x8, ULA Low | 70 | 5.2 |

Table 2-10 Simulation results for 8Rx PDSCH CA with rank 8, TDD, 30kHz SCS

| Bandwidth (MHz) | MCS | Rank | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 5 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18,9 |
| 10 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18,7 |
| 15 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19.1 |
| 20 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18,9 |
| 25 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 18,8 |
| 30 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19,5 |
| 40 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19,5 |
| 50 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 19,5 |
| 60 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 20.1 |
| 80 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 20.1 |
| 90 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 20,4 |
| 100 | 17 | 8 | TDLA30-10 | 8x8, ULA Low | 70 | 20,6 |

# 3	Summary

In this contribution, we provided our simulation results for 8Rx PDSCH CA requirements.

# References

[1]	R4-2316921: WF for UE 8Rx performance requirements, Huawei, HiSilicon, RAN4#109, Chicago, US