3GPP TSG-RAN4 Meeting #110	R4-2401560

Athens, Greece, Febuaray 26th  –  March 1st  , 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.133 on RRM core requirements for Idle/Inactive measurement for Scell/SCG setup |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_Mob_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-09 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-18 FeMob enh, RAN4 reached agreement to introduce a network configured validity timer for Idle/Inactive measurement for CA/DC report. According to exception sheet in RP-233969, the additional measurement shall not be within the scope |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove the part of the additional measurement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Corresponding RRM requirement will be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4 and 5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

## 4.x	Measurement report for fast CA/DC setup

### 4.x.1	Introduction

A UE supporting [solution based on existing measurement] shall report the idle mode measurement results on the inter-frequency CA and DC candidate frequencies/cells indicated by higher layers and meet the requirement specified in this clause. The UE shall perform idle mode measurements provided that the serving cell support early measurement and is within the validity area. The idle mode measurement requirements apply to a configured carrier frequency and the serving cell are among the supported band combination of the UE.

### 4.x.2	Measurement Requirements

For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 the UE shall support the idle mode CA measurements on the serving cell, and carriers configured for idle mode CA/DC measurement and meet corresponding measurement requirements defined in clause 4.4.2.

[For a UE which supports [solution based on existing measurement] but not idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 the UE shall support the idle mode CA measurements on the serving cell, and carriers configured for idle mode CA/DC measurement and meet corresponding measurement requirements defined in clause 4.2.2.The UE physical layer shall be capable of reporting SS-RSRP and SS-RSRQ measurements of the carriers configured to higher layers, with measurement accuracy as specified in clauses 10.1.4B and 10.1.5B and 10.1.9B and 10.1.10B, respectively]

### 4.x.3	Measurement Report Requirements

For a UE which supports [solution based on existing measurement] the UE shall be able to report valid measurement results upon RRC setup complete. The measurement results are considered valid if the following conditions are met:

- the measurements are performed within the last [X] seconds before msg1 transmission for RRC resume/setup request, where [X] is configured by [TBD], and

- the measurement results satisfy measurement accuracy requirement at the measurement instance.

Otherwise, the measurement results are considered invalid. The UE shall not report invalid measurement results. If network doesn’t provide configuration of [X], UE is not required to perform validity check but the measurement results satisfy measurement accuracy requirement at the measurement instance.

Editor’s note, the section may be further updated, if needed.

End of Change 1

Start of Change 2

## 5.x	Measurement report for fast CA/DC setup

### 5.x.1	Introduction

A UE supporting [solution based on existing measurement] shall perform the inactive mode measurement on the inter-frequency CA and DC candidate frequencies/cells indicated by higher layers and meet the requirement specified in this clause. The UE shall perform idle mode measurements provided that the serving cell support early measurement and is within the validity area. The idle mode measurement requirements apply to a configured carrier frequency and the serving cell are among the supported band combination of the UE.

### 5.x.2	Measurement Requirements

The requirements in clause 4.x.2 shall apply.

### 5.x.3	Measurement Report Requirements

The requirements in clause 4.x.3 shall apply.

End of Change 2