3GPP TSG-RAN WG4 Meeting #110              	R4-2401578

Athens, Greece, 26th Feb-1st March, 2024

Agenda item:	8.21.4.2

Source:	Samsung

Title:	Discussion and simulation results on BS demodulation requirements for Rel-18 FeMIMO

Document for:	Discussion

- Introduction

In the last RAN4 meeting, the test scope and test setup of BS demodulation performance part for FeMIMO was under discussion. The related agreement was captured in the WF [1] as following

- Scope of BS demodulation performance requirements - clarify the details of applicability rule for Rel-18 DMRS ports - Unless otherwise stated, PUSCH requirements with enhanced DM-RS port configuration shall apply only for a BS declaring support of enhanced DM-RS port type (see D.xxx in table 4.6-1) - [a BS that passes tests with enhanced DM-RS port can consider corresponding legacy PUSCH tests as passed. Definition of “corresponding” needs to be further specified]. FFS on specific wording - clarify if BS demodulation requirements are needed for FR2 STxMP - Do not define FR2 STxMP demodulation requirements in Rel-18, postpone the discussion on BS performance introduction with UE FR2 STxMP to future release. - Test setup and simulation assumptions for Rel-18 DMRS - DMRS ports - Rank 1 for 1Tx: {8} - Rank 2 for 2Tx: {8, 9} - CBW - Option 1 - 15KHz SCS, 5MHz - 30KHz SCS, 10MHz - Option 2: maximum CBW - Other options are not precluded - MCS - Option 1: MCS 16 - Option 2: MCS 21 (64QAM) - Other options are not precluded

In this contribution, the view on remaining issue of test setup is provided. Meanwhile, the initial simulation results for Rel-18 DMRS port enhancement are provided for alignment purpose.

# 2	Discussion

In the last meeting, PUSCH requirement with DMRS port enhancement was agreed to introduce with test applicability

rule

## 2.1	Test applicability rule

As agreed in the previous meeting as following for Rel-18 enhanced DMRS port

- Unless otherwise stated, PUSCH requirements with enhanced DM-RS port configuration shall apply only for a BS declaring support of enhanced DM-RS port type (see D.xxx in table 4.6-1) - [a BS that passes tests with enhanced DM-RS port can consider corresponding legacy PUSCH tests as passed. Definition of “corresponding” needs to be further specification]. FFS on specific wording

In the previous meeting, it was agreed to introduce PUSCH demodulation requirements with selecting limited case(s) to configure the new defined DMRS ports for no more than 2 DMRS ports. Since only limited test cases will be defined, and no change the total number of test cases, only the test cases with same test parameters but configuring the legacy DM-RS port can be skipped if the BS passes tests with enhanced DM-RS port.

Then, the following test applicability rule can be considered as “a BS that passes tests with enhanced DM-RS port can consider the tests with same test parameters but configuring legacy DM-RS port as passed”

Proposal 1:  The following test applicability rule can be considered

- a BS that passes tests with enhanced DM-RS port (enhanced-dmrs-Type_r18) can consider the tests with same test parameters but configuring legacy DM-RS port as passed

## 2.2	Test setup

In the last meeting, the initial test setup with DMRS enhancement was under discussion.

SCS&CBW

Regarding SCS &BW, both the minimum CBW and maximum CBW were considered for evaluation. From baseband processing perspective, there is no difference for channel estimation. Since the DMRS enhancement is mainly targeting for FR1, to reduce the test effort, we would like to specify limited CBW per SCS.  Considering RAN4 has introduced test applicability rule with different CBW, the minimum CBW per SCS, e.g., 15KHz SCS with 5MHz, and 30KHz SCS with 10MHz, can fulfill the test purpose and can guarantee the BS declaring support of enhanced DM-RS port type to be tested.

MCS

Regarding MCS, since TDLC300-100 is agreed, MCS 16 can be considered for PUSCH requirement definition. As shown in our results, with high MCS level, such as MCS21, the maximum throughput cannot be achieved under 2T2R antenna configuration. Therefore, similar as Rel-15, MCS 16 can be reused.

In summary, the following test parameters can be considered for PUSCH requirement with DMRS enhancement.

Proposal 2:  The following simulation assumption can be considered for PUSCH requirement with DMRS enhancement

- SCS&CBW

  - 15KHz SCS ,5MHz

  - 30KHz SCS, 10MHz

- MCS:  MCS 16

# 3	Simulation Results

In this section, the initial simulation results are provided based on agreed simulation assumption as following

Table 3-1: simulation assumption for PUSCH with Rel-18 DMRS port

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15 kHz SCS: 3D1S1U, S=10D:2G:2U 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {8}, {8,9} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
|  | DM-RS type | enhanced-dmrs-Type_r18 |
| Time domain | PUSCH mapping type | A and B |
| resource | Start symbol | 0 |
| assignment | Allocation length | 14 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
| assignment | Frequency hopping | Disabled |
| TPMI index for 2Tx two-layer spatial multiplexing transmission |  | 0 |
| TPMI index for 4Tx four-layer spatial multiplexing transmission |  | 0 |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to FDD and TDD with different UL-DL pattern. |  |  |

Table 2:  initial simulation results for PUSCH with Rel-18 DMRS port with 1T2R, mapping type A

| Test Cases | SCS&BW | RB | MCS | Channel | Mapping Type | Length of symbol | Numb of Tx and Rx | Number of DMRS | SNR @70% with Rel-18 DMRS ports |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15KHz 5MHz | 25 | 16 | TDLC300-100 | A | 14 | 1Tx2Rx | 1+1 (2,11) | 8.7 |
| 2 | 30KHz 10MHz | 24 | 16 | TDLC300-100 | A | 14 | 1Tx2Rx | 1+1 (2,11) | 8.7 |
| 3 | 15KHz 20MHz | 106 | 16 | TDLC300-100 | A | 14 | 1Tx2Rx | 1+1 (2,11) | 9.1 |
| 4 | 30KHz 100MHz | 273 | 16 | TDLC300-100 | A | 14 | 1Tx2Rx | 1+1 (2,11) | 9.0 |
| 5 | 15KHz 5MHz | 25 | 21 | TDLC300-100 | A | 14 | 1Tx2Rx | 1+1 (2,11) | 12.8 |
| 6 | 30KHz 10MHz | 24 | 21 | TDLC300-100 | A | 14 | 1Tx2Rx | 1+1 (2,11) | 12.7 |
| 7 | 15KHz 20MHz | 106 | 21 | TDLC300-100 | A | 14 | 1Tx2Rx | 1+1 (2,11) | 13.4 |
| 8 | 30KHz 100MHz | 273 | 21 | TDLC300-100 | A | 14 | 1Tx2Rx | 1+1 (2,11) | 13.6 |

Table 3:  initial simulation results for PUSCH with Rel-18 DMRS port with 2T2R, mapping type A

| Test Cases | SCS&BW | RB | MCS | Channel | Mapping Type | Length of symbol | Numb of Tx and Rx | Number of DMRS | SNR @70% with Rel-18 DMRS ports |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15KHz 5MHz | 25 | 16 | TDLC300-100 | A | 14 | 2Tx2Rx | 1+1 (2,11) | 16.8 |
| 2 | 30KHz 10MHz | 24 | 16 | TDLC300-100 | A | 14 | 2Tx2Rx | 1+1 (2,11) | 18.7 |
| 3 | 15KHz 20MHz | 106 | 16 | TDLC300-100 | A | 14 | 2Tx2Rx | 1+1 (2,11) | 17.0 |
| 4 | 30KHz 100MHz | 273 | 16 | TDLC300-100 | A | 14 | 2Tx2Rx | 1+1 (2,11) | 19.5 |
| 5 | 15KHz 5MHz | 25 | 21 | TDLC300-100 | A | 14 | 2Tx2Rx | 1+1 (2,11) | 22.1 |
| 6 | 30KHz 10MHz | 24 | 21 | TDLC300-100 | A | 14 | 2Tx2Rx | 1+1 (2,11) | N/A |
| 7 | 15KHz 20MHz | 106 | 21 | TDLC300-100 | A | 14 | 2Tx2Rx | 1+1 (2,11) | 22.5 |
| 8 | 30KHz 100MHz | 273 | 21 | TDLC300-100 | A | 14 | 2Tx2Rx | 1+1 (2,11) | N/A |

Table 4:  initial simulation results for PUSCH with Rel-18 DMRS port with 1T2R, mapping type B

| Test Cases | SCS&BW | RB | MCS | Channel | Mapping Type | Length of symbol | Numb of Tx and Rx | Number of DMRS | SNR @70% with Rel-18 DMRS ports |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15KHz 5MHz | 25 | 16 | TDLC300-100 | B | 14 | 1Tx2Rx | 1+1 (2,11) | 8.7 |
| 2 | 30KHz 10MHz | 24 | 16 | TDLC300-100 | B | 14 | 1Tx2Rx | 1+1 (2,11) | 8.8 |
| 3 | 15KHz 20MHz | 106 | 16 | TDLC300-100 | B | 14 | 1Tx2Rx | 1+1 (2,11) | 9.0 |
| 4 | 30KHz 100MHz | 273 | 16 | TDLC300-100 | B | 14 | 1Tx2Rx | 1+1 (2,11) | 9.0 |
| 5 | 15KHz 5MHz | 25 | 21 | TDLC300-100 | B | 14 | 1Tx2Rx | 1+1 (2,11) | 12.8 |
| 6 | 30KHz 10MHz | 24 | 21 | TDLC300-100 | B | 14 | 1Tx2Rx | 1+1 (2,11) | 12.7 |
| 7 | 15KHz 20MHz | 106 | 21 | TDLC300-100 | B | 14 | 1Tx2Rx | 1+1 (2,11) | 13.4 |
| 8 | 30KHz 100MHz | 273 | 21 | TDLC300-100 | B | 14 | 1Tx2Rx | 1+1 (2,11) | 13.6 |

Table 5:  initial simulation results for PUSCH with Rel-18 DMRS port with 2T2R, mapping type B

| Test Cases | SCS&BW | RB | MCS | Channel | Mapping Type | Length of symbol | Numb of Tx and Rx | Number of DMRS | SNR @70% with Rel-18 DMRS ports |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15KHz 5MHz | 25 | 16 | TDLC300-100 | B | 14 | 2Tx2Rx | 1+1 (2,11) | 16.8 |
| 2 | 30KHz 10MHz | 24 | 16 | TDLC300-100 | B | 14 | 2Tx2Rx | 1+1 (2,11) | 18.7 |
| 3 | 15KHz 20MHz | 106 | 16 | TDLC300-100 | B | 14 | 2Tx2Rx | 1+1 (2,11) | 17.0 |
| 4 | 30KHz 100MHz | 273 | 16 | TDLC300-100 | B | 14 | 2Tx2Rx | 1+1 (2,11) | 19.4 |
| 5 | 15KHz 5MHz | 25 | 21 | TDLC300-100 | B | 14 | 2Tx2Rx | 1+1 (2,11) | 22.1 |
| 6 | 30KHz 10MHz | 24 | 21 | TDLC300-100 | B | 14 | 2Tx2Rx | 1+1 (2,11) | N/A |
| 7 | 15KHz 20MHz | 106 | 21 | TDLC300-100 | B | 14 | 2Tx2Rx | 1+1 (2,11) | 22.4 |
| 8 | 30KHz 100MHz | 273 | 21 | TDLC300-100 | B | 14 | 2Tx2Rx | 1+1 (2,11) | N/A |

Observation 1: The maximum throughput cannot be achieved with MCS 21 under 2T2R antenna condition

Observation 2:  Around 0.5dB performance degradation with large CBW condition compared with small CBW condition.

# 4	Conclusion

In this contribution, the view on the remaining issue of test setup for PUSCH requirements with DMRS enhancement was provided. Meanwhile, the initial results are provided for alignment purpose

Proposal 1:  The following test applicability rule can be considered

- a BS that passes tests with enhanced DM-RS port (enhanced-dmrs-Type_r18) can consider the tests with same test parameters but configuring legacy DM-RS port as passed

Proposal 2:  The following simulation assumption can be considered for PUSCH requirement with DMRS enhancement

- SCS&CBW

  - 15KHz SCS ,5MHz

  - 30KHz SCS, 10MHz

- MCS:  MCS 16

Observation 1: The maximum throughput cannot be achieved with MCS 21 under 2T2R antenna condition

Observation 2:  Around 0.5dB performance degradation with large CBW condition compared with small CBW condition.

# References

R4-2321141, WF on NR_MIMO_evo_DL_UL_demod