3GPP TSG-RAN WG4 #110	R4-2401606

# Athens, Greece, February 26th – March 1st, 2024

Agenda item:	8.25.5

Source: 	Qualcomm Incorporated

Title: 	RRM Performance Requirements for Mobile IAB

Document for:	Approval

# Introduction

Discussion on the definition of the mobile IAB-MT RRM performance requirements started in RAN4#109 and a WF was agreed in [1]. In this paper we discuss some further details of the RRM performance requirements and test cases.

# Discussion

## Measurement Accuracy Requirements

In [1] the following was agreed:

- RAN4 to introduce intra-frequency and L1-RSRP absolute and relative measurement accuracy requirements for mobile IAB-MT by reusing the corresponding UE requirements from 38.133 as follows: for SS-RSRP/RSRQ/SINR and CSI-RS RSRP/RSRQ/SINR, SSB/CSI-RS L1-RSRP in FR1 and FR2-1.

A new section to introduce these requirements is needed in TS 38.174, this should be 12.5B – Measurement Performance requirements. The requirements to be reused can be found in the following sections of TS 38.133:

Clause 10.1.2 – FR1 RSRP

Clause 10.1.3 – FR2 RSRP

Clause 10.1.6 – RSRP reporting mapping

Clause 10.1.7 – FR1 RSRQ

Clause 10.1.8 – FR2 RSRQ

Clause 10.1.11 – RSRQ reporting mapping

Clause 10.1.12 – FR1 SINR

Clause 10.1.13 – FR2 SINR

Clause 10.1.16 – SINR report mapping

Clause 10.1.19 – FR1 L1-RSRP

Clause 10.1.20 – FR2 L1-RSRP

The side conditions would have to be adapted for the mIAB-MT, the requirements for the measurement procedures in Section 12.4B can be used as reference.

Proposal 1: The measurement performance requirements for mIAB-MT should be introduced in Clause 12.5B of TS 38.174. The requirements to be reused from TS 38.133 can be found in the list above.

## RRM Tests

The high level list of RRM tests to be introduced for the mIAB-MT was agreed in [1]. Most of the sections with the corresponding UE tests to be reused were also captured in the WF [1].

The list agreed in [1] missed the measurement accuracy test cases which are also needed to verify the corresponding requirements, these can be introduced based on the existing UE tests from TS 38.133

Proposal 2: Introduce the following measurement accuracy test cases based on the corresponding tests from TS 38.133:

- FR1 SS-RSRP accuracy test : Section A.6.7.1.1

- FR1 SS-RSRQ accuracy test: Section A.6.7.2.1

- FR1 SS-SINR accuracy test: Section A.6.7.3.1

- FR1 L1-SINR accuracy test: Section A.6.7.4.1

- FR2 SS-RSRP accuracy test : Section A.7.7.1.1

- FR2 SS-RSRQ accuracy test: Section A.7.7.2.1

- FR2 SS-SINR accuracy test: Section A.7.7.3.1

- FR2 L1-SINR accuracy test: Section A.7.7.4.1

Some RRM tests were already defined for the LA IAB-MT in Rel-16. The test configurations are defined in Annex G.1 of TS 38.174 and the tests are defined in Annex G.2. The test configurations can be reused for the mIAB-MT tests.

Proposal 3: Reuse the test configurations in Annex G.1 for the mIAB-MT RRM tests.

Depending on the test cases, some new configurations might be needed. These can be further discussed when the draft CR for the test cases will be submitted.

## CR Split

The CRs needed to conclude the performance work should be split among multiple companies. This can be done during the RAN4 discussion and capture in a WF as outcome of RAN4#110.

Proposal 4: The following CR work split should be discussed and agreed:

| Requirements/Tests | Company |
| --- | --- |
| Measurement Accuracy requirements |  |
| Measurement Accuracy test cases |  |
| Handover tests |  |
| Active TCI switch tests |  |
| Measurement procedures – intra-frequency measurement tests |  |
| Transmit timing tests |  |
| Signaling characteristics |  |

# Conclusion

In this paper we further analyzed the RRM performance requirements and test case definition for the mIAB-MT. We propose the following:

Proposal 1: The measurement performance requirements for mIAB-MT should be introduced in Clause 12.5B of TS 38.174. The requirements to be reused from TS 38.133 can be found in the list below:

Clause 10.1.2 – FR1 RSRP

Clause 10.1.3 – FR2 RSRP

Clause 10.1.6 – RSRP reporting mapping

Clause 10.1.7 – FR1 RSRQ

Clause 10.1.8 – FR2 RSRQ

Clause 10.1.11 – RSRQ reporting mapping

Clause 10.1.12 – FR1 SINR

Clause 10.1.13 – FR2 SINR

Clause 10.1.16 – SINR report mapping

Clause 10.1.19 – FR1 L1-RSRP

Clause 10.1.20 – FR2 L1-RSRP

Proposal 2: Introduce the following measurement accuracy test cases based on the corresponding tests from TS 38.133:

- FR1 SS-RSRP accuracy test : Section A.6.7.1.1

- FR1 SS-RSRQ accuracy test: Section A.6.7.2.1

- FR1 SS-SINR accuracy test: Section A.6.7.3.1

- FR1 L1-SINR accuracy test: Section A.6.7.4.1

- FR2 SS-RSRP accuracy test : Section A.7.7.1.1

- FR2 SS-RSRQ accuracy test: Section A.7.7.2.1

- FR2 SS-SINR accuracy test: Section A.7.7.3.1

- FR2 L1-SINR accuracy test: Section A.7.7.4.1

Proposal 3: Reuse the test configurations in Annex G.1 for the mIAB-MT RRM tests.

Proposal 4: The following CR work split should be discussed and agreed:

| Requirements/Tests | Company |
| --- | --- |
| Measurement Accuracy requirements |  |
| Measurement Accuracy test cases |  |
| Handover tests |  |
| Active TCI switch tests |  |
| Measurement procedures – intra-frequency measurement tests |  |
| Transmit timing tests |  |
| Signaling characteristics |  |

# References

- RP-2321534, “WF on NR_mobile_IAB”, Qualcomm Incorporated