3GPP TSG-RAN WG4 Meeting #110										                      R4-2401614

Athens, Greece, Feb 26th – Mar 01th, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on core requirement for SL AoA and SL RTOA measurement requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-01-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Reason for change#1: For measurement for multiple UEs, the delay requirements need to be defined in spec. Reason for change#2: If the reception/transmission of the slots containing SL-PRS is dropped, the measurement period can be extended. Since it is a normal procedure because of the selection/reselection of the sync source in SL, it should be also noted and specified in SL positioning. Reason for change#3: No need to define accuracy requirement for SL AoA, but to define requirement for SL RTOA. Reason for change#4: TS 38.355 has already defined the signalling which is [TBD] in current SL positioning requirement. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change#1: Capture the delay requirement for SL AoA and SL RTOA measurement for multiple UEs. Change#2: Capture the measurement requirements when the reception/transmission of the slots containing SL-PRS is dropped. Change#3: Remove the FFS of whether to define accuracy requirements for SL AoA and SL RTOA, and make the related change in other contents. Change#4: Remove the square brackets for signalling specified in TS 38.355. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There still have pending issues for corresponding core requirements on SL positioning measurement in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Clause 12A.6, 12A.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.355 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

## 12A.6	SL AoA measurements

#### 12A.6.1	Introduction

The requirements in clause 12A.6.5 shall apply provided the UE has received sl-AOA-RequestLocationInformation from LMF or another UE via SLPP requesting the UE to measure and report SL AoA measurements defined in TS 38.215 [4].

#### 12A.6.2	Requirements Applicable

The requirements in clause 12A.6 apply for periodic, aperiodic, and triggered SL AoA measurements, provided:

-	SL AoA related side conditions given in clause [TBD] for FR1 are met for a corresponding Band.

#### 12A.6.3	Measurement Capability

SL AoA measurement capability is as indicated by the UE in sl-AOA-ProvideCapabilities according to TS 38.355 [37].

#### 12A.6.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE reporting to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

For UE reporting to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the sidelink STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

The measurement reporting delay excludes any delay caused by no SL resources or no SL-PRS resources for UE to send the measurement report.

The reported SL AoA measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses [TBD].

#### 12A.6.5	Measurement Period Requirements

When the physical layer receives sl-AOA-ProvideAssistanceData message from sl-AOA-RequestLocationInformation message from LMF or another UE via SLPP, the UE shall be able to measure multiple  SL AoA measurements, defined in TS 38.215 [4], during TSL AoA,total defined as:

TSL AoA,total=n=1NTSL AoA, n+N−1∗max(TSL AoA, effect,n)

Where

n is the index of positioning UE,

N is the total number of positioning UEs, and

TSL AoA, effect,n is the periodicity of SL AoA measurement for positioning UE n

TSL AoA, n is the measurement period for SL AoA measurement of positioning UE n as specified below:

TSL AoA,totaln=s=1STSL AoA, effect,n,s ,

where,

S is the number of samples for the SL AoA measurement, defined as below:

  - S = 1 for SL-PRS BW>48 PRBs,

  - S = 4 for 24 PRBs ≤SL-PRS BW≤48 PRBs,

TSL AoA,effect,n,s is defined as below,

TSL AoA,effect,n,s=tn.s+1−tn,s, for s<S, provided that TSL AoA,effect,n,s≥Tdur,n,s+TSLproc,n , where tn,s and tn,s+1 are the start of the s-th and (s+1)-th slot, respectively, where UE is configured to measure SL-PRS,

TSL AoA,effect,n,s=Tdur,n,s+ΔSLproc,n , for s=S,

Tdur,n,s is the SL-PRS duration for SL-PRS sample s of the SL AoA measurement,

ΔSLproc,n=[TBD] is the processing time.

If the reception/transmission of the slots containing SL-PRS is dropped, the measurement period can be longer.

Editor’s note: FFS whether to specify how exactly the measurement period is extended when the reception/transmission of the slots containing SL-PRS is dropped

The requirements in clause 12A.6 do not apply if the SL-PRS configuration given by sl-PRS-AssistanceData exceeds any of the UE measurement capabilities given by sl-PRS-ResourcesCapability in NR-SL-AOA-ProvideCapabilities.

When a synchronization reference source change occurs at the measuring UE or at the UE configured to transmit SL-PRS for the SL AoA measurement, while the UE is performing the SL AoA measurement, then the UE shall continue and complete the on-going SL AoA measurements while meeting the measurement period requirement in this clause

## 12A.7	SL RTOA measurements

#### 12A.7.1	Introduction

The requirements in clause 12A.7.5 shall apply provided the UE has received sl-RTOA-RequestLocationInformation from LMF or another UE via SLPP requesting the UE to measure and report SL RTOA measurements defined in TS 38.215 [4].

#### 12A.7.2	Requirements Applicable

The requirements in clause 12A.7 apply for periodic, aperiodic, and triggered SL RTOA measurements, provided:

-	SL RTOA related side conditions given in clause [TBD] for FR1 are met for a corresponding Band.

#### 12A.7.3	Measurement Capability

SL RTOA measurement capability is as indicated by the UE in sl-RTOA-ProvideCapabilities according to TS 38.355 [37].

#### 12A.7.4	Measurement Reporting Requirements

The measurement reporting delay is defined as the time between the moment when the measurement report is triggered and the moment when the UE starts to transmit the measurement report over the air interface.

For UE reporting to LMF, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the DCCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the uplink DCCH. The delay uncertainty is: 2 x TTIDCCH where TTIDCCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

For UE reporting to another UE, this requirement assumes that the measurement report is not delayed by other SLPP signalling on the STCH. This measurement reporting delay excludes a delay uncertainty resulted when inserting the measurement report to the TTI of the sidelink STCH. The delay uncertainty is: 2 x TTISTCH where TTISTCH is the duration of subframe or slot or subslot when the measurement report is transmitted on the PSSCH with subframe or slot or subslot duration.

The measurement reporting delay excludes any delay caused by no SL resources or no SL-PRS resources for UE to send the measurement report.

The reported SL RTOA measurement values contained in measurement reports shall be based on the measurement report mapping requirements specified in clauses [TBD].

The SL RTOA measurements performed and reported according to this section shall meet the SL RTOA measurement accuracy requirements in clause [TBD], for each measured SL-PRS resource.

#### 12A.7.5	Measurement Period Requirements

When the physical layer receives sl-RTOA-ProvideAssistanceData message from sl-RTOA-RequestLocationInformation message from LMF or another UE via SLPP, the UE shall be able to measure multiple  SL RTOA measurements, defined in TS 38.215 [4], during TSL RTOA,total defined as:

TSL RTOA,total=n=1NTSL RTOA, n+N−1∗max(TSL RTOA, effect,n)

Where

N is the total number of positioning UEs, and

TSL RTOA, effect,n is the periodicity of SL RTOA measurement for positioning UE n

TSL RTOA, n is the measurement period for SL RTOA measurement of positioning UE n as specified below:

TSL RTOA,total,n=s=1STSL RTOA, effect,n,s ,

where,

S is the number of samples for the SL RTOA measurement, defined as below:

  - S = 1 for SL-PRS BW>48 PRBs,

  - S = 4 for 24 PRBs ≤SL-PRS BW≤48 PRBs,

TSL RTOA,effect,n,s is defined as below,

TSL RTOA,effect,n,s=tn,s+1−tn,s, for s<S, provided that TSL RTOA,effect,n,s≥Tdur,n,s+TSLproc,n , where tn,s and tn,s+1 are the start of the s-th and (s+1)-th slot, respectively, where UE is configured to measure SL-PRS,

TSL RTOA,effect,n,s=Tdur,n,s+ΔSLproc,n , for s=S,

Tdur,n,s is the SL-PRS duration for SL-PRS sample s of the SL RTOA measurement,

ΔSLproc,n=[TBD] is the processing time.

If the reception/transmission of the slots containing SL-PRS is dropped, the measurement period can be longer.

Editor’s note: FFS whether to specify how exactly the measurement period is extended when the reception/transmission of the slots containing SL-PRS is dropped.

The requirements in clause 12A.6 do not apply if the SL-PRS configuration given by [NR-SL-PRS-AssistanceData exceeds] exceeds any of the UE measurement capabilities given by [NR-SL-PRS-ResourcesCapability in NR-SL-RTOA-ProvideCapabilities].

If the synchronization reference source changes at the measuring UE, while the UE is performing the SL RTOA measurements, the UE shall restart the SL RTOA measurements after the synchronization reference source change.

Editor’s note: FFS whether to limit the number of restarting.

End of Change 1