3GPP TSG-RAN WG4 Meeting # 110	R4-2401681

# Athens, GR, 26 Feb – 01 Mar, 2024

Title: 	Simulation results for eDSS performance requirements

Source: 	Huawei, HiSilicon

Agenda item:	8.27.2

Document for:	Discussion

Background

In last meeting, a WF on enhancement of DSS demodulation requirements was approved and many open issues were left. In this contribution, we provide our simulation results.

- Simulation results

This section provides corresponding simulation results based on evaluation cases in WF[1] with simulation assumptions in Appendix.

Figure 2-1: BLER-SNR curve for PDCCH

Table 2-1: PDCCH simulation results for eDSS

| Test case | SCS(KHz)/CBW (MHz) | CORESET RB | Payload | AL | Propagation condition | Antenna configuration | SNR(dB)@ 1% BLER |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 2x2 Low | 1.5 |
| 2 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 2x2 Low | -3.4 |
| 3 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 2x4 Low | -3.6 |
| 4 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 2x4 Low | -7.0 |
| 5 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 4x2 Low | -0.3 |
| 6 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 4x2 Low | -4.1 |
| 7 | 15/10 | 48 | 39 | 4 | TDLA30-10 | 4x4 Low | -4.2 |
| 8 | 15/10 | 48 | 39 | 8 | TDLC300-100 | 4x4 Low | -7.2 |

- Conclusion

In this contribution we provide our simulation results for eDSS performance requirements.

- Reference

[1]    R1-2321139  WF for Enhancement of Dynamic Spectrum Sharing demodulation requirements. Ericsson

- Appendix

Table 5-1: Simulation assumptions

| Parameters | Values |
| --- | --- |
| Duplex mode | FDD |
| SCS | 15 kHz |
| Bandwidth | 10 MHz |
| Channel model | TDLA30-10 and TDLC300-100 |
| Antenna configuration | 2x2, 2x4, 4x2, 4x4 |
| CRS | 4 port CRS |
| DCI payload (excluding CRC) | 39 bits |
| Interleaving | Non-interleaved |
| Precoding | Precoder cycling per REG bundle |
| REG bundle size | 6 |
| Shift index | 0 |
| CORSET duration | 2 (2nd symbol and 3rd symbol) |
| CORSET RB | 48 |
| AL | 4, 8 |
| - Note 1: The bandwidth of NR CORESET is within the bandwidth of LTE. - Note 2: gNB punctures the PDCCH data and DMRS REs overlapped with LTE CRS. Note 3: PDCCH channel estimation is assumed to use only the clean PDCCH symbol. |  |