3GPP TSG-RAN WG4 Meeting # 110	R4-2401683

# Athens, GR, 26 Feb – 01 Mar, 2024

Title: 	Simulation results on UE performance for Non-collocated CA

Source: 	Huawei, HiSilicon

Agenda item:	8.7.4

Document for:	Discussion

Background

In last meeting, a WF on NonCol_intraB_ENDC_NR_CA Demod [1] was agreed. In this contribution we provide our simulation results.

Discussions

The simulation assumptions are captured in Table 2-1:

Table 2-1: Simulation assumptions

|  | Type 2 UE NR-CA demodulation requirements |
| --- | --- |
| Received power difference | 25 dB |
| Received time difference | 33 µs |
| Channel model | AWGN (TS38.101-4 B.1.1) |
| Maximum number of HARQ transmission | 1 |
| Signal/Noise setting | Set both Es and Noc to set SNR test points |
| Throughput measurement procedure | Measure both carriers |
| Antenna configuration | 2x2 for both carriers |
| Rank configuration | Rank1 for carrier with lower power Rank2 for carrier with higher power |
| MCS Table | MCS Table 1 for lower CC MCS Table 2 for stronger CC |
| Tx EVM | 6% for weaker CC (QPSK) 3% for stronger CC (256QAM) |
| Test metric | 70% of the maximum throughput |

The simulation results are captured in Table 2-2 and 2-3.

Table 2-2: Simulation results for weaker CC

| MCS | SNR(dB)@70% of max TP (Ideal) | SNR(dB)@70% of max TP (Impairment) |
| --- | --- | --- |
| 4 | -4.3 | -3.3 |
| 5 | -3.4 | -2.4 |

Table 2-3: Simulation results for stronger CC

| MCS | SNR(dB)@70% of max TP (Ideal) | SNR(dB)@70% of max TP (Impairment) |
| --- | --- | --- |
| 21 | 19.5 | 20.5 |
| 22 | 20.4 | 21.4 |
| 23 | 22.1 | 23.1 |
| 24 | 22.6 | 23.6 |
| 25 | 24.4 | 25.4 |

Conclusion

In this paper we provide our simulation results for Non-collocated CA

.

Reference

[1]    R4-2321052  WF for NonCol_intraB_ENDC_NR_CA_Demod. Ericsson