3GPP TSG-RAN WG4 Meeting #110	R4-2401709

# Athens, Greece, 26th Feb - 1st Mar, 2024

Title: 	Simulation results for NR FR2 multi-Rx chain DL reception

Source: 	Huawei, HiSilicon

Agenda item:	8.3.4.1

Document for:	Discussion

Background

During RAN4#109 meeting, WF [1] on UE demodulation and CSI requirements for FR2 multi-Rx DL reception was approved. In this contribution, we provide our updated simulation results for NR FR2 multi-Rx chain DL reception.

Simulation

PDSCH

Here we provide the updated simulation results for UE PDSCH demodulation for NR FR2 multi-Rx chain DL reception for alignment, as shown in Table 2.1-4 to Table 2.1-6, based on simulation assumption as shown in Table 2.1-1 to Table 2.1-3 respectively.

Table 2.1-1 Simulation assumption for mDCI mTRP fully-overlapping

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  | TRxP #1 |  |
| PDCCH configuration | TCI state |  | TCI State #1 | TCI State #2 |
|  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  | TDD |  |
| Active DL BWP index |  |  | 1 |  |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | k0 |  | 0 |  |
|  | Starting symbol (S) |  | 1 |  |
|  | Length (L) |  | Specific to each Reference channel as defined in A.3.2.2 |  |
|  | PRB bundling type |  | Static |  |
|  | PRB bundling size |  | 2 |  |
|  | Resource allocation type |  | Type 1 |  |
|  | RBG size |  | Config2 |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  | 1+1: {1000} | 1+1: {1002} |
|  | TCI state |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  | Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
| Resource allocation |  |  | Fully-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  | us | -0.0.0625 |  |
| Frequency offset of the second TRxP from the first TRxP |  | Hz | 600 |  |
| Precoding configuration |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |

Table 2.1-2 Simulation assumption for mDCI mTRP non-overlapping

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  | TRxP #1 |  |
| PDCCH configuration | TCI state |  | TCI State #1 | TCI State #2 |
|  | CORESETPoolIndex |  | 0,1 |  |
| Duplex mode |  |  | TDD |  |
| Active DL BWP index |  |  | 1 |  |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | k0 |  | 0 |  |
|  | Starting symbol (S) |  | 1 |  |
|  | Length (L) |  | Specific to each Reference channel as defined in A.3.2.2 |  |
|  | PRB bundling type |  | Static |  |
|  | PRB bundling size |  | 2 |  |
|  | Resource allocation type |  | Type 1 |  |
|  | RBG size |  | Config2 |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  | 2+2: {1000,1001} | 2+2: {1002,1003} |
|  | TCI state |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  | Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
| Resource allocation |  |  | non-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  | us | 0.25 |  |
| Frequency offset of the second TRxP from the first TRxP |  | Hz | 0 |  |
| Precoding configuration |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs. Transmission from TRxP #1 uses CORESETPoolIndex 0 and transmission from TRxP #2 uses CORESETPoolIndex 1 |  |  |  |  |

Table 2.1-3 Simulation assumption for sDCI SDM

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | TRxP #1(Note 1) | TRxP #2(Note 1) |
| Transmit TRxP of SSB |  |  | TRxP #1 |  |
| PDCCH configuration | TCI state |  | TCI State #1 |  |
|  | CORESETPoolIndex |  | 0 |  |
| Duplex mode |  |  | TDD |  |
| Active DL BWP index |  |  | 1 |  |
| PDSCH configuration | Mapping type |  | Type A |  |
|  | k0 |  | 0 |  |
|  | Starting symbol (S) |  | 1 |  |
|  | Length (L) |  | Specific to each Reference channel as defined in A.3.2.2 |  |
|  | PRB bundling type |  | Static |  |
|  | PRB bundling size |  | 2 |  |
|  | Resource allocation type |  | Type 1 |  |
|  | RBG size |  | Config2 |  |
|  | VRB-to-PRB mapping type |  | Non-interleaved |  |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |  |
| PDSCH DMRS configuration | Antenna port indexes |  | 1+1: {1000} | 1+1: {1002} |
|  | TCI state |  | TCI State #1 | TCI State #2 |
|  | DMRS Type |  | Type 1 |  |
|  | Number of additional DMRS |  | 1 |  |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |  |
| Resource allocation |  |  | Full-overlapping |  |
| Timing offset of the second TRxP from the first TRxP |  | us | -0.0625 |  |
| Frequency offset of the second TRxP from the first TRxP |  | Hz | 600 |  |
| Precoding configuration |  |  | SP Type I, independent precoding generation is applied for both TRxPs, random per slot with PRB bundling granularity. |  |
| Note 1: PDSCH transmission is done from both TRxPs (PDSCH Layer 0 is transmitted from TRxP #1 and PDSCH layer 1 is transmitted from TRxP #2) |  |  |  |  |

Table 2.1-4 Simulation results for mDCI mTRP fully-overlapping

| Case number | MCS | Rank | UE processing | ρ | SNR@70%maximum throughput |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Ideal | Impairment |
| 1 | 17 | 1+1 | separate | -12dB | 9.8 | 11.8 |

Table 2.1-5 Simulation results for mDCI mTRP non-overlapping

| Case number | MCS | Rank | UE processing | ρ | SNR@70%maximum throughput |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Ideal | Impairment |
| 1 | 13 | 2+2 | separate | -12dB | 11.2 | 13.2 |

Table 2.1-6 Simulation results for sDCI SDM

| Case number | MCS | Rank | UE processing | ρ | SNR@70%maximum throughput |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Ideal |  | Impairment |  |
|  |  |  |  |  | 1 PTRS port | 2 PTRS port | 1 PTRS port | 2 PTRS port |
| 1 | 17 | 1+1 | separate | -12dB | 10.0 | 10.0 | 12.0 | 12.0 |

PMI reporting

Here we provide the updated simulation results for UE PMI reporting for NR FR2 multi-Rx chain DL reception for alignment, as shown in Table 2.2-2 and Figure 2.2-1, based on simulation assumption as shown in Table 2.2-1.

Table 2.2-1 Simulation assumption for UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception

| Parameter |  | Unit | Value |  |
| --- | --- | --- | --- | --- |
|  |  |  | TRxP #1 (Note 1) | TRxP #2 (Note 1) |
| Transmit TRxP of SSB |  |  | TRxP #1 |  |
| PDCCH configuration | TCI state |  | TCI State #1 |  |
|  | CORESETPoolIndex |  | 0 |  |
| Duplex mode |  |  | TDD |  |
| Bandwidth |  | MHz | 100 |  |
| Subcarrier spacing |  | kHz | 120 |  |
| TDD DL-UL configurations |  |  | FR2.120-1 as specified in Annex A of TS 38.101-4 |  |
| Active DL BWP index |  |  | 1 |  |
| Propagation channel |  |  | TDLA30-35 |  |
| Antenna configuration per TRxP |  |  | XP 2x4 (N1,N2) = (2,1), (4 antenna at UE across 2 panels) |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 of TS 38.101-4 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 1.75 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |

Table 2.2-2 Simulation results for UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception

| Case Number | MCS | γ |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 70% |  | 90% |  |
|  |  | 1 PTRS port | 2 PTRS port | 1 PTRS port | 2 PTRS port |
| 1 | 13 | 1.31 | 1.31 | 1.68 | 1.68 |
| 2 | 11 | 1.31 | 1.31 | 1.69 | 1.69 |

Figure 2.2-1 Simulation results for UE CSI reporting requirements for NR FR2 multi-Rx chain DL reception

Conclusions

In this contribution, we provide our updated simulation results for NR FR2 multi-Rx chain DL reception.

Reference

R4-2321140, WF on UE demodulation and CSI requirements for FR2 multi-Rx DL reception, RAN4#109, Qualcomm