3GPP TSG-RAN WG4 Meeting #110	R4-2401719

# Athens, Greece, 26th Feb - 1st Mar, 2024

Title: 	Simulation assumption on demodulation requirements for NR NTN enhancements

Source: 	Huawei, HiSilicon

Agenda item:	8.18.8

Document for:	Approval

Background

In this contribution, the simulation assumption on demodulation requirements for NR NTN enhancements is captured for simulation results alignment.

Discussion

UE side

The following TDD pattern is used for all UE cases:

- DDDSU, S slot:10D+2G+2U

PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 1 |
|  | Length (L) |  | 13 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CSI-RS for tracking | CSI-RS periodicity | Slots | 160 for CSI-RS resource 1,2,3,4. |
|  | CSI-RS offset | Slots | 80 for CSI-RS resource 1 and 2 81 for CSI-RS resource 3 and 4 |
| Number of HARQ Processes |  |  | 16 for Test 1-1, Test 1-2 32 for Test 1-3 For Test 1-4: 4 with feedback disabled, 12 with feedback enabled in 16 HARQ processes with re-Tx disable for all HARQ processes and only transmit initial transmissions with NDI toggling. Throughput shall be measured on processes with HARQ enabled. Which 4 processes to disable are randomly select at test configuration. |
| Maximum number of HARQ transmission |  |  | 4 for Test 1-1, Test 1-2, Test 1-3 Disabled for all HARQ processes for Test 1-4 |

| Test | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Test metric |
| --- | --- | --- | --- | --- | --- |
| 1-1 | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-[600] | TBD | 70% of maximum throughput |
| 1-2 | 200 / 120 | 16QAM, 0.48 | NTN-TDLC5-[600] | TBD | 70% of maximum throughput |
| 1-3 | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-[600] | TBD | 70% of maximum throughput |
| 1-4 | 200 / 120 | QPSK, 0.30 | NTN-TDLC5-[600] | TBD | 70% of maximum throughput |

PDCCH

FFS

| Bandwidth (MHz) | SCS (kHz) | CORESET RB | CORESET duration | Aggregation level | CCE to REG mapping type | REG bundle size | Interleaver size | Shift index | DCI format | Payload (without CRC) | Propagation Condition | Antenna configuration and correlation Matrix | Test metric |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100 | 120 | 60 | 1 | 2 | Interleaved | 2 | 3 | 0 | 1_0 | 40 | NTN-TDLC5-[600] | TBD | 1% of Pm-dsg |
| 100 | 120 | 60 | 1 | 4 | Interleaved | 6 | 2 | 0 | 1_1 | 56 | NTN-TDLC5-[600] | TBD | 1% of Pm-dsg |
| 100 | 120 | 60 | 1 | 8 | Interleaved | 2 | 3 | 0 | 1_1 | 56 | NTN-TDLC5-[600] | TBD | 1% of Pm-dsg |
| 100 | 120 | 60 | 2 | 16 | Interleaved | 2 | 3 | 0 | 1_0 | 40 | NTN-TDLC5-[600] | TBD | 1% of Pm-dsg |

SAN side

Above 10GHz bands

PUSCH

Normal PUSCH with CP-OFDM

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled |
|  | Time density (LPT-RS) | Disabled |
| Antenna configuration |  | 1x1, 1x2 |
| Propagation condition |  | NTN-TDLC5-3000 Low |
| MCS |  | MCS 2/[16/20] in Table 1 |
| SCS |  | [120kHz] |
| Channel bandwidth |  | 50MHz |
| Test metric |  | 70% of maximum throughput |

Normal PUSCH with DFT-s-OFDM

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Enabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | {0} |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
| Frequency domain resource | RB assignment | 30 PRBs in the middle of the test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled |
|  | Time density (LPT-RS) | Disabled |
| Antenna configuration |  | 1x1, 1x2 |
| Propagation condition |  | NTN-TDLC5-3000 Low |
| MCS |  | MCS 2 in Table 1 |
| SCS |  | [120kHz] |
| Channel bandwidth |  | 50MHz |
| Test metric |  | 70% of maximum throughput |

PUSCH repetition Type A

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 3, 0, 3 [Note 1] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS symbols | Pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port(s) | 0 |
|  | DM-RS sequence generation | NID=0, nSCID =0 |
| Time domain resource | PUSCH mapping type | B |
|  | Start symbol index | 0 |
|  | Allocation length | 10 |
|  | PUSCH aggregation factor | n2 |
| Frequency domain resource | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| PT-RS configuration | Frequency density (KPT-RS) | Disabled |
|  | Time density (LPT-RS) | Disabled |
| Antenna configuration |  | 1x1, 1x2 |
| Propagation condition |  | NTN-TDLC5-3000 Low |
| MCS |  | MCS 5 in Table 3 |
| SCS |  | [120kHz] |
| Channel bandwidth |  | 50MHz |
| Test metric |  | 70% of maximum throughput |
| NOTE 1: The effective RV sequence is {0,2,3,1} with slot aggregation |  |  |

PUCCH

PUCCH format 0

| Parameter | Test |
| --- | --- |
| Number of UCI information bits | 1 |
| Number of PRBs | 1 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | N/A for 1 symbol Enabled for 2 symbols |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs - 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 13 for 1 symbol 12 for 2 symbols |
| Antenna configuration | 1x1, 1x2 |
| Propagation condition | NTN-TDLC5-3000 Low |
| SCS | [120kHz] |
| Channel bandwidth | 50MHz |
| Test metric | 1% of DTX to ACK probability 1% of ACK missed detection probability |

PUCCH format 1

| Parameter | Test |
| --- | --- |
| Number of information bits | 2 |
| Number of PRBs | 1 |
| Number of symbols | 14 |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (nrofPRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Initial cyclic shift | 0 |
| First symbol | 0 |
| Index of orthogonal cover code (timeDomainOCC) | 0 |
| Antenna configuration | 1x1, 1x2 |
| Propagation condition | NTN-TDLC5-3000 Low |
| SCS | [120kHz] |
| Channel bandwidth | 50MHz |
| Test metric | 1% of DTX to ACK probability 0.1% of NACK to ACK probability 1% of ACK missed detection probability |

PUCCH format 2 (ACK missed detection)

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| Starting RB location | 0 |
| Intra-slot frequency hopping | N/A |
| Number of PRBs | 4 |
| Number of symbols | 1 |
| The number of UCI information bits | 4 |
| First symbol | 13 |
| DM-RS sequence generation | NID0=0 |
| Antenna configuration | 1x1, 1x2 |
| Propagation condition | NTN-TDLC5-3000 Low |
| SCS | [120kHz] |
| Channel bandwidth | 50MHz |
| Test metric | 1% of DTX to ACK probability 1% of ACK missed detection probability |

PUCCH format 2 (UCI BLER)

| Parameter | Value |
| --- | --- |
| Modulation order | QSPK |
| First PRB prior to frequency hopping | 0 |
| Intra-slot frequency hopping | enabled |
| Frist PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Number of PRBs | 9 |
| Number of symbols | 2 |
| The number of UCI information bits | 22 |
| First symbol | 12 |
| DM-RS sequence generation | NID0=0 |
| Antenna configuration | 1x1, 1x2 |
| Propagation condition | NTN-TDLC5-3000 Low |
| SCS | [120kHz] |
| Channel bandwidth | 50MHz |
| Test metric | 1% of BLER |

PUCCH format 3

| Parameter | Test 1 | Test 2 |
| --- | --- | --- |
| Modulation order | QPSK |  |
| First PRB prior to frequency hopping | 0 |  |
| Intra-slot frequency hopping | enabled |  |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |  |
| Group and sequence hopping | neither |  |
| Hopping ID | 0 |  |
| Number of PRBs | 1 | 3 |
| Number of symbols | 14 | 4 |
| The number of UCI information bits | 16 | 16 |
| First symbol | 0 | 0 |
| Antenna configuration | 1x1, 1x2 |  |
| Propagation condition | NTN-TDLC5-3000 Low |  |
| SCS | [120kHz] |  |
| Channel bandwidth | 50MHz |  |
| Test metric | 1% of BLER |  |

PUCCH format 4

| Parameter | Value |
| --- | --- |
| Modulation order | QPSK |
| First PRB prior to frequency hopping | 0 |
| Number of PRBs | 1 |
| Intra-slot frequency hopping | enabled |
| First PRB after frequency hopping | The largest PRB index – (Number of PRBs – 1) |
| Group and sequence hopping | neither |
| Hopping ID | 0 |
| Number of symbols | 14 |
| The number of UCI information bits | 22 |
| First symbol | 0 |
| Length of the orthogonal cover code | n2 |
| Index of the orthogonal cover code | n0 |
| Antenna configuration | 1x1, 1x2 |
| Propagation condition | NTN-TDLC5-3000 Low |
| SCS | [120kHz] |
| Channel bandwidth | 50MHz |
| Test metric | 1% of BLER |

PRACH

| Parameter | Test |
| --- | --- |
| Burst format | B4, C2 |
| Time error tolerance | 0.13us |
| Ncs | 69 |
| Logical sequence index | 0 |
| v | 0 |
| Antenna configuration | 1x1, 1x2 |
| Propagation condition | NTN-TDLC5-3000 Low |
| Frequency offset | 3000Hz |
| SCS | [120kHz] |
| Channel bandwidth | 50MHz |
| Test metric | 1% of DTX to ACK probability 1% of ACK missed detection probability |

PUSCH with DMRS bundling

Normal PUSCH with CP-OFDM

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence [Note 1] | 0, 0, 0, 0 [Note1] |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | 0 |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A, B |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
|  | PUSCH aggregation factor | n8, n4 |
| pusch-TimeDomainWindowLength |  | 8 for n8, 4 for n4 |
| Frequency domain resource assignment | RB assignment | TBD |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| Antenna configuration |  | 1x1, 1x2 |
| Propagation condition |  | NTN-TDLA100-200 Low |
| MCS |  | MCS 4 in Table 1 |
| SCS |  | 15kHz, 30kHz |
| Channel bandwidth |  | 5MHz for 15kHz SCS 10MHz for 30kHz SCS |
| Test metric |  | 70% of maximum throughput |
| Note 1: The effective RV sequence is {0, 2, 3, 1} with slot aggregation. |  |  |

Proposals

In this contribution, the simulation assumption on demodulation requirements for NR NTN enhancements is captured for simulation results alignment.

Reference

R4-2402660, Topic summary for [110][320] NR_NTN_enh_SAN_UE_demod, RAN4#110, Huawei, HiSilicon

R4-2402865, WF on NR_NTN_enh_SAN_UE_demod, RAN4#110, Huawei, HiSilicon