3GPP TSG-RAN WG4 Meeting#110	R4-2401852

# Athens, Greece, 26 February – 1 March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2134 | rev | - | Current version: | 17.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (Power_Limit_CA_DC) Applicability of exceptions to REFSENS for CA and SUL to HPUE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Power_Limit_CA_DC |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | HPUEs in the field do not indicate support of DL-only CA configurations because minimum requirements or exceptions to these affecting DL SCells are missing for the higher power class. While the performance of the DL SCells may be impacted in some sceanrios due to the higher UL power, any potential UL output power reduction due to a missing requirement means that the UL coverage must be validated before adding DL Scells. This delays further the CA configuration. HPUEs should be able to indicate support of CA band combinations with DL-only CA (non-CA in the UL) for a higher power class if allowed exceptions for a lower power class or the standard CA REFSENS requirement for the supported higher power class is met irrespective of the notes in the tables of Clause 5.5A. TX requirements for UL CA configurations are specified in clause 6. Their DL fallbacks with non-CA in the UL also have to meet the CA REFSENS requirements in Clause 7. These requirements are specified for UL CA configurations with one UL carrier active; only IMD for UL inter-band CA is verified at specific test points with one active UL carrier in each of two bands (along with one exceptional case for harmonics). The standard non-CA requirements in clause 7.3.2 should apply for each DL of a supported BC unless an exception is allowed for a specific combination of UL and DL configurations applicable for the NR bands of the BC, e.g. an exception for cross-band isolation for the supported power class. When a higher power class is specified for a band (including non-CA REFSENS requirements), potential exceptions must be evaluated for all CA band combinations including this band as part of basket WIs. When evaluated for a BC this will noted in the tables of CA configurations in clause 5.5A.3 as e.g. “Minimum requirements for Power Class 2 are applicable for this uplink combination with 1Tx antenna connector in each band or single uplink carrier with up to 2Tx antenna connectors in this downlink/uplink combination”. Notwithstanding, in the absence of an exception for a HPUE, this UE should still be able to indicate support of a non-CA configuration in the UL with the higher power class e.g. PC1.5 if the UE complies - with an allowed exception for an UL/DL configuration corresponding to the said non-CA configuration with the UL power limitied to that of a lower power class e.g. PC2 or PC3 for which an exception is specified or - with the standard CA REFSENS requirement also for the higher power class PC1.5 irrespective of any allowed exception. This would allow HPUEs in the field indicate support a CA configuration with non-CA in the UL for the supported higher power class regardless of the notes on ‘applicability’ of requirements in clause 5.5A and power derating due to missing requirements is avoided. These notes cannot be checked by the gNB to validate the UE power capability. While minimum requirements for all power classes are preferrable also for non-CA in the UL, the above would not impair performance in the field under all circumstances whilst recognising that degradation of DL SCell performance may indeed be slightly worse for a higher power class in some cases with or without a requirement. Example: harmonic mixing exceptions for a non-CA configurations in the UL specifies the MSD for the DL Scell for different power classes for the worst case: the MSD for a band combination increases with the UL power class as shown for the example CA_n25-n77 (from the tables in clause 7.3A.4) but compliance with the requirement for PC3 only as per the above would not imply excessive MSD for a PC1.5 UL that otherwise meets the standard TX requirements for PC1.5 for this UL band: Likewise, exceptions for cross-band isolation for a non-CA configurations in the UL specifies the MSD for the DL Scell for different power classes: the MSD for a band combination increases with the UL power class as shown for the example CA_n41-n77 (from the tables in clause 7.3A.6) but compliance with the requirement for PC3 only would not imply excessive MSD for a PC1.5 UL that otherwise meets the standard TX requirements for PC1.5 for this UL band: |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changes implemented to allow HPUEs indicate support of CA band combinations with DL-only CA (non-CA in the UL) for a higher power class if allowed exceptions for a lower power class or the standard CA REFSENS requirement for the supported higher power class. Clause 7.3A.2.1 (intra-band contiguous CA): Requirements (DRIBC) for a specified CA configuration apply for the default power class unless specified for a supported higher power class (not applicable for FDD in Rel-17). Clause 7.3A.2.2 (intra-band NC CA): Requirements (DRIBNC) for a specified CA configuration apply for the default power class unless specified for a supported higher power class (not applicable for FDD in Rel-17). Clause 7.3A.2.3 (inter-band CA): Exceptions to CA REFSENS requirements clauses 7.3A.4 and 7.3A.6 only apply for inter-band CA configurations with one component carrier per operating band and the UL assigned to one NR band that include at least one combination of UL and DL configurations listed in any one of these two clauses for the band combination of the CA configuration. For an exceptional UL band the CA REFSENS degradation is only specified for a specific UL/DL configuration. For other UL bands the standard requirement in clause 7.3A.2.3 applies. Clause 7.3A.4 (exceptions for harmonics): In case an exception is specified for at least one power class: a UE indicating a BandNR capability ue-PowerClass higher than PC3 for the NR UL band shall meet the requirements in this clause for applicable UL/DL configurations with the uplink power limited to the nominal maximum output power for PC3 unless otherwise specified for the supported power class. In case there is no applicable UL/DL configuration listed for a BC (not allowed, missing or yet to be evaluated) for any power class: for inter-band CA configurations not including any of the UL/DL configurations in this clause, the requirements in sub-clause 7.3A.2.3 apply for any power class. Clause 7.3A.6 (exceptions for cross-band isolation): Changes analogous to those for 7.3A.4. Clause 7.3C.2: corresponding changes for SUL for completeless (the applicability of exceptions clearer for SUL). Isolated impact analysis: UEs not implemented in accordance with this CR would not be impacted by the changes of this CR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | HPUEs in the field report a lower power class when a DL Scell is added with non-CA in the UL due to missing requirements for the supported top-level band combination or missing exceptions for CA REFSENS in 38.101-1 for the supported higher power class. The derating of the power capability implies that the CA configuration/activation may be delayed due to necessary validation of UL coverage before CA configuration. The NW must evaluate the notes on applicable requirements in clause 5.5 to validate the power capability (should be clear from the UE capability alone). Unclear from the standard CA REFSENS requirements in clause 7.3A.2.3 if an exception for an inter-band CA configuration is not specified because - an exception is not allowed - or an exception is not yet specified/included in 38.101-1 and which of the test configuration that applies when an exception is allowed (the ‘exceptions’ are different test configurations). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.3A.2.1, 7.3A.2.2, 7.3A.2.3, 7.3A.4, 7.3A.6, 7.3C.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  | . |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< start of changes >

### 7.3A.2	Reference sensitivity power level for CA

#### 7.3A.2.1	Reference sensitivity power level for Intra-band contiguous CA

For intra-band contiguous carrier aggregation, the throughput of each component carrier shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, and Table 7.3.2-3.

For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.2-3 and the downlink PCC carrier center frequency shall be configured closer to uplink operating band than any of the downlink SCC center frequency.

A UE supporting one uplink carrier and indicating a BandNR capability ue-PowerClass higher than the default power class for a band shall meet the requirements specified in sub-clause 7.3.2 with the uplink power limited to the nominal maximum output power for the default power class unless otherwise specified for the supported power class.

For aggregation of two or more downlink FDD carriers with two uplink carriers, the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3A.2.1-1 and the reference sensitivity power level increased by ΔRIBC. The requirements apply with all downlink carriers active. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signaling value NS_01 (Table 6.2.3.1-1) configured.

Table 7.3A.2.1-1: Intra-band contiguous CA uplink configuration for reference sensitivity

| CA configuration | SCS (PCC/SCC) (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation (LCRB) | UL SCC allocation (LCRB) | PCC ΔRIBC (dB) | SCC ΔRIBC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n5B | 15/15 | 10MHz + 10MHz | 10 (RBstart = 0) | 10 (RBstart = 42) | 30.8 | 26.1 | FDD |
| CA_n7B | 15/15 | 10MHz + 40MHz | 9 (RBstart = 26) | 36 (RBstart = 180) | 34 | 25 | FDD |
| NOTE 1: All combinations of channel bandwidths defined in Table 5.5A.1-1. NOTE 2: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 3: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2A.4. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. |  |  |  |  |  |  |  |

#### 7.3A.2.2	Reference sensitivity power level for Intra-band non-contiguous CA

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, throughput of each downlink component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) and parameters specified in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-2, and Table 7.3A.2.2-1 with the reference sensitivity power level increased by ΔRIBNC  given in Table 7.3A.2.2-1 for the SCC(s).

For aggregation of two or more downlink FDD carriers with one uplink carrier the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3A.2.2-1. The requirements apply with all downlink carriers active. Unless given by Table 7.3.2-4, the reference sensitivity requirements shall be verified with the network signaling value NS_01 (Table 6.2.3.1-1) configured.

A UE indicating a BandNR capability ue-PowerClass higher than the default power class for a band shall meet the requirements specified in Table 7.3A.2.2-1 with the uplink power limited to the nominal maximum output power for the default power class unless otherwise specified for the supported power class.

Table 7.3A.2.2-1: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity in FDD bands.

< text omitted >

#### 7.3A.2.3	Reference sensitivity power level for Inter-band CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one NR band the throughput shall be ≥ 95 % of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1 with parameters specified in  Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-1c, Table 7.3.2-1d, Table 7.3.2-2, Table 7.3.2-3, and in Table 7.3F.2-1, Table 7.3F.2-2, Table 7.3F.2-3 for inter-band CA with one shared spectrum channel access band, modified in accordance with clause 7.3A.3.2. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active. Exceptions to reference sensitivity are allowed in accordance with clause 7.3A.4, 7.3A.5 and 7.3A.6.

The exceptions specified in clauses 7.3A.4 and 7.3A.6 only apply for inter-band CA configurations with one component carrier per operating band and the uplink assigned to one NR band that include at least one combination of UL and DL configurations listed in any one of these two clauses for the band combination of the CA configuration. For these CA configurations the parameters in Table 7.3.2-1a, Table 7.3.2-1b, Table 7.3.2-1c, Table 7.3.2-1d, Table 7.3.2-2, Table 7.3.2-3, Table 7.3F.2-1, Table 7.3F.2-2 and Table 7.3F.2-3 are replaced by those of the specific test configurations in the respective clauses 7.3A.4 and 7.3A.6.

For the combination of intra-band and inter-band carrier aggregation, the intra-band CA relaxation, ΔRIBC and ΔRIBNC, are also applied according to the clause 7.3A.2.1 and 7.3A.2.2.

#### 7.3A.2.4	Void

< text omitted >

### 7.3A.4	Reference sensitivity exceptions due to UL harmonic interference for CA

Sensitivity degradation is allowed for different combinations of UL configurations and DL channel bandwidths if a band in frequency range 1 is impacted by UL harmonic interference from another band which belongs to NR band in frequency range 1 of the same downlink CA configuration. Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for either single band uplink or PC3 or PC2 CA are specified in Table 7.3A.4-1. For these exceptions, only the listed test points in Table 7.3A.4-1 are needed to be tested.

A UE indicating a BandNR capability ue-PowerClass higher than PC3 for the NR UL band shall meet the requirements for applicable UL/DL configurations as specified in Table 7.3A.4-1 and Table 7.3A.4-4 with the uplink power limited to the nominal maximum output power for PC3 unless otherwise specified for the supported power class in Table 7.3A.4-4a, Table 7.3A.4-4b or Table 7.3A.4-4d.

For inter-band CA configurations not including any of the UL/DL configurations in Table 7.3A.4-1, Table 7.3A.4-4, Table 7.3A.4-4a, Table 7.3A.4-4b or Table 7.3A.4-4d, the requirements in sub-clause 7.3A.2.3 apply.

Table 7.3A.4-1: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC3 aggressor NR UL band for NR DL CA FR1

< text omitted >

### 7.3A.6	Reference sensitivity exceptions due to cross band isolation for CA

Sensitivity degradation is allowed for a band if it is impacted by UL of another band part which belongs to NR band of the same NR CA configuration due to cross band isolation issues. The reference sensitivity degradation for the victim band due to cross band isolation is specified only for the specific uplink and downlink test points specified in Table 7.3A.6-1 for either PC3 and PC2 NR CA from a PC3 aggressor NR UL band, and for PC2 NR CA, in Table 7.3A.6-1afrom a PC2 aggressor NR UL band, and in Table 7.3A.6-1b from a PC1.5 aggressor NR single band uplink

In Tables 7.3A.6-1, 7.3A.6-1a and 7.3A.6-1b the following terminology is used to define the source of cross-band isolation interference:

- “ACLR1” indicates that the first adjacent channel of the aggressor UL band falls into the Rx channel of victim band.

- “ACLR2” indicates that the second adjacent channel of the aggressor UL band falls into the Rx channel of victim band.

- “>ACLR2” indicates that neither the first, nor the second adjacent channel of the aggressor UL band falls into the Rx channel of victim band.

A UE indicating a BandNR capability ue-PowerClass higher than PC3 for the NR UL band shall meet the requirements for applicable UL/DL configurations as specified in Table 7.3A.6-1 and Table 7.3A.6-1c with the uplink power limited to the nominal maximum output power for default power class for the bands unless otherwise specified for the supported power class in Table 7.3A.6-1b or Table 7.3A.6-1c.

For inter-band CA configurations not including the UL/DL configurations in Table 7.3A.6-1, Table 7.3A.6-1a, Table 7.3A.6-1b or Table 7.3A.6-1c, the requirements in sub-clause 7.3A.2.3 apply.

Table 7.3A.6-1: Reference sensitivity exceptions (MSD) and uplink/downlink configurations due to cross band isolation from a PC3 aggressor NR UL band for NR CA FR1

< text omitted >

### 7.3C.2	Reference sensitivity power level for SUL

For SUL operation, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in Table 7.3.2-1a, Table 7.3.2-1b and Table 7.3.2-2 shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3 or supplementary uplink transmission bandwidth less than or equal to that specified in Table 7.3C.2-1 with reference measurement channels as specified in Annexes A.2.2.2, A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1), unless sensitivity degradation is allowed in this clause of this specification. These exceptions also apply to any higher order CA or DC combination containing one of the exception combinations in this clause as subset.

For SUL operation with downlink CA, the reference receive sensitivity (REFSENS) requirement for downlink bands specified in clause 7.3A.2 shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3.2-3 or supplementary uplink transmission bandwidth less than or equal to that specified in Table 7.3C.2-1 with reference measurement channels as specified in Annexes A.2.2.2,  A.3.2, and A.3.3 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1), unless sensitivity degradation is allowed in this clause of this specification. These exceptions also apply to any higher order CA or DC combination containing one of the exception combinations in this clause as subset.

Table 7.3C.2-1: Supplementary uplink configuration for reference sensitivity

| NR Band / SCS of SUL band / Channel bandwidth of the DL band / NRB |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DL band | SUL band | SCS of SUL band (kHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| n1 | n80 | 15 | 160 | 160 | 160 | 160 | 160 | 160 | 160 |  |  |  |  |  |  |
| n1 | n841 | 15 | 25 | 50 | 75 | 100 | 128 | 128 | 128 | 128 |  |  |  |  |  |
| n3 | n801 | 15 | 25 | 50 | 50 | 50 | 50 | 50 | 50 |  |  |  |  |  |  |
| n24 | n99 | 15 | 25 | 50 |  |  |  |  |  |  |  |  |  |  |  |
| n28 | n831 | 15 | 25 | 25 | 25 | 25 |  | 25 |  |  |  |  |  |  |  |
| n41 | n80 | 15 |  | 160 | 160 | 160 |  | 160 | 160 | 160 | 160 |  | 160 | 160 | 160 |
| n41 | n81 | 15 |  | 100 | 100 | 100 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n41 | n83 | 15 |  | 100 | 100 | 100 |  | 100 | 100 | 100 | 100 |  | 100 | 100 | 100 |
|  |  | 30 |  | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n41 | n95 | 15 |  | 75 | 75 | 75 |  | 75 | 75 | 75 | 75 |  | 75 | 75 | 75 |
| n41 | n97 | 30 |  | 216 | 216 | 216 |  | 216 | 216 | 216 | 216 | 216 | 216 | 216 | 216 |
| n41 | n98 | 15 |  | 216 | 216 | 216 |  | 216 | 216 | 216 | 216 |  | 216 | 216 | 216 |
| n41 | n99 | 15 |  | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n48 | n99 | 15 |  | 50 | 50 | 50 |  | 50 | 50 | 50 | 50 |  | 50 | 50 | 50 |
| n77 | n80 | 15 |  | 160 | 160 | 160 |  |  | 160 | 160 | 160 |  | 160 | 160 | 160 |
| n77 | n84 | 15 |  | 100 | 100 | 100 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n77 | n99 | 15 |  | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 | 50 |
| n78 | n80 | 15 |  | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 | 160 |
| n78 | n81 | 15 |  | 100 | 100 | 100 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n78 | n82 | 15 |  | 100 | 100 | 100 |  |  | 100 | 100 | 100 |  | 100 | 100 | 100 |
| n78 | n83 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n78 | n84 | 15 |  | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 | 100 |
| n78 | n86 | 15 |  | 216 | 216 | 216 |  |  | 216 | 216 | 216 |  | 216 | 216 | 216 |
| n79 | n80 | 15 |  |  |  |  |  |  | 160 | 160 | 160 |  | 160 |  | 160 |
| n79 | n83 | 15 |  |  |  |  |  |  | 100 | 100 | 100 |  | 100 |  | 100 |
|  |  | 30 |  |  |  |  |  |  | 50 | 50 | 50 |  | 50 |  | 50 |
| n79 | n81 | 15 |  |  |  |  |  |  | 100 | 100 | 100 |  | 100 |  | 100 |
| n79 | n84 | 15 |  |  |  |  |  |  | 100 | 100 | 100 |  | 100 |  | 100 |
| n79 | n95 | 15 |  |  |  |  |  |  | 75 | 75 | 75 |  | 75 |  | 75 |
| n79 | n97 | 15 |  |  |  |  |  |  | 270 | 270 | 270 |  | 270 |  | 270 |
| n79 | n98 | 15 |  |  |  |  |  |  | 216 | 216 | 216 |  | 216 |  | 216 |
| NOTE 1: The Tx-Rx carrier center frequency separation between SUL band and DL band is the same as the Tx-Rx carrier center frequency separation of DL band specified in table 5.4.4-1 from TS 38.101-1. The channel bandwidth of SUL band is the same as DL band. This restriction of REFSENS configurations applies also for these carriers when applicable SUL configuration is part of a higher order configuration. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

For the UE that supports any of the SUL operation given in Table 7.3C.2-2, exceptions to the requirements specified in Table 7.3.2-1a and Table 7.3.2-1b are allowed for different combinations of UL configurations and DL channel bandwidths when the uplink is active in a lower frequency band and is within a specified frequency range such that transmitter harmonics fall within the downlink transmission bandwidth assigned in a higher band as noted in Table 7.3C.2-2. For these exceptions, only the listed test points in Table 7.3C.2-2 are needed to be tested.

For SUL configurations not including the UL/DL configurations in Table 7.3C.2-2, the requirements in sub-clause 7.3C.2-1 apply.

Table 7.3C.2-2: Reference sensitivity and uplink/downlink configurations for SUL operation (exceptions due to uplink harmonic issue)

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| n80 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n80 | n77 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n80 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n80 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n80 | n78 | 10 | 15 | 50 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n80 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n81 | n41 | 5 | 15 | 16 (RBstart=4) | 10 | 13 | NOTE 3 | UL3/DL1 direct-hit |
| n81 | n41 | 5 | 15 | 25 (RBstart=0) | 100 | 3.5 | NOTE 3 | UL3/DL1 direct-hit |
| n81 | n78 | 5 | 15 | 16 (RBstart=4) | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n81 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 1.4 | NOTE 4 | UL4/DL1 direct-hit |
| n81 | n79 | 5 | 15 | 25 (RBstart=0) | 40 | 6.8 | NOTE 5 | UL5/DL1 direct-hit |
| n81 | n79 | 5 | 15 | 25 (RBstart=0) | 100 | 4.4 | NOTE 5 | UL5/DL1 direct-hit |
| n82 | n78 | 5 | 15 | 16 (RBstart=4) | 10 | 10.8 | NOTE 4 | UL4/DL1 direct-hit |
| n82 | n78 | 5 | 15 | 20 (RBstart=2) | 100 | 1.0 | NOTE 4 | UL4/DL1 direct-hit |
| n83 | n78 | 5 | 15 | 10 (RBstart=8) | 10 | 10.4 | NOTE 5 | UL5/DL1 direct-hit |
| n83 | n78 | 5 | 15 | 25 (RBstart=0) | 100 | 0.7 | NOTE 5 | UL5/DL1 direct-hit |
| n84 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n84 | n77 | 10 | 15 | 100 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n84 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n86 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n86 | n78 | 10 | 15 | 100 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n86 | n78 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| n97 | n79 | 5 | 15 | 100 (RBstart=0) | 40 | 29.4 | NOTE 2 | UL2/DL1 direct-hit |
| n97 | n79 | 5 | 15 | 270 (RBstart=0) | 100 | 25.3 | NOTE 2 | UL2/DL1 direct-hit |
| n99 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 23.9 | NOTE 2 | UL2/DL1 direct-hit |
| n99 | n77 | 10 | 15 | 100 (RBstart=0) | 100 | 13.8 | NOTE 2 | UL2/DL1 direct-hit |
| n99 | n77 | 5 | 15 | 25 (RBstart=0) | 10 | 1.1 | NOTE 6 | UL2/DL1 near-miss |
| NOTE 1: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. NOTE 2: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 4: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: The requirements are only applicable to channel bandwidths no larger than 20 MHz and with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. |  |  |  |  |  |  |  |  |

Table 7.3C.2-3: Void

For the UE that supports any of the SUL operation given in Table 7.3C.2-4, reference sensitivity degradation is allowed for different combinations of UL configurations and DL channel bandwidths when a DL band is impacted by UL band due to cross band isolation issues. For these exceptions, only the listed test points in Table 7.3C.2-4 are needed to be tested.

For SUL configurations not including the UL/DL configurations in Table 7.3C.2-2, the requirements in sub-clause 7.3C.2-1 apply.

Table 7.3C.2-4: Reference sensitivity and uplink/downlink configurations for SUL operation (exceptions due to cross band isolation)

### 7.3C.3	ΔRIB,c for SUL

< end of changes >