3GPP TSG-WG4 Meeting #110	R4-2401864

# Athens, Greece, February, 26 - March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4185 | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to Case 1 core requirements for NR_MG_enh2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN WG4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Clean-up of Case 1 requirements for NR_MG_enh2 is needed, as the association of concurrent gap and NCSG to frequency layers is inconsistent and the list of gap combination configurations for collisions involving NCSGs is incomplete. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - In subclause 8.19.5, Pre-MG (de-)activation delay requirements in case of concurrent measurement gap, depending on configuredpriorities, are corrected. - In subclause 9.1.12.3, Pre-MG (de-)activation delay requirements in case of two Pre-MGs, are added. - In subclause 9.1.12.4, collision requirements for Pre-MG (de-) activation and concurrent MG are corrected and the case of Pre-MG deactivation with Pre-MG having lower priority is added. - Editorial errors are removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistent requirements in 8.19.5 and 9.1.2 for Case 1 requirements. Missed scenarios for concurrent MG + Pre-MG deactivation. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.19.5, 9.1.12.3, 9.1.12.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--- Start of change 1 ---

### 8.19.5	Activation/deactivation delay requirements for multiple pre-configured measurement gaps

The requirements in this clause apply to a UE configured with multiple pre-configured measurement gaps within concurrent gaps.

#### 8.19.5.1	Activation/deactivation delay requirements for non-overlapped activation/deactivation of multiple pre-configured measurement gaps

The requirements in this clause only apply when the activation/deactivation procedures of the individual pre-configured measurement gaps do not overlap in time.

When multiple pre-configured measurement gaps are activated/deactivated non-overlapped upon DCI/timer-based BWP switch, upon SCell activation/deactivation or upon RRC reconfiguration, for each individual pre-configured measurement gap, the requirements defined in clauses 8.19.2, 8.19.3 and 8.19.4 apply.

#### 8.19.5.2	Activation/deactivation delay requirements for fully overlapped activation/deactivation of multiple pre-configured measurement gaps

The requirements in this clause only apply when the activation/deactivation procedures of the individual pre-configured measurement gaps fully overlap in time.

Fully overlapped activation/deactivation of pre-configured measurement gaps can occur in the following cases:

-	Both pre-configured measurement gaps are triggered by the same event.

-	Two pre-configured measurement gaps are triggered by two events of the same type at the same time.

When multiple pre-configured measurement gaps are activated/deactivated simultaneously, the activation/deactivation delay equals the delay requirements defined at clause 8.19.2, 8.19.3 and 8.19.4 for DCI/timer-based BWP switch, SCell activation/deactivation or RRC reconfiguration, respectively, plus an additional 2ms post-processing time.

#### 8.19.5.3	Pre-configured measurement gap activation/deactivation delay when colliding with a measurement gap

When the pre-configured measurement gap activation procedure is overlapped with one measurement gap occasion and the pre-configured measurement gap has higher or lower priority, the pre-configured gap activation shall be applied 5ms after the ending point of the overlapping measurement gap.

When the pre-configured measurement gap deactivation procedure is overlapped with one measurement gap occasion, requirements specified in 8.19.2.1 in case of DCI/timer based BWP switch, or specified in 8.19.3 in case of SCell activation/deactivation, or specified in case of RRC reconfiguration, respectively, apply for the Pre-MG deactivation delay: if the pre-configured measurement gap has higher priority than the gap occasion, requirements for single Pre-MG, i.e. no overlap of Pre-MG deactivation procedure and (dropped) gap occasion apply, else if the measurement gap has higher priority, requirements for overlap of Pre-MG deactivation procedure and (non-dropped) gap occasion apply.

--- End of change 1 ---

--- Start of change 2 ---

#### 9.1.12.3	Collisions involving Pre-MG(s)

Collisions between a Pre-MG and a measurement gap may occur only when the Pre-MG is activated. No collisions can occur between a per-FR Pre-MG and a per-FR measurement gap when they are configured in different FRs.

Collisions between two Pre-MGs may occur only when both Pre-MGs are activated. No collisions can occur between per-FR Pre-MGs when they are configured in different FRs.

In case the (de-)activation procedure for a first Pre-MG has full overlap with the (de-)activation procedure for a second Pre-MG, the requirements for full overlap, as specified in clause 8.19.5.2 apply, regardless of configured priorities. No requirements are specified for the case of partial overlap.

The requirements for [concurrent measurement gaps with Pre-MG] apply provided that the two measurement gaps (at least one of the gaps is activated Pre-MG) colliding with each other are configured with different priorities.

#### 9.1.12.4	Collision between Pre-MG activation/deactivation and measurement gap

A measurement gap occasion and a Pre-MG activation/deactivation procedure collide when the ending point of the Pre-MG activation/deactivation procedure occurs anywhere within a time period starting 4ms before the starting point of the gap occasion and ending 4ms after the ending point of the gap occasion. The ending point of the Pre-MG activation/deactivation procedure when colliding with a measurement gap is defined in clause 8.19.5.3.

When a collision occurs between a measurement gap occasion and a Pre-MG activation procedure, and the Pre-MG is configured with higher or lower priority, the UE shall perform measurements during the measurement gap occasion and the activation of the Pre-MG is delayed until 5ms after the ending point of the measurement gap occasion.

When a collision occurs between a measurement gap occasion and a Pre-MG deactivation procedure, and the Pre-MG is configured with higher priority, the measurement gap occasion shall be dropped. The measurement gap occasion shall remain to be dropped until the ending point of the Pre-MG deactivation procedure.

When a collision occurs between a measurement gap occasion and a Pre-MG deactivation procedure, and the Pre-MG is configured with lower priority, the measurement gap occasion shall not be dropped, and the deactivation of the Pre-MG is delayed until 5ms after the ending point of the measurement gap occasion.

When the activated Pre-MG and measurement gap meets the collision rule defined in 9.1.8.3 and the Pre-MG is configured with lower priority, the UE shall perform measurements in the occasion of the measurement gap regardless of whether colliding with the Pre-MG activation procedure.

The UE is expected to transmit PUCCH/PUSCH/SRS or receive PDCCH/PDSCH/TRS/CSI-RS for CQI in the corresponding NR serving cells in the slots of the configured Pre-MG that are dropped according to the requirements in clause 9.1.8.4.

--- End of change 2 ---