# 3GPP TSG-RAN WG4 Meeting #110                                                              R4-2401876

# Athens, GR, 26 Feb – 01 Mar, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.876 | CR | 0002 | rev |  | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_ATG-Core) CR for TR 38.876 to update frequency error, configured transmitted power, SEM and transmit intermodulation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ATG-Core |  |  |  |  |  | Date: |  |  | 2024-2-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In last version, Frequency error requirement is based on the assumption that UE rely on the ATG BS location broadcasted by NTN SIB19. but in last meeting, RAN2 specify ATG specific SIB19, the requirements need to be updated accordingly. SEM requirement is missing in previous TR. There is no Tx intermodulation, so correspondingly section should be deleted. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Update frequency error requirements with ATG specific SIB signaling. - Add SEM requirements. - Delete Tx intermodulation section. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec is not complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.2.1, 7.1.2.4, 7.1.2.11 and 7.1.2.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <<Start of Change for TR 38.876>>

7.1.2.1	Frequency error

The doppler frequency for ATG UE can be determined by,

fD =  cosθ∗vc∗f_c

Where,

-	fD is Doppler frequency, f_c is the carrier frequency, which is set as 5GHz for n79.

-	v is the speed of the aircraft which is 1200km/h.

-	θ is the elevation angle between UE and gNB.

Since the potential maximum cell range is up to 200-300km [2] and the normal commercial airplane altitude is 10km, the elevation angle is only about 2~3 degrees when UE doing initial access at cell edge. Then the maximum DL doppler frequency is approaching 5.5kHz and the UL doppler frequency compared to the configured UL central frequency is approaching 5.5kHz if without any compensation which may cause not negligible impact for link performance and access successful rate. ATG UE needs to perform frequency compensation. The frequency accuracy requirement will be written assuming Doppler frequency pre-compensation.

The ATG BS coarse location information captured in SIB22 specified in TS 38.331 for ATG could be broadcasted for ATG UE to do frequency compensation if necessary. UE shall rely on the ATG BS location broadcasted by the IE atg-gNB-Location-r18 in ATG SIB 22 in 38.331 if pre-compensation is based on SIB22. Where, The atg-gNB-Location-r18 contains location information used as a reference location. The value of the field is same as Ellipsoid-Point defined in TS37.355 [49]. The first/leftmost bit of the first octet contains the most significant bit.

.

## <<Next Change for TR 38.876>>

7.1.2.4	Configured transmitted power

The UE is allowed to set its configured maximum output power PCMAX,f,c for carrier f of serving cell c in each slot. The configured maximum output power PCMAX,f,c is set within the following bounds:

PCMAX_L,f,c ≤  PCMAX,f,c  ≤  PCMAX_H,f,c with

PCMAX_L,f,c = MIN {PEMAX,c, PMaxOutputPower}

PCMAX_H,f,c = PEMAX,c

where

PEMAX,c is the value given by  ATG specific p-Max IE or the field additionalPmax of the NR-NS-PmaxList IE, whichever is applicable according to TS 38.331[7];

PMaxOutputPower is the maximum UE output power at maximum modulation order and full PRB configurations which is indicated by ATG UE capability;

## <<Next Change for TR 38.876>>

7.1.2.11	SEM requirements

ATG co-existence simulations results show the same ACLR value is applicable for all output power levels. The higher output power level the higher adjacent channel leakage level. So in theory, the higher output power the more relaxed SEM requirements. For ATG, 31dBm is used as the output power transition point to differentiate different SEM requirements. Here 31dBm refers to output power of PC1 since handhold UE spec specified the same SEM levels for all UEs up to PC1, i.e. 31dBm. If the actual transimission power of ATG UE is less than or equal to 31dBm, legacy  spectrum emission mask requirements for handhold UE applies; if the actual transmission power of ATG UE is larger than 31dBm, the requirements of spectrum emission mask shall be relaxed with scaling factor equal to (the actual transmission power minus 31) dB.

## <<Next Change for TR 38.876>>

## <<End Change for TR 38.876>>