3GPP TSG-WG4 Meeting #110 	R4-2402015

# Athens, Greece, February 26 - March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | DRAFT | rev | - | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR interruption requirements for interRAT measurements without gaps |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MG_enh2-Core |  |  |  |  |  | Date: |  |  | 2024-02-16 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | DRX interruption behavior needs to be defined for UEs supporting Rel 18 need for interruption |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Definition of interruption requirements when DRX is configured -Fixing IEs related to the need for interruption feature considering the latest RAN2 specification |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Gapless measurements for UEs that need interruption cannot be use in combination with DRX. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.8.2.22 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Change #1>

#### 7.8.2.22	Interruptions during inter-RAT NR measurements without measurement gap

For a UE indicating ‘no-gap-with-interruption’ with the capability [interRAT-NeedForIntrNR-r18] for inter-RAT NR measurement, the UE is allowed to cause interruptions on PCell or activated SCell(s) due to inter-RAT NR measurements with interruption ratio no more than the requirements specified below.

For the individual inter-RAT NR measurements performed on a frequency layer that corresponds to a configured measurement object i, Tcycle,i is the measurement cycle specified in Table 7.8.2.x-1, where Nfreq is defined in clause 8.1.2.4.29.1.1.

Table 8.2.2.2.X-1: Tcycle,i length for inter-RAT NR MOi

| DRX cycle | TCycle,I |
| --- | --- |
| No DRX | max (80ms, SMTC period) x Nfreq |
| [DRX cycle ≤ 320ms] | FFS |
| [DRX cycle>320ms] | FFS |

Editors’ note: Discussion is ongoing on cases where DRX is configured. Further update to this sub-clause subjects to the conclusions of the discussion.

When DRX is not configured, the total allowed maximum interruption ratio (D) on each of the active serving cells due to UE measurements without gap applied in this sub-clause is specified as

2LTcycle,i

D=i=1N2LTcycle,i

Where,

- N is the total number of NR MOs that UE indicates [nogap-interruption] for the band which the NR MO belongs to, and none of the SMTCs of the NR MOs are overlapped by the measurement gap, and

- L is the maximum interruption length for each interruption occasion, which shall not exceed 1 subframe.

The interruptions are allowed for all the serving cells in the same FR as NR SCell being measured if UE supports per-FR measurement gaps, and all the serving cells if UE does not support per-FR measurement gaps.

When DRX is configured, the UE is allowed to cause an average of 2 interruptions of length L due to the measurement on target carrier i per TCycle,i, provided that the interruption follows the conditions:

-If drx-onDurationTimer is larger than [10] ms,

-Interruptions are allowed, except for the first and last slot containing PDCCH in the DRX ON duration

-Otherwise,

-if the minimum DRX cycle is smaller than [40] ms:

-Interruptions are not allowed in the DRX ON duration.

-Otherwise, if the minimum DRX cycle is larger than or equal to [40] ms:

-Interruptions are not allowed in the DRX ON duration.

-If the DRX ON duration is extended by drx-InactivityTimer, and drx-InactivityTimer is larger than [4] ms:

-Interruptions are allowed, except for the last slot containing PDCCH while drx-InactivityTimer is running.

## <End of Change #1>