3GPP TSG-RAN WG4 Meeting #110	R4-2402090

Athens, Greece, February 19 – March 01, 2024

Source: 	Nokia, America Movil

Title: 					Simulation results for LTE CA_2C

Agenda Item: 			9.1.2.1

Document for:	Discussion

# 1	Introduction

This contribution presents simulation results for LTE CA_2C band for different bandwidth combinations taking into account an additional spectrum emission mask.

# 2	Simulation assumptions and scenario

- UL LTE channel for B2 at 1850-1910 MHz, SCS = 15 kHz

- I/Q image -28 dBc

- Carrier leakage -28 dBc

- CIM3 -60 dBc

- Power class PC3

- Only additional spectrum emission mask is considered; other gating factors (ACLR, spurious emission mask, EVM etc.) are excluded.

- The additional SEM is shown in Table 1. It is modified from the general LTE CA SEM by changing -10 dBm to -13 dBm in the ± 1-5 MHz segment. This modification is done following the additional SEM used for band 2 in the single-carrier case (NS_03).

Table 1: Spectrum emission limits for CA_2C

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB +100RB (24.95 MHz) | 50RB +75RB (24.75 MHz) | 50RB +100RB (29.9 MHz) | 75RB +75RB (30 MHz) | 75RB +100RB (34.85 MHz) | 100RB +100RB (39.8 MHz) | Measurement bandwidth |
| ± 0-1 | -22 | -22 | -22.5 | -22.5 | -23.5 | -24 | 30 kHz |
| ± 1-5 | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| ± 5-24.75 | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| ± 24.75-24.95 | -13 | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| ± 24.95-29.75 | -25 | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| ± 29.75-29.9 | -25 |  | -13 | -13 | -13 | -13 | 1 MHz |
| ± 29.9-29.95 | -25 |  | -25 | -13 | -13 | -13 | 1 MHz |
| ± 29.95-30 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| ± 30-34.85 |  |  | -25 | -25 | -13 | -13 | 1 MHz |
| ± 34.85-34.9 |  |  | -25 | -25 | -25 | -13 | 1 MHz |
| ± 34.9-35 |  |  |  | -25 | -25 | -13 | 1 MHz |
| ± 35-39.8 |  |  |  |  | -25 | -13 | 1 MHz |
| ± 39.8-39.85 |  |  |  |  | -25 | -25 | 1 MHz |
| ± 39.85-44.8 |  |  |  |  |  | -25 | 1 MHz |

Table 2: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA_2C

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations (NOTE 3) | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_2C |  | 5 | 20 |  |  |  | 40 | 0 |
|  |  | 10 | 15, 20 |  |  |  |  |  |
|  |  | 15 | 10, 15, 20 |  |  |  |  |  |
|  |  | 20 | 5, 10, 15, 20 |  |  |  |  |  |

# 3	Simulation results

A complete set of A-MPR results on LTE CA_2C for different bandwidth combinations listed in Table 2 are provided in Appendix. In particular, the different value of the required backoff compared to the MPR defined in [1, Table 6.2.3A-1] are presented in triangle plots.

In Figures 1 and 2, we specifically present results for QPSK and 16 QAM modulations, respectively, which show  allocations that may require A-MPR.

20MHz+10MHz

20MHz+10MHz

10MHz+20MHz

10MHz+20MHz

15MHz+10MHz

15MHz+10MHz

10MHz+15MHz

10MHz+15MHz

20MHz+20MHz

20MHz+20MHz

15MHz+15MHz

15MHz+15MHz

20MHz+15MHz

20MHz+15MHz

15MHz+20MHz

15MHz+20MHz

Figure 1: Examples of A-MPR needed for CA_2C w.r.t. QPSK modulation

20MHz+20MHz

20MHz+20MHz

15MHz+15MHz

15MHz+15MHz

Figure 2: Examples of A-MPR needed for CA_2C w.r.t. 16QAM modulation

Proposal 1: Consider defining A-MPR for CA_2C as shown in the table below.

Table: Contiguous Allocation A-MPR for CA_2C (power class 3)

| CA Bandwidth Class C | RBStart | LCRB [RBs] | RBStart +LCRB [RBs] | A-MPR for QPSK [dB] | A-MPR for 16 QAM |
| --- | --- | --- | --- | --- | --- |
| 50RB / 75 RB | 0 - 1 | 8 - 12 | N/A | ≤0.7dB | N/A |
|  | 112 -117 | N/A | >123 |  |  |
| 50RB / 100 RB | 0 | 10-12 | N/A | ≤0.5dB | N/A |
|  | 138-140 | N/A | 150 |  |  |
| 75 RB / 75 RB | 0 - 2 | 8-16 | N/A | ≤0.7dB | N/A |
|  | 132-142 | N/A | >147 |  |  |
|  | 0 | 9 | N/A | N/A | ≤0.5dB |
|  | 141 | N/A | 150 |  |  |
| 75 RB / 100 RB | 0 - 3 | 8-16 | N/A | ≤0.5dB | N/A |
|  | 156-167 | N/A | >171 |  |  |
| 100 RB / 100 RB | 0 - 3 | 8-18 | N/A | ≤0.7dB | N/A |
|  | 179-192 | N/A | >196 |  |  |
|  | 0 | 15 | N/A | N/A | ≤0.5dB |
|  | 185 | N/A | 200 |  |  |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation |  |  |  |  |  |

Given the small or not needed A-MPR for all allocations it can be further discussed if there is any need to define any A-MPR or it can be ignored.

Proposal 2: RAN4 to further discuss if A-MPR is needed or can be omitted.

# 4	Conclusions

We have provided simulation results for LTE CA_2C with different configurations of the bandwidth combinations. Based on the results we have made the following proposal.

Proposal 1: Consider defining A-MPR for CA_2C as shown in the table below.

Table: Contiguous Allocation A-MPR for CA_2C (power class 3)

| CA Bandwidth Class C | RBStart | LCRB [RBs] | RBStart +LCRB [RBs] | A-MPR for QPSK [dB] | A-MPR for 16 QAM |
| --- | --- | --- | --- | --- | --- |
| 50RB / 75 RB | 0 - 1 | 8 - 12 | N/A | ≤0.7dB | N/A |
|  | 112 -117 | N/A | >123 |  |  |
| 50RB / 100 RB | 0 | 10-12 | N/A | ≤0.5dB | N/A |
|  | 138-140 | N/A | 150 |  |  |
| 75 RB / 75 RB | 0 - 2 | 8-16 | N/A | ≤0.7dB | N/A |
|  | 132-142 | N/A | >147 |  |  |
|  | 0 | 9 | N/A | N/A | ≤0.5dB |
|  | 141 | N/A | 150 |  |  |
| 75 RB / 100 RB | 0 - 3 | 8-16 | N/A | ≤0.5dB | N/A |
|  | 156-167 | N/A | >171 |  |  |
| 100 RB / 100 RB | 0 - 3 | 8-18 | N/A | ≤0.7dB | N/A |
|  | 179-192 | N/A | >196 |  |  |
|  | 0 | 15 | N/A | N/A | ≤0.5dB |
|  | 185 | N/A | 200 |  |  |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation |  |  |  |  |  |

Given the small needed A-MPR for some allocations it can be further discussed if all the proposed A-MPR values are needed or some can be neglected.

Proposal 2: RAN4 to further discuss if A-MPR is needed or can be omitted.

# 5	Appendix: A-MPR triangle plots

# 6	Reference

[1] 3GPP TS 36.101 v18.4.0: “Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception (Release 18)”.