3GPP TSG RAN WG4 Meeting #110	R4-2402213

Athens, Greece, 26th Feb – 1st Mar, 2024

Source: 	Huawei, HiSilicon

Title:	PC2 A-MPR for band n7 NS_46

Agenda Item:	7.20.2

WI Code:	HPUE_NR_FR1_FDD_R18

Document for:	Approval

# 1	Introduction

As per the WF [1], the following issues are to be further discussed for NS_46 A-MPR:

- Further check the following issues:

| Channel Bandwidth | Potential Actions |
| --- | --- |
| 10MHz | - Further check if A-MPR is needed. |
| 15/20MHz | - If the exception case happens that more than 54 RB is scheduled after power class fallback to PC3, the requirement on the lower bound of the configured maximum output power PCMAX_L is unspecified, while the existing requirement for PCMAX_H is unchanged. - Further check whether to allow PC2 capable UEs to reduce their output power 23 dBm – max(MPR,4) dB in case PC2 needs to fall back to PC3 operation and transmit more than 54 RB, which is applicable for both SC-FDMA and CP-OFDM and all modulation orders. |
| 25MHz | - Further check the A-MPR; - Align the carrier centre frequency range for BW=25MHz with that for PC3 when defining the A-MPR requirements, namely 2534.5 ≤ FC ≤ 2557.5MHz. |
| 40MHz | - Consider to define a new region (e.g. 4.14<=RBend*12*SCS<5.04) with PC2 A-MPR = PC3 A5 + 3, instead of expanding the region for A4. |
| 10 to 25MHz plus 50MHz | - Modify the A-MPR regions as proposed in R4-2320669. |

- Taking into account the results in R4-2315375 (from RAN4-108bis), R4-2320669 and R4-2320652 when defining the PC2 A-MPR requirements for NS_46.

# 2	Discussion

Further to the simulations, we have evaluated the potential A-MPR using practical power amplifiers. Based on the measurement results, we conclude that the current PC3 A-MPR requirements are sufficient for PC2 operations.

Proposal 1: The current A-MPR requirements for NS_46 can be reused for PC2 UEs.

The proposal from [2] as shown below is used as the starting point for further discussion.

Table 1: A-MPR regions for NS_46 for PC2

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | 2505 ≤ FC ≤ 2565 | ≥ 7.92 | > 0 | A9 |
|  |  | ≥ 7.92 | < 5.4 | A10 |
| 15 MHz | 2507.5 ≤ FC ≤ 2562.5 | ≥ 11.16 | > 0 | A9 |
|  |  | ≥ 11.16 | < 5.4 | A10 |
| 20 MHz | 2510 ≤ FC ≤ 2560 | ≥12.6, < 15.3 | ≥ 10.8 | A7 |
|  |  | ≥ 15.3 | > 0 | A9 |
|  |  | ≥ 14.4 | < 5.4 | A10 |
| 25 MHz | 2512.5 ≤ FC ≤ 2557.5 | < 14.4 | >max (0, 12*SCS*RBend - 2.7) | A5 |
|  |  | ≥ 14.4, <18.9 | > 9.9 | A7 |
|  |  | ≥ 18.9 | ≥ 7.2 | A9 |
|  |  | ≥ 17.64 | < 7.2 | A10 |
| 30 MHz | 2515 ≤ FC ≤ 2555 | ≥0, <1.44 | >0 | A4 |
|  |  | ≥1.44, <13.5 | >max (0, 12*SCS*RBend -1.8) | A5 |
|  |  | ≥13.5, <19.8 | >11.52 | A6 |
|  |  | ≥19.8, <25.92 | >6.3 | A7 |
|  |  | ≥25.92 | >0 | A8 |
|  |  | > 20.7 , < 25.92 | ≤ 6.3 | A10 |
| 35 MHz | 2517.5 ≤ FC ≤ 2552.5 | ≥0, <3.42 | >0 | A4 |
|  |  | ≥3.42, <15.84 | >max (0, 12*SCS*RBend - 3.06) | A5 |
|  |  | ≥15.84, <22.68 | >12.6 | A6 |
|  |  | ≥22.68, <28.8 | >9.0 | A7 |
|  |  | ≥28.8 | >0 | A8 |
|  |  | > 24.3 , < 28.8 | ≤ 9.0 | A10 |
| 40 MHz | 2520 ≤ FC ≤ 2550 | ≥0, <4.5 | >0 | A4 |
|  |  | ≥4.5, <18 | >max (0, 12*SCS*RBend - 4.5) | A5 |
|  |  | ≥18, <25.74 | >13.5 | A6 |
|  |  | ≥25.74, <32.4 | >12.6 | A7 |
|  |  | ≥32.4 | >0 | A8 |
|  |  | > 27.9 , < 32.4 | ≤ 12.6 | A10 |
| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥0, <9 | >0 | A4 |
|  |  | ≥9, <21.6 | >max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥21.6, <31.5 | >18 | A6 |
|  |  | ≥31.5, <39.6 | >16.2 | A7 |
|  |  | ≥39.6 | >0 | A8 |
|  |  | > 33.84, < 39.6 | ≤ 19.8 | A10 |

Table 2: A-MPR for NS_46 for PC2

| Modulation/Waveform |  | A4 | A5 | A6 | A7 | A8 | A9 | A10 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | QPSK | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 16 QAM | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 64 QAM | 6.5 | 2.5 | 3.5 | 6 | 10 | 6.5 | 3.5 |
|  | 256 QAM | 6.5 |  |  | 6 | 10 | 6.5 |  |
| CP-OFDM | QPSK | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 16 QAM | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 64 QAM | 6.5 | 3.5 | 5.5 | 7 | 11 | 8 | 4.5 |
|  | 256 QAM | 6.5 |  |  | 7 | 11 | 8 |  |

As pointed out in our previous contribution [3], a number of issues are identified in this starting point proposal, which are duplicated below:

- Overlapping regions exist for BW=50/20/15/10MHz;

- New regions compared with PC3 A-MPR are proposed for BW=25/40MHz, which might result in excessive back-off than PC3;

- The range of carrier centre frequency could be excessive for BW=10 to 25MHz.

To address the above issues, we have made the following proposals.

For BW=40MHz, the PC2 A-MPR regions for A4 and A5 in the starting point proposal [1] are slightly different from those for PC3. More specifically, the region for A4 expands into the region for A5. Given that the PC2 A4 is more than 3dB larger than PC3 A5, a new region should be defined instead, with A-MPR value = PC3_A5+3.

Additionally, the carrier centre frequency range for BW=25MHz is set to 2512.5 ≤ FC ≤ 2557.5MHz, which is different from the range specified for PC3 A-MPR, namely 2534.5 ≤ FC ≤ 2557.5MHz.

Proposal 2: Regarding the candidate proposal from [2], consider the following corrections/modifications:

For BW=40MHz, consider to define a new region (e.g. 4.14<=RBend*12*SCS<4.5) with PC2 A-MPR = PC3 A5 + 3, instead of expanding the region for A4.

Align the carrier centre frequency ranges for BW=15/20/25MHz with those for PC3 when defining the A-MPR requirements.

For BW=10 to 25MHz plus 50MHz, correct the overlapping A-MPR regions as follows:

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | [TBC] ≤ FC ≤ 2565 | ≥ 7.92 | ≥ 5.4 | [A9] |
|  |  | ≥ 7.92 | < 5.4 | [A10] |
| 15 MHz | [2560.5] ≤ FC ≤ 2562.5 | ≥ 11.16 | ≥ 5.4 | [A9] |
|  |  | ≥ 11.16 | < 5.4 | [A10] |
| 20 MHz | [2552] ≤ FC ≤ 2560 | ≥12.6, < 15.3 | ≥ 10.8 | A7 |
|  |  | ≥ 15.3 | > 0 | [A9] |
|  |  | ≥ 14.4, < 15.3 | < 5.4 | [A10] |
| 25 MHz | 2534.5 ≤ FC ≤ 2557.5 | < 14.4 | >max (0, 12*SCS*RBend - 2.7) | A5 |
|  |  | ≥ 14.4, <18.9 | > 9.9 | A7 |
|  |  | ≥ 18.9 | ≥ 7.2 | A9 |
|  |  | ≥ 17.64 | < 7.2 | A10 |
| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥0, <9 | >0 | A4 |
|  |  | ≥9, <21.6 | >max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥21.6, <31.5 | >18 | A6 |
|  |  | ≥31.5, <39.6 | >16.2 | A7 |
|  |  | ≥39.6 | >0 | A8 |
|  |  | > 33.84, < 39.6 | ≤ 16.2 | A10 |
| 40 MHz | 2520 ≤ FC ≤ 2550 | ≥0, <4.14 | >0 | A4 |
|  |  | ≥4.14, <4.5 | >0 | 5/6.5dB |
|  |  | ≥4.5, <18 | >max (0, 12*SCS*RBend - 4.5) | A5 |
|  |  | ≥18, <25.74 | >13.5 | A6 |
|  |  | ≥25.74, <32.4 | >12.6 | A7 |
|  |  | ≥32.4 | >0 | A8 |
|  |  | > 27.9 , < 32.4 | ≤ 12.6 | A10 |

For BW=10MHz, no A-MPR is defined for PC3. For PC2, the candidate solution [2] proposed A9 and A10 for different regions, where A9=6.5/8dB. As a result, a PC2 UE may have lower maximum output power than a PC3 UE for the same RB allocation, which should not be allowed. Similar situation should be avoided for the case of BW=15/20MHz as well.

Proposal 3: No A-MPR is needed for BW=10MHz.

Proposal 4: Ensure that the PC2 A-MPR is not 3dB higher than that for PC3 for the same RB allocation, particularly for BW=15/20MHz.

It has been pointed out that:

- The PC3 A-MPR is only specified for BW>20MHz;

- In NS_46, RB restrictions (<=54 RB) are applied for BW=15/20MHz when the carrier centre frequency is within certain range.

##### 6.5.3.3.25	Requirement for network signalling value "NS_46"

When "NS_46" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.5.3.3.25-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.5.3.1-1 from the edge of the channel bandwidth.

Table 6.5.3.3.25-1: Additional requirements for “NS_46”

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 2570 | - | 2575 | +1.6 | 5 | 1, 2 |
| Frequency range | 2575 | - | 2595 | -15.5 | 5 | 1, 2 |
| Frequency range | 2595 | - | 2620 | -40 | 1 | 1 |
| NOTE 1: This requirement is applicable for all carriers confined in 2500-2570 MHz. Sepcial restrictions apply for channel bandwidths up to 20MHz: For carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB with the minimum supported SCS of 15KHz. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

Whether and how to lift the RB restriction for PC2 have been discussed, and a number of candidate solutions are proposed as follows:

Alt-1 from [4]: For simplicity and to avoid inconsistencies in case of power class fallback re-use the PC3 RB restrictions for PC2.

Alt-2 from [3]: For BW=15/20MHz, if the exception case happens that more than 54 RB is scheduled after power class fallback to PC3, the requirement on the lower bound of the configured maximum output power PCMAX_L is unspecified, while the existing requirement for PCMAX_H is unchanged.

Alt-3 from [2]: Allow PC2 capable UEs to reduce their output power 23 dBm – max(MPR,4) dB in case PC2 needs to fall back to PC3 operation and transmit more than 54 RB. Apply this for both SC-FDMA and CP-OFDM and all modulation orders.

It can be seen that Alt-1 is simple but sacrificing spectrum utilisation. Both Alt-2 and Alt-3 would lift the RB restriction for PC2 UEs, however, Alt-3 defines some additional requirement for the case of power class fallback, which is unnecessary in our view.

If the power class fallback (PC2 to PC3) is caused by the network (e.g. via p-Max signalling), the network shall not schedule more than 54 RBs (to be compatible with legacy PC3 UEs);

If the power class fallback is caused by duty-cycle exceedance, the UE itself would reduce the Tx power to be <=23dBm. The UE may need more back off to meet the additional emission requirements signalled in NS_46.

In any case, a PC2 UE can simply follow the PC2 A-MPR to determine the lower bound of MOP P_CMAX_L, while leave P_CMAX_H affected by P_EMAX and ∆PPowerClass as usual. The PC3 A-MPR if defined usually provides extra relaxation for P_CMAX_L. In practice, a PC2 UE usually does not store two sets of A-MPR tables.

Observation 1: There’s no fundamental difference between reducing the MOP by 3dB and power class fallback. In the event of power class fallback, a PC2 compliant UE can follow the PC2 power control requirements (in the absence of PC3 A-MPR for >54RBs) and reduce its Tx power to ensure its MOP is no more than 23dBm. The exact details can be left for UE implementation.

Proposal 5: Remove the RB restriction for PC2 UEs while keep it for PC3. And there’s no need to define additional requirement for the corner case of having power class fallback and more than 54 RBs simultaneously.

# 3	Conclusion

Based on further study, we have concluded with the following observations and proposals:

Proposal 1: The current A-MPR requirements for NS_46 can be reused for PC2 UEs.

Proposal 2: Regarding the candidate proposal from [2], consider the following corrections/modifications:

For BW=40MHz, consider to define a new region (e.g. 4.14<=RBend*12*SCS<4.5) with PC2 A-MPR = PC3 A5 + 3, instead of expanding the region for A4.

Align the carrier centre frequency ranges for BW=15/20/25MHz with those for PC3 when defining the A-MPR requirements.

For BW=10 to 25MHz plus 50MHz, correct the overlapping A-MPR regions as follows:

| Channel Bandwidth, MHz | Carrier Center Frequency, Fc, MHz | Regions |  | A-MPR |
| --- | --- | --- | --- | --- |
|  |  | RBend*12*SCS MHz | LCRB*12*SCS MHz |  |
| 10 MHz | [TBC] ≤ FC ≤ 2565 | ≥ 7.92 | ≥ 5.4 | [A9] |
|  |  | ≥ 7.92 | < 5.4 | [A10] |
| 15 MHz | [2560.5] ≤ FC ≤ 2562.5 | ≥ 11.16 | ≥ 5.4 | [A9] |
|  |  | ≥ 11.16 | < 5.4 | [A10] |
| 20 MHz | [2552] ≤ FC ≤ 2560 | ≥12.6, < 15.3 | ≥ 10.8 | A7 |
|  |  | ≥ 15.3 | > 0 | [A9] |
|  |  | ≥ 14.4, < 15.3 | < 5.4 | [A10] |
| 25 MHz | 2534.5 ≤ FC ≤ 2557.5 | < 14.4 | >max (0, 12*SCS*RBend - 2.7) | A5 |
|  |  | ≥ 14.4, <18.9 | > 9.9 | A7 |
|  |  | ≥ 18.9 | ≥ 7.2 | A9 |
|  |  | ≥ 17.64 | < 7.2 | A10 |
| 50 MHz | 2525 ≤ FC ≤ 2545 | ≥0, <9 | >0 | A4 |
|  |  | ≥9, <21.6 | >max (0, 12*SCS*RBend - 7.2) | A5 |
|  |  | ≥21.6, <31.5 | >18 | A6 |
|  |  | ≥31.5, <39.6 | >16.2 | A7 |
|  |  | ≥39.6 | >0 | A8 |
|  |  | > 33.84, < 39.6 | ≤ 16.2 | A10 |
| 40 MHz | 2520 ≤ FC ≤ 2550 | ≥0, <4.14 | >0 | A4 |
|  |  | ≥4.14, <4.5 | >0 | 5/6.5 |
|  |  | ≥4.5, <18 | >max (0, 12*SCS*RBend - 4.5) | A5 |
|  |  | ≥18, <25.74 | >13.5 | A6 |
|  |  | ≥25.74, <32.4 | >12.6 | A7 |
|  |  | ≥32.4 | >0 | A8 |
|  |  | > 27.9 , < 32.4 | ≤ 12.6 | A10 |

Proposal 3: No A-MPR is needed for BW=10MHz.

Proposal 4: Ensure that the PC2 A-MPR is not 3dB higher than that for PC3 for the same RB allocation, particularly for BW=15/20MHz.

Observation 1: There’s no fundamental difference between reducing the MOP by 3dB and power class fallback. In the event of power class fallback, a PC2 compliant UE can follow the PC2 power control requirements (in the absence of PC3 A-MPR for >54RBs) and reduce its Tx power to ensure its MOP is no more than 23dBm. The exact details can be left for UE implementation.

Proposal 5: Remove the RB restriction for PC2 UEs while keep it for PC3. And there’s no need to define additional requirement for the corner case of having power class fallback and more than 54 RBs simultaneously.

# References

- R4-2321715 WF on HPUE for FDD bands, China Unicom, RAN4#109

- R4-2320652 PC2 A-MPR for bands n7, n13 and n28, Qualcomm, RAN4#109

- R4-2320669 PC2 A-MPR for band n7 NS_46, Huawei, HiSilicon, RAN4#109

- R4-2315375 A-MPR discussion for FDD HPUE NS_46, Apple, RAN4#108bis

- R4-2316473 PC2 A-MPR for band n7 NS_46, Huawei, HiSilicon, RAN4#108bis