3GPP TSG-RAN WG4 Meeting #110	R4-2402313

Athens, Greece, 26 Feb – 01 Mar, 2024

Agenda item:	5.1.3

Source:            	Google Inc.

Title:                	Discussion on IE supportedBandwidthCombinationSetIntraENDC

Document for:	Approval

# Introduction

In RAN4#108, an issue of intra-band EN-DC BCS reporting for inter-band EN-DC band combination with two additional intra-band EN-DC components has been raised for discussion. In Rel-17, some inter-band EN-DC band combinations with two intra-band EN-DC components are introduced in the following table. It was identified in the paper [8] that this signaling IE might encounter an ambiguous issue if two intra-band EN-DC/NE-DC components, e.g., DC_3A_n3A and DC_41A_n41A, is within an inter-band EN-DC/NE-DC band combination, e.g., DC_3A-41A_n3A-n41A or DC_1A-3A-41A_n3A-n41A. Since the IE supportedBandwidthCombinationSetIntraENDC can only indicate one intra-band EN-DC component, there will be an ambiguous issue for the following example combinations that the network does not know which BCS of the intra-band EN-DC component the UE is referring to. In addition, the UE is also not able to indicate the BCSs of the two intra-band EN-DC components due to the current signalling restriction.

| EN-DC configuration | Uplink EN-DC configuration (NOTE1) |
| --- | --- |
| DC_3A-41A_n3A-n41A | DC_3A_n3A4 DC_3A_n41A DC_41A_n3A |
| DC_1A-3A-41A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A4 DC_3A_n41A DC_41A_n3A |

The WF [9] is approved in RAN4#108 to encourage companies to have further investigation on whether current signaling is sufficient to report the BCS(s) of the intra-band EN-DC components for a higher order inter-band EN-DC band combination with two intra-band EN-DC components.

# Way Forward - Companies are encouraged to check whether the current RAN2 signaling exists an issue on reporting the BCS(s) of the intra-band EN-DC components for a higher order inter-band EN-DC band combination with two intra-band EN-DC components.

In RAN4#109, one discussion paper [10] and one LS [11] are submitted for the further discussion. The LS to RAN2 is almost agreed but one issue is raised to further check another IE intrabandENDC-support impact which is also related to the signalling for the band combination including intra-band EN-DC components. For this reason, we would like to split the discussion into another our paper [12] about the IE intrabandENDC-support for inter-band EN-DC band combinations with two additional intra-band EN-DC components. In this paper, we would like to share our view on the IE supportedBandwidthCombinationSetIntraENDC for inter-band EN-DC band combinations with two additional intra-band EN-DC components in the following.

# Discussion

From RRC signaling perspective, the UE is allowed to indicate E-UTRA bandwidth combination sets (LTE CA BCS), NR bandwidth combination sets (NR CA BCS) for a supported inter-band EN-DC/NE-DC band combination. If the inter-band EN-DC/NE-DC band combination includes additional intra-band EN-DC/NE-DC component, RAN2 has introduced a signaling IE, supportedBandwidthCombinationSetIntraENDC, to indicate the EN-DC/EN-DC bandwidth combination sets (EN-DC/NE-DC BCS) for the intra-band EN-DC/NE-DC component. Hence, the UE is allowed to additionally signal EN-DC/NE-DC BCS for this kind of band combination. According to TS38.101-3 sub-clause 4.2, the UE should support a product set or a common set among these three bandwidth combination sets which it signals to support.

[38.101-3][Sub-clause 4.2][V17.11.1]

A terminal which supports an EN-DC or NE-DC configuration shall support:

If any subsets of the EN-DC or NE-DC configuration do not specify its own bandwidth combination sets in 5.3B, then the terminal shall support the same E-UTRA bandwidth combination sets it signals the support for in E-UTRA CA configuration part of E-UTRA – NR DC and shall support the same NR bandwidth combination sets it signals the support for in NR CA configuration part of E-UTRA – NR DC.

Else if one of the subsets of the EN-DC or NE-DC configuration specify its own bandwidth combination sets in 5.3B, then the terminal shall support a product set of channel bandwidth for each band specified by E-UTRA bandwidth combination sets, NR bandwidth combination sets, and EN-DC or NE-DC bandwidth combination sets it singnals the support. A terminal which supports an inter-band EN-DC or NE-DC configuration with a certain UL configuration shall support the all lower order DL configurations of the lower order EN-DC or NE-DC combinations, which have this certain UL configuration and the fallbacks of this UL configuration.

In RAN4#98-e, EN-DC/NE-DC BCS reporting for inter-band EN-DC band combination with one additional intra-band EN-DC component has been further discussed. The RAN4 LS to RAN2 [1][2], RAN4 CRs [3][4][5] and RAN2 CRs [6][7] are agreed to clarify the definition of IE supportedBandwidthCombinationSetIntraENDC. The major clarification in RAN4 CRs is that if this IE is not indicated, the network assumes the UE supports BCS0 for the intra-band EN-DC/NE-DC as defined in the TS 38.101-3. As for RAN2 CRs, the major clarification is that the reporting condition is mandatory if the UL configuration is intra-band EN-DC configuration with a common band on the LTE and NR side, otherwise is optional.

For example, referring to the following IE supportedBandwidthCombinationSetIntraENDC definition, if the UE would like to indicate EN-DC/NE-DC BCS for inter-band EN-DC/NE-DC band combination with additional intra-band EN-DC/NE-DC component, it is observed that the EN-DC/NE-DC BCS reporting criteria can be divided into two categories. The first category is that the UE should be mandatory to report EN-DC BCS if both UL and DL configuration for the intra-band EN-DC component, e.g., DL DC_3A_n3A-n77A with UL DC_3A_n3A, is supported within inter-band EN-DC band combination. The other category is that the UE should be optional to report EN-DC BCS if both UL and DL configuration for the intra-band EN-DC component is not supported within inter-band EN-DC band combination, e.g., DL DC_3A_n3A-n77A with UL DC_3A_n77A, where the network would assume the UE supports BCS0 if the IE is not indicated. In addition, it is worth to note that this IE can indicate only one intra-band EN-DC/NE-DC component of an inter-band EN-DC/NE-DC band combination.

[38.306][V17.7.0]

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| supportedBandwidthCombinationSetIntraENDC Defines the supported bandwidth combination set for a band combination that allows configuration of at least one EUTRA serving cell and at least one NR serving cell in the same band, as defined in the TS 38.101-3 [4], table 5.3B.1.2-1 and table 5.3B.1.3-1. - For intra-band (NG)EN-DC with additional inter-band CA component(s) of LTE and/or NR, the field defines the bandwidth combinations for the intra-band (NG)EN-DC component. - For intra-band NE-DC with additional inter-band CA component(s) of LTE and/or NR, the field defines the bandwidth combinations for the intra-band NE-DC component. Field encoded as a bit map, where bit N is set to "1" if UE support Bandwidth Combination Set N for this band combination as defined in the TS 38.101-3 [4]. The leading / leftmost bit (bit 0) corresponds to the Bandwidth Combination Set 0, the next bit corresponds to the Bandwidth Combination Set 1 and so on. - It is mandatory if the band combination is an intra-band (NG)EN-DC/NE-DC combination supporting both UL and DL intra-band (NG)EN-DC/NE-DC parts with additional inter-band NR/LTE CA component. - It is optional if the band combination is an intra-band (NG)EN-DC/NE- DC combination without supporting UL in both the bands of the intra- band (NG)EN-DC/NE-DC UL part. If not included, the network assumes the UE supports BCS0 as defined in TS 38.101-3 [4], table 5.3B.1.2-1 and table 5.3B.1.3-1 for the intra-band (NG)EN-DC/NE-DC. | BC | CY | N/A | N/A |

However, in Rel-17, as shown in the above table, it is observed in TS 38.101-3 that DC_3A-41A_n3A-n41A is introduced in sub-clause 5.5B.4.3 and DC_1A-3A-41A_n3A-n41A is introduced in sub-clause 5.5B.4.4, where these two band combinations are inter-band EN-DC band combinations with two additional intra-band EN-DC components. In our understanding, the current definition of EN-DC/NE-DC BCS reporting is only applicable to inter-band EN-DC band combination with only one intra-band EN-DC component. Hence, the rule for EN-DC/NE-DC BCS reporting becomes ambiguous. The UE does not know how to indicate this IE for inter-band EN-DC band combination, e.g., DC_3A-41A_n3A-n41A or DC_1A-3A-41A_n3A-n41A, with two additional intra-band EN-DC components, e.g., DC_3A_n3A and DC_41A_n41A. The network also cannot differentiate which one of intra-band EN-DC component is associated with the reported EN-DC/NE-DC BCS.

Take the inter-band EN-DC band combination, DC_1A-3A-41A_n3A-n41A, with two additional intra-band EN-DC components, e.g., the first intra-band EN-DC component DC_3A_n3A and the second intra-band EN-DC component DC_41A_n41A, as an example. According to BCS definition in RAN4 specification, the UE can indicate “BCS0” for LTE CA component CA_1A-3A-41A and indicate “BCS0/BCS1/BCS2” for NR CA component CA_n3A-n41A. For EN-DC BCS part, the UE can indicate “BCS0/BCS1” for the first intra-band EN-DC components DC_3A_n3A and can indicate “BCS0/BCS1/BCS2” for the second intra-band EN-DC component DC_41A_n41A. However, only one BCS IE can be used to indicate these two intra-band EN-DC components. Depending on different implementations, the network may have different interpretations for this IE. In one implementation, if the UE does not report EN-DC BCS, it would be possible for the network to assume the UE supports BCS0 for DC_3A_n3A and BCS0 for DC_41A_n41A. However, in other implementations, if the UE reports BCS1 in EN-DC BCS, the network cannot know whether BCS1 is associated with DC_3A_n3A or DC_41A_n41A.

[36.101][V17.12.0]

| E-UTRA CA Configuration | Uplink CA configurations (NOTE 5) | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A-41A9 | CA_1A-3A | 1 |  |  | Yes | Yes | Yes | Yes | 60 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 41 |  |  | Yes | Yes | Yes | Yes |  |  |

[38.101-1][V17.12.0]

| NR CA configuration | Uplink CA configuration or single uplink carrier10 | NR Band | Channel bandwidth (MHz) (NOTE 3) | Bandwidth combination set |
| --- | --- | --- | --- | --- |
| CA_n3A-n41A | n418 CA_n3A-n41A8 | n3 | 5, 10, 15, 20, 25, 30 | 0 |
|  |  | n41 | 10, 15, 20, 40, 50, 60, 80, 90, 100 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30 | 1 |
|  |  | n41 | 10, 15, 20, 40, 50, 60 |  |
|  |  | n3 | 5, 10, 15, 20, 25, 30, 40 | 2 |
|  |  | n41 | 10, 15, 20, 30, 40, 50, 60, 80, 90, 100 |  |

[38.101-3][V17.12.0]

| Downlink EN-DC configuration | Uplink EN-DC configurations | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth (MHz) | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Channel bandwidths for E-UTRA carrier (MHz) | Channel bandwidths for NR carrier (MHz) | Channel bandwidths for E-UTRA carrier (MHz) |  |  |
| DC_3A_n3A | DC_3A_n3A |  | 5, 10, 15, 20, 25, 30 | 5, 10, 15, 20 | 50 | 0 |
|  |  |  | 5, 10, 15, 20, 25, 30 | 5, 10, 15, 20 | 50 | 1 |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20, 25, 30 |  |  |  |
| DC_41A_n41A | DC_41A_n41A | 20 | 40, 60, 80,100 |  | 120 | 0 |
|  |  |  | 40, 60, 80,100 | 20 |  |  |
|  |  | 20 | 40, 50, 60, 80,100 |  | 120 | 1 |
|  |  |  | 40, 50, 60, 80,100 | 20 |  |  |
|  |  | 20 | 10, 20, 30, 40, 50, 60, 80,100 |  | 120 | 2 |
|  |  |  | 10, 20, 30, 40, 50, 60, 80,100 | 20 |  |  |
|  |  | 10 | 20, 30, 40, 50, 60, 80,100 |  |  |  |
|  |  |  | 20, 30, 40, 50, 60, 80,100 | 10 |  |  |

In the following table, we summarize the possible DL/UL configurations and how the IE would be indicated by using the example band combination DC_1A-3A-41A_n3A-n41A with the assumption of

- reporting BCS0 for LTE CA

- reporting BCS0 for NR CA,

- reporting BCS0 for the first intra-band EN-DC component DC_3A_n3A

- reporting BCS2 for the second intra-band EN-DC component DC_41A_n41A.

In Case#1/Case#2/Case#4/Case#5, the UL configurations satisfy the optional reporting condition for EN-DC BCS reporting rule, which means that the UE does not need to report BCS0 for EN-DC and the network would assume the UE support BCS0. In one implementation, if the UE does not report EN-DC BCS, the network may interpret BCS0 for both intra-band EN-DC DC_3A_n3A and DC_41A_n41A, where the network may use BCS0 for DC_41A_n41A as the reference BW information to configure the UE instead of using BCS2. In another implementation, if the UE reports BCS2 for EN-DC BCS, the network may interpret BCS2 for both intra-band EN-DC DC_3A_n3A and DC_41A_n41A. However, the network cannot acquire BCS2 for DC_3A_n3A due to no BCS2 definition for DC_3A_n3A in the specification. Hence, for both implementations, the network cannot know the exact supported EN-DC BCS for these two additional intra-band EN-DC components by indicating only one IE supportedBandwidthCombinationSetIntraENDC, where the UE can actually support EN-DC BCS0 for DC_3A_n3A and EN-DC BCS2 for DC_41A_n41A.

In Case#3, the UL configuration satisfies the mandatory reporting condition for EN-DC BCS reporting rule, and the UE should indicate EN-DC BCS to the network. In one implementation, if the UE reports BCS0 in EN-DC BCS, the network may interpret BCS0 for both intra-band EN-DC DC_3A_n3A and DC_41A_n41A, where the network may use BCS0 for DC_41A_n41A as the reference BW information to configure the UE instead of using BCS2. In another implementation, if the UE reports BCS2 in EN-DC BCS, the network may interpret BCS2 for both intra-band EN-DC DC_3A_n3A and DC_41A_n41A. However, the network cannot acquire BCS2 for DC_3A_n3A due to no BCS2 definition for DC_3A_n3A. Hence, for both implementations, the network still cannot know the exact supported EN-DC BCS for these two additional intra-band EN-DC components by indicating only one IE supportedBandwidthCombinationSetIntraENDC, where the UE can actually support EN-DC BCS0 for DC_3A_n3A and EN-DC BCS2 for DC_41A_n41A.

Moreover, from n41 BW configuration perspective, the example band combination DC_1A-3A-41A_n3A-n41A can be referred to the following BCS

- {40,60,80,100} (MHz) for DC_41A_n41A_BCS0,

- {10,20,30,40,50,60,80,100} (MHz) for DC_41A_n41A_BCS2

- {10,15,20,40,50,60,80,90,100} (MHz) for CA_n3A-n41_BCS0.

In one implementation for EN-DC BCS reporting, if the network interprets DC_41A_n41A_BCS0 as the reference BW information, the UE should support n41 BW configuration {40, 60, 80,100} (MHz) due to the product set between DC_41A_n41A_BCS0 and CA_n3A-n41_BCS0.

In another implementation for EN-DC BCS reporting, if the network interprets DC_41A_n41A_BCS2 as the reference BW information, the UE should support n41 BW configuration {10, 20, 40, 50, 60, 80, 100} (MHz) due to the product set between DC_41A_n41A_BCS2 and CA_n3A-n41_BCS0.

Therefore, if the network cannot exactly understand that DC_41A_n41A_BCS2 is supported by the UE, the network may use DC_41A_n41A_BCS0 to configure n41 BW={40, 60, 80,100} instead of using DC_41A_n41A_BCS0 to configure n41 BW={10,20,40,50,60,80,100}, where n41 BW={10, 20, 50} cannot be configured if the network interprets the UE supports DC_41A_n41A_BCS0.

Furthermore, from n3 BW configuration perspective, n3 BW configuration are all {5, 10, 15, 20, 25, 30} in DC_3A_n3A_BCS0, DC_3A_n3A_BCS1 and CA_n3A-n41_BCS0. However, although the n3 BW configuration is the same, the network still cannot configure n3 BW configuration with LTE B3 carrier frequency lower than NR n3 carrier frequency if the network interprets the UE supports DC_3A_n3A_BCS0 instead of DC_3A_n3A_BCS1.

| Example Band Combination: DC_1A-3A-41A_n3A-n41A |  |  |  |
| --- | --- | --- | --- |
| BCS categories |  | Supported BCS |  |
| LTE CA part BCS |  | CA_1A-3A-41A_BCS0 |  |
| NR CA part BCS |  | CA_n3A-n41A_BCS0 |  |
| Intra-band EN-DC part BCS |  | DC_3A_n3A_BCS0 |  |
| Intra-band EN-DC part BCS |  | DC_41A_n41A_BCS2 |  |
| Case | DL Configuration | UL Configuration | supportedBandwidthCombinationSetIntraENDC |
| Case#1 | DC_1A-3A-41A_n3A-n41A | DC_1A_n3A | Not report BCS0 for DC_3A_n3A? or Report BCS2 for DC_41A_n41A? |
| Case#2 | DC_1A-3A-41A_n3A-n41A | DC_1A_n41A | Not report BCS0 for DC_3A_n3A? or Report BCS2 for DC_41A_n41A? |
| Case#3 | DC_1A-3A-41A_n3A-n41A | DC_3A_n3A | Report BCS0 for DC_3A_n3A? or Report BCS2 for DC_41A_n41A? |
| Case#4 | DC_1A-3A-41A_n3A-n41A | DC_3A_n41A | Not report BCS0 for DC_3A_n3A? or Report BCS2 for DC_41A_n41A? |
| Case#5 | DC_1A-3A-41A_n3A-n41A | DC_41A_n3A | Not report BCS0 for DC_3A_n3A? or Report BCS2 for DC_41A_n41A? |

Hence, considering the above all discussion, it is inferred that the UE cannot indicate the BCSs of the two intra-band EN-DC components due to the current signalling IE restriction. Even if the IE is indicated by the first intra-band EN-DC component or the second intra-band EN-DC component, the network still does not know which BCS is associated with the first or second intra-band EN-DC component. Furthermore, some BW configuration for the inter-band EN-DC combination may be limited due to not enough EN-DC BCS information for the first and second intra-band EN-DC components. In our view, in order to solve the ambiguous issue by indicating EN-DC BCS reporting with only one IE for inter-band EN-DC band combination with two additional intra-band EN-DC components, it is proposed that RAN4 confirms the ambiguous signaling issue of IE supportedBandwidthCombinationSetIntraENDC to separately indicate the first and the second intra-band EN-DC BCS for the inter-band EN-DC band combinations, e.g., DC_3A-41A_n3A-n41A or DC_1A-3A-41A_n3A-n41A, with two additional intra-band EN-DC components, e.g., the first intra-band EN-DC component DC_3A_n3A and the second intra-band EN-DC component DC_41A_n41A, and RAN4 send LS to RAN2 to find a solution for the ambiguous signaling issue.

Proposal 1: RAN4 confirms the ambiguous signaling issue of IE supportedBandwidthCombinationSetIntraENDC to separately indicate the first and the second intra-band EN-DC BCS for the inter-band EN-DC band combinations, e.g., DC_3A-41A_n3A-n41A or DC_1A-3A-41A_n3A-n41A, with two additional intra-band EN-DC components, e.g., the first intra-band EN-DC component DC_3A_n3A and the second intra-band EN-DC component DC_41A_n41A.

Proposal 2: RAN4 send LS in the Annex to RAN2 for the ambiguous signaling issue.

# Conclusion

The proposals in this contribution are summarized in the following.

Proposal 1: RAN4 confirms the ambiguous signaling issue of IE supportedBandwidthCombinationSetIntraENDC to separately indicate the first and the second intra-band EN-DC BCS for the inter-band EN-DC band combinations, e.g., DC_3A-41A_n3A-n41A or DC_1A-3A-41A_n3A-n41A, with two additional intra-band EN-DC components, e.g., the first intra-band EN-DC component DC_3A_n3A and the second intra-band EN-DC component DC_41A_n41A.

Proposal 2: RAN4 send LS in the Annex to RAN2 for the ambiguous signaling issue.

# Reference

- R4-2102149, “Reply LS to RP-202935 on BCS reporting and support for intra-band EN-DC band combinations”, RAN4#98-e, T-Mobile USA

- R4-2103401, “Second Reply LS to RP-202935 = R4-2100025 on BCS reporting and support for intra-band EN-DC band combinations”, RAN4#98-e, T-Mobile USA

- R4-2103402, “CR for 38.101-3: clarification of intra-band EN-DC BCS applicability”, RAN4#98-e, T-Mobile USA

- R4-2103403, “CR for 38.101-3: clarification of intra-band EN-DC BCS applicability”, RAN4#98-e, T-Mobile USA

- R4-2103404, “CR for 38.101-3: clarification of intra-band EN-DC BCS applicability”, RAN4#98-e, T-Mobile USA

- R2-2102216, “Clarification on the supportedBandwidthCombinationSetIntraENDC capability”, RAN2#113-e, Nokia, Nokia Shanghai Bell

- R2-2102217, “Clarification on the supportedBandwidthCombinationSetIntraENDC capability”, RAN2#113-e, Nokia, Nokia Shanghai Bell

- R4-2313499, “Discussion on IE supportedBandwidthCombinationSetIntraENDC”, RAN4#108, Google

- R4-2314875, “WF on IE supportedBandwidthCombinationSetIntraENDC”, RAN4#108, Google

- R4-2320181, “Discussion on IE supportedBandwidthCombinationSetIntraENDC”, RAN4#109, Google

- R4-2320241, “LS on IE supportedBandwidthCombinationSetIntraENDC”, RAN4#109, Google

- R4-2402318, “Discussion on IE intraBandENDC-Support”, RAN4#110, Google

# Annex

3GPP TSG-RAN WG4 Meeting #110	R4-23XXXXX

Athens, Greece, 26 Feb – 01 Mar, 2024

Title:	[Draft] LS on IE supportedBandwidthCombinationSetIntraENDC

Response to:

Release:	Rel-17

Work Item:	TEI17

Source:	RAN4

To:	RAN2

Cc:

Contact Person:

Name: Clement Huang

E-mail Address: clementhuang@google.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1. Overall Description:

RAN4 confirms that the current definition of IE supportedBandwidthCombinationSetIntraENDC is ambiguous to separately indicate the first and the second intra-band EN-DC BCS for the inter-band EN-DC band combinations, e.g., DC_3A-41A_n3A-n41A or DC_1A-3A-41A_n3A-n41A, with two additional intra-band EN-DC components, e.g., the first intra-band EN-DC component DC_3A_n3A and the second intra-band EN-DC component DC_41A_n41A, where these band combinations in the following table are introduced from Rel-17.

| EN-DC configuration | Uplink EN-DC configuration (NOTE1) |
| --- | --- |
| DC_3A-41A_n3A-n41A | DC_3A_n3A4 DC_3A_n41A DC_41A_n3A |
| DC_1A-3A-41A_n3A-n41A | DC_1A_n3A DC_1A_n41A DC_3A_n3A4 DC_3A_n41A DC_41A_n3A |

RAN4 would like to request RAN2 to check how to solve the ambiguous signaling issue for the inter-band EN-DC band combinations with two additional intra-band EN-DC components.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully asks RAN2 to take the above decisions into account to update RAN2 specifications.

3. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #110bis             15th April – 19th April 2024              Changsha, CN

TSG-RAN WG4 Meeting #111                  20th May – 24th May 2024               Fukuoka City, JP