3GPP TSG-RAN-WG4 Meeting #110	R4-2402410

# Athens, Greece, February 26 – March 1, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-2 | CR | 0732 | rev |  | Current version: | 18.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Beam correspondence requirement for power classes other than PC3 in initial access and RRC_INACTIVE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_req_Ph3-Core |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | Cat F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Beam correspondence requirement for initial access and RRC_INACTIVE is not introduced for PC5/6/7 yet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PC7 beam correspondence requirement in initial access and RRC_INACTIVE is introduced only for EIRP spherical coverage with 2 dB relaxation from RRC_CONNECTED in the same way as PC3. PC5/6 beam correspondence requirement in initial access and RRC_INACTIVE is introduced only for EIRP spherical coverage without additional relaxation from RRC_CONNECTED. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PC5/6/7 BC requirements remain unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.6.6, 6.6.7, 6.6.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-2 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### <Start of Changes>

#### 6.2.1.5	UE maximum output power for power class 5

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). In case of initial access and RRC_INACTIVE, the cumulative period of measurement shall equal or exceed 1ms. The minimum output power values for EIRP are found in Table 6.2.1.5-1. The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.5-1: UE minimum peak EIRP for power class 5

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 30 |
| n258 | 30.4 |
| n259 | 27.7 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance NOTE 2: Minimum peak EIRP does not apply to initial access and RRC_INACTIVE. |  |

The maximum output power values for TRP and EIRP are found in Table 6.2.1.5-2 below. The maximum allowed EIRP is derived from regulatory requirements. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.5-2: UE maximum output power limits for power class 5

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n259 | 23 | 43 |

The minimum EIRP at the 85th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2.1.5-3 below. The requirement is verified with the test metric of EIRP (Link=Spherical coverage grid, Meas=Link angle).

Table 6.2.1.4-3: UE spherical coverage for power class 5

| Operating band | Min EIRP at 85 %-tile CDF (dBm) |
| --- | --- |
| n257 | 22 |
| n258 | 22.4 |
| n259 | 19.7 |
| NOTE 1: Minimum EIRP at 85 %-tile CDF is defined as the lower limit without tolerance NOTE 2: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1. |  |

For the UEs that support multiple FR2 bands, minimum requirement for peak EIRP and EIRP spherical coverage in Tables 6.2.1.5-1 and 6.2.1.5-3 shall be decreased per band, respectively, by the peak EIRP relaxation parameter MBP,n and EIRP spherical coverage relaxation parameter MBS,n, as defined in Table 6.2.1.5-4..

Table 6.2.1.5-4: UE multi-band relaxation factors for power class 5

| Band | MBP,n (dB) | MBS,n (dB) |
| --- | --- | --- |
| n257 | 0.7 | 0.7 |
| n258 | 0.7 | 0.7 |
| n259 | 0.5 | 0,5 |

#### 6.2.1.6	UE maximum output power for power class 6

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). In case of initial access and RRC_INACTIVE, the cumulative period of measurement shall equal or exceed 1ms. The minimum output power values for EIRP are found in Table 6.2.1.6-1. The requirement is verified with the test metric of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.6-1: UE minimum peak EIRP for power class 6

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 30 |
| n258 | 30.4 |
| n261 | 30 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance NOTE 2: Minimum peak EIRP does not apply to initial access and RRC_INACTIVE. |  |

The maximum output power values for TRP and EIRP are found in Table 6.2.1.6-2 below. The maximum allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.6-2: UE maximum output power limits for power class 6

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n261 | 23 | 43 |

The minimum EIRP measured over the spherical coverage evaluation areas specified below is defined as the spherical coverage requirement and is found in Table 6.2.1.6-3 below. UE spherical coverage evaluation areas are found in Table 6.2.1.6-3a below, by consisting of Area-1 and Area-2, in the reference coordinate system in Annex J.1. The requirement is verified with the test metric of EIRP (Link= Spherical coverage grid, Meas=Link angle).

Table 6.2.1.6-3: UE spherical coverage for power class 6

| Operating band | Min EIRP over UE spherical coverage evaluation areas (dBm) |
| --- | --- |
| n257 | 20 |
| n258 | 20.4 |
| n261 | 20 |
| NOTE 1: Minimum EIRP over UE spherical coverage evaluation areas is defined as the lower limit without tolerance NOTE 2: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1. NOTE 3: The requirements in this table are applicable to FR2 PC6 UE with the network signalling [highSpeedMeasFlag-r17] configured as [set2]. |  |

Table 6.2.1.6-3a: UE spherical coverage evaluation areas for power class 6

|  | θ range (degree) | ϕ range (degree) |
| --- | --- | --- |
| Area-1 | 90 to 60 | - 37.5 to + 37.5 |
| Area-2 | 90 to 60 | 142.5to 217.5 |
| NOTE 1: When testing power class 6 UEs, DUT orientation can be determined according to the UE spherical coverage evaluation areas, not necessarily following default alignment in Figure J.1-2 or positioning guidelines in clause J.3. NOTE 2: High speed train deployment is expected to be w.r.t. the reference coordination system: θ = 90 (degree) corresponds to the ground plane the train is running on, and ϕ= 0 or 180 with θ = 90 are the train track directions. |  |  |

For the UEs that support multiple FR2 bands, minimum requirement for peak EIRP and EIRP spherical coverage in Tables 6.2.1.6-1 and 6.2.1.6-3 shall be decreased per band, respectively, by the peak EIRP relaxation parameter MBP,n and EIRP spherical coverage relaxation parameter MBS,n, as defined in Table 6.2.1.6-4.

Table 6.2.1.6-4: UE multi-band relaxation factors for power class 6

| Band | MBP,n (dB) | MBS,n (dB) |
| --- | --- | --- |
| n257 | 0.7 | 0.7 |
| n258 | 0.7 | 0.7 |
| n261 | 0.7 | 0.7 |

#### 6.2.1.7	UE maximum output power for power class 7

The following requirements define the maximum output power radiated by the UE for any transmission bandwidth within the channel bandwidth for non-CA configuration, unless otherwise stated. The period of measurement shall be at least one sub frame (1ms). In case of initial access and RRC_INACTIVE, the cumulative period of measurement shall equal or exceed 1ms. The minimum output power values for EIRP are found in Table 6.2.1.7-1. The requirement is verified with the test metric of total component of EIRP (Link=TX beam peak direction, Meas=Link angle).

Table 6.2.1.7-1: UE minimum peak EIRP for power class 7

| Operating band | Min peak EIRP (dBm) |
| --- | --- |
| n257 | 16.4 |
| n258 | 16.4 |
| n261 | 16.4 |
| NOTE 1: Minimum peak EIRP is defined as the lower limit without tolerance NOTE 2: Void NOTE 3: Minimum peak EIRP does not apply to initial access and RRC_INACTIVE. |  |

The maximum output power values for TRP and EIRP are found on the Table 6.2.1.7-2. The max allowed EIRP is derived from regulatory requirements [8]. The requirements are verified with the test metrics of TRP (Link=TX beam peak direction, Meas=TRP grid) in beam locked mode and the total component of EIRP (Link=TX beam peak direction, Meas=Link angle.

Table 6.2.1.7-2: UE maximum output power limits for power class 7

| Operating band | Max TRP (dBm) | Max EIRP (dBm) |
| --- | --- | --- |
| n257 | 23 | 43 |
| n258 | 23 | 43 |
| n261 | 23 | 43 |

The minimum EIRP at the 50th percentile of the distribution of radiated power measured over the full sphere around the UE is defined as the spherical coverage requirement and is found in Table 6.2.1.7-3 below. The requirement is verified with the test metric of the total component of EIRP (Link=Beam peak search grids, Meas=Link angle).

Table 6.2.1.7-3: UE spherical coverage for power class 7

| Operating band | Min EIRP at 50 %-tile CDF (dBm) |
| --- | --- |
| n257 | 5.5 |
| n258 | 5.5 |
| n261 | 5.5 |
| NOTE 1: Minimum EIRP at 50 %-tile CDF is defined as the lower limit without tolerance in RRC_CONNECTED. NOTE 2: The requirements in this table are verified only under normal temperature conditions as defined in Annex E.2.1. NOTE 3: Minimum EIPR at 50%-tile CDF is defined as the lower limit minus 2 dB in initial access and RRC_INACTIVE |  |

For power class 7 UEs that support multiple FR2 bands, minimum requirement for peak EIRP and EIRP spherical coverage in Tables 6.2.1.7-1 and 6.2.1.7-3 shall be decreased per band, respectively, by the peak EIRP relaxation parameter MBP,n and EIRP spherical coverage relaxation parameter MBS,n, as defined for power class 3 in Table 6.2.1.3-4.

### <Next Changes>

### 6.6.6	Beam correspondence for power class 5

#### 6.6.6.1	General

The beam correspondence requirement for power class 5 UEs in RRC_CONNECTED consists of two components: UE minimum peak EIRP (as defined in Clause 6.2.1.5), and UE spherical coverage (as defined in Clause 6.2.1.5). The beam correspondence requirement is fulfilled if the UE satisfies one of the following conditions, depending on the UE's beam correspondence capability IE beamCorrespondenceWithoutUL-BeamSweeping, as defined in TS 38.306 [14]:

--	If beamCorrespondenceWithoutUL-BeamSweeping is supported, the UE shall meet the minimum peak EIRP requirement according to Table 6.2.1.5-1 and spherical coverage requirement according to Table 6.2.1.5-3 with its autonomously chosen UL beams and without uplink beam sweeping.  Such a UE is considered to have met the beam correspondence tolerance requirement.

-	If beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16 are supported, the UE shall meet the minimum peak EIRP requirement according to Table 6.2.1.5-1 and spherical coverage requirement according to Table 6.2.1.5-3 using the side conditions for SSB based enhanced beam correspondence requirements as defined in Clause 6.6.6.3.2.

-	If beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceCSI-RS-based-r16 are supported, the UE shall meet the minimum peak EIRP requirement according to Table 6.2.1.5-1 and spherical coverage requirement according to Table 6.2.1.5-3 using the side conditions for CSI-RS based enhanced beam correspondence requirements as defined in Clause 6.6.6.3.3.

The beam correspondence requirement for power class 5 UEs in initial access and in RRC_INACTIVE consists of UE spherical coverage only:

-	UE shall meet the spherical coverage requirement for initial access and RRC_INACTIVE according to Table 6.2.1.5-3, with its autonomously chosen UL beams and without uplink beam sweeping, using the side conditions for initial access and in RRC_INACTIVE as defined in Clause 6.6.6.3.4.

#### 6.6.6.2	(Reserved)

Editor’s note: FFS if power class 5 UE can rely on UL beam sweeping to meet min peak EIRP and spherical requirements.

#### 6.6.6.3	Side Conditions

##### 6.6.6.3.1	Side Condition for beam correspondence based on SSB and CSI-RS

The beam correspondence requirements are only applied under the following side conditions:

-	The downlink reference signals including both SSB and CSI-RS are provided and Type D QCL shall be maintained between SSB and CSI-RS.

-	The reference measurement channel for beam correspondence is fulfilled according to the CSI-RS configuration in Annex A.3.

-	For beam correspondence, conditions for L1-RSRP measurements are fulfilled according to Table 6.6.6.3.1-1 and Table 6.6.6.3.1-2.

Table 6.6.6.3.1-1: Conditions for SSB based L1-RSRP measurements for beam correspondence

| Angle of arrival | NR operating bands | Minimum SSB_RP Note 2 | SSB Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm / SCSSSB | dB |
|  |  | SCSSSB = 120 kHz |  |
| All angles Note 1 | n257 | -103.6 | ≥6 |
|  | n258 | -103.6 |  |
|  | n259 | -100.5 |  |
| NOTE 1: For UEs that support multiple FR2 bands, the Minimum SSB_RP values for all angles are increased by ΔMBS, the UE multi-band relaxation factor in dB specified in clause 6.2.1.5 NOTE 2: Values specified at the radiated requirements reference point to give minimum SSB Ês/Iot, with no applied noise. |  |  |  |

Table 6.6.6.3.1-2: Conditions for CSI-RS based L1-RSRP measurements for beam correspondence

| Angle of arrival | NR operating bands | Minimum CSI-RS_RP Note 2 | CSI-RS Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm / SCSCSI-RS | dB |
|  |  | SCSCSI-RS = 120 kHz |  |
| All angles Note 1 | n257 | -103.6 | ≥6 |
|  | n258 | -103.6 |  |
|  | n259 | -100.5 |  |
| NOTE 1: For UEs that support multiple FR2 bands, the Minimum CSI-RS_RP values are increased by ΔMBS, the UE multi-band relaxation factor in dB specified in clause 6.2.1.5 NOTE 2: Values specified at the radiated requirements reference point to give minimum CSI-RS Ês/Iot, with no applied noise. |  |  |  |

##### 6.6.6.3.2	Side Condition for SSB based enhanced Beam Correspondence requirements

The beam correspondence requirements for beam correspondence based on SSB are only applied under the following side conditions:

-	The downlink reference signal SSB is provided, and CSI-RS is not provided.

-	For beam correspondence, conditions for L1-RSRP measurements are fulfilled according to Table 6.6.6.3.1-1.

##### 6.6.6.3.3	Side Condition for CSI-RS based enhanced Beam Correspondence requirements

The beam correspondence requirements for beam correspondence based on CSI-RS are only applied under the following side conditions:

-	The downlink reference signals including both SSB and CSI-RS are provided.

-	The reference measurement channel for beam correspondence is fulfilled according to the CSI-RS configuration in Annex A.3.

-	For beam correspondence, conditions for L1-RSRP measurements are fulfilled according to Table 6.6.4.3.1-2 and SSB signal is provided according to Table 6.6.6.3.3-1.

Table 6.6.6.3.3-1: SSB signal conditions for CSI-RS based beam correspondence requirements

| Angle of arrival | NR operating bands | Minimum SSB_RP Note 2 | SSB Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm / SCSSSB | dB |
|  |  | SCSSSB = 120 kHz |  |
| All angles Note 1 | n257 | -108.6 | ≥1 |
|  | n258 | -108.6 |  |
|  | n259 | -105.5 |  |
| NOTE 1: For UEs that support multiple FR2 bands, the Minimum SSB_RP values for all angles are increased by ΔMBS, the UE multi-band relaxation factor in dB specified in clause 6.2.1.5 NOTE 2: Values specified at the radiated requirements reference point to give minimum SSB Ês/Iot, with no applied noise. |  |  |  |

##### 6.6.6.3.4	Side Condition for Beam Correspondence requirements in initial access and RRC_INACTIVE

The beam correspondence requirements for beam correspondence in initial access and RRC_INACTIVE are only applied under side conditions in Clause 6.6.6.3.2.

#### 6.6.6.4	Applicability

For UEs supporting more than one type of beam correspondence, the following applicability rules apply:

-	If a UE meets enhanced beam correspondence requirements either based on SSB or based on CSI-RS, it is considered to have met the beam correspondence requirements based on SSB and CSI-RS.

-	For a UE supporting either SSB based or CSI-RS based enhanced beam correspondence, the UE shall meet the supported enhanced beam correspondence requirements.

-	For a UE supporting both SSB based and CSI-RS based enhanced beam correspondence, the UE shall meet both SSB based and CSI-RS based enhanced beam correspondence requirements and the following applicability rules for verifying the requirements apply:

-	The enhanced beam correspondence requirements shall be verified with the SSB based enhanced beam correspondence side conditions in clause 6.6.6.3.2. If UE meets the SSB based enhanced beam correspondence requirements using the side conditions in clause 6.6.6.3.2 and meets the minimum peak EIRP requirement as defined in clause 6.2.1.5 using the CSI-RS based side conditions in clause 6.6.6.3.3, where the link direction is determined in the SSB based enhanced beam correspondence test, the UE is considered to have met both the SSB based and CSI-RS based enhanced beam correspondence requirements.

-	Otherwise, if UE does not meet the minimum peak EIRP requirement as defined in clasue 6.2.1.3 using the CSI-RS based side conditions in clause 6.6.6.3.3, the enhanced beam correspondence requirements shall be further verified for the UE with the CSI-RS based enhanced beam correspondence side conditions in clause 6.6.6.3.3.

For the beam correspondence requirement for power class 5 UEs in initial access and in RRC_INACTIVE, the following applicability rules apply:

- If a UE meets beam correspondence requirements in initial access, it is considered to have met the beam correspondence requirements in RRC_INACTIVE.

### 6.6.7	Beam correspondence for power class 6

#### 6.6.7.1	General

The beam correspondence requirement for power class 6 UEs in RRC_CONNECTED consists of two components: UE minimum peak EIRP (as defined in Clause 6.2.1.6), and UE spherical coverage (as defined in Clause 6.2.1.6).

Power class 6 UE shall mandatorily support beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16. The UE shall meet the minimum peak EIRP requirement according to Table 6.2.1.6-1 and spherical coverage requirement according to Table 6.2.1.6-3 using the side conditions for SSB based enhanced beam correspondence requirements as defined in Clause 6.6.7.3.2.

If the UE also support beamCorrespondenceCSI-RS-based-r16, the UE shall also meet the minimum peak EIRP requirement according to Table 6.2.1.6-1 and spherical coverage requirement according to Table 6.2.1.6-3 using the side conditions for CSI-RS based enhanced beam correspondence requirements as defined in Clause 6.6.7.3.3.

The beam correspondence requirement for power class 6 UEs in initial access and in RRC_INACTIVE consists of UE spherical coverage only:

-	UE shall meet the spherical coverage requirement for initial access and RRC_INACTIVE according to Table 6.2.1.6-3, with its autonomously chosen UL beams and without uplink beam sweeping, using the side conditions for initial access and in RRC_INACTIVE as defined in Clause 6.6.7.3.4.

#### 6.6.7.2	(Void)

Editor’s note: Not need to define beam correspondence tolerance requirement because power class 6 UE shall mandatorily support beamCorrespondenceWithoutUL-BeamSweeping.

#### 6.6.7.3	Side Conditions

##### 6.6.7.3.1	(Void)

Editor’s note: Not need to define the side condition for beam correspondence based on SSB and CSI-RS, because power class 6 UE shall mandatorily support SSB based enhanced beam correspondence.

##### 6.6.7.3.2	Side Condition for SSB based enhanced Beam Correspondence requirements

The beam correspondence requirements for beam correspondence based on SSB are only applied under the following side conditions:

-	The downlink reference signal SSB is provided, and CSI-RS is not provided.

-	For beam correspondence, conditions for L1-RSRP measurements are fulfilled according to Table 6.6.7.3.2-1.

Table 6.6.7.3.2-1: Conditions for SSB based L1-RSRP measurements for beam correspondence

| Angle of arrival | NR operating bands | Minimum SSB_RP Note 2 | SSB Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm / SCSSSB | dB |
|  |  | SCSSSB = 120 kHz |  |
| All angles Note 1 | n257 | -101.4 | ≥6 |
|  | n258 | -101.6 |  |
|  | n261 | -101.4 |  |
| NOTE 1: For UEs that support multiple FR2 bands, the Minimum SSB_RP values for all angles are increased by ΔMBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1.6. NOTE 2: Values specified at the radiated requirements reference point to give minimum SSB Ês/Iot, with no applied noise. |  |  |  |

##### 6.6.7.3.3	Side Condition for CSI-RS based enhanced Beam Correspondence requirements

The beam correspondence requirements for beam correspondence based on CSI-RS are only applied under the following side conditions:

-	The downlink reference signals including both SSB and CSI-RS are provided.

-	The reference measurement channel for beam correspondence is fulfilled according to the CSI-RS configuration in Annex A.3.

-	For beam correspondence, conditions for L1-RSRP measurements are fulfilled according to Table 6.6.7.3.3-2 and SSB signal is provided according to Table 6.6.7.3.3-1.

Table 6.6.7.3.3-1: SSB signal conditions for CSI-RS based beam correspondence requirements

| Angle of arrival | NR operating bands | Minimum SSB_RP Note 2 | SSB Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm / SCSSSB | dB |
|  |  | SCSSSB = 120 kHz |  |
| All angles Note 1 | n257 | -106.4 | ≥1 |
|  | n258 | -106.6 |  |
|  | n261 | -106.4 |  |
| NOTE 1: For UEs that support multiple FR2 bands, the Minimum SSB_RP values for all angles are increased by ΔMBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1.6 NOTE 2: Values specified at the radiated requirements reference point to give minimum SSB Ês/Iot, with no applied noise. |  |  |  |

Table 6.6.7.3.3-2: Conditions for CSI-RS based L1-RSRP measurements for beam correspondence

| Angle of arrival | NR operating bands | Minimum CSI-RS_RP Note 2 | CSI-RS Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm / SCSCSI-RS | dB |
|  |  | SCSCSI-RS = 120 kHz |  |
| All angles Note 1 | n257 | -101.4 | ≥6 |
|  | n258 | -101.6 |  |
|  | n261 | -101.4 |  |
| NOTE 1: For UEs that support multiple FR2 bands, the Minimum CSI-RS_RP values are increased by ΔMBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1.6 NOTE 2: Values specified at the radiated requirements reference point to give minimum CSI-RS Ês/Iot, with no applied noise. |  |  |  |

##### 6.6.7.3.4	Side Condition for Beam Correspondence requirements in initial access and RRC_INACTIVE

The beam correspondence requirements for beam correspondence in initial access and RRC_INACTIVE are only applied under side conditions in Clause 6.6.7.3.2.

#### 6.6.7.4	Applicability

For UEs supporting more than one type of beam correspondence, the following applicability rules apply:

-	If a UE meets enhanced beam correspondence requirements either based on SSB or based on CSI-RS, it is considered to have met the beam correspondence requirements based on SSB and CSI-RS.

-	For a UE supporting either SSB based or CSI-RS based enhanced beam correspondence, the UE shall meet the supported enhanced beam correspondence requirements.

-	For a UE supporting both SSB based and CSI-RS based enhanced beam correspondence UE shall meet the both SSB based and CSI-RS based enhanced beam correspondence requirements and the following applicability rules for verifying the requirements apply:

-	The enhanced beam correspondence requirements shall be verified with the SSB based enhanced beam correspondence side conditions in clause 6.6.7.3.2. If the UE meets the SSB based enhanced beam correspondence requirements using the side conditions in clause 6.6.7.3.2 and meets the minimum peak EIRP requirement as defined in clause 6.2.1.6 using the CSI-RS based side conditions in clause 6.6.7.3.3, where the link direction is determined in the SSB based enhanced beam correspondence test, the UE is considered to have met both the SSB based and CSI-RS based enhanced beam correspondence requirements.

-	Otherwise, if UE does not meet the minimum peak EIRP requirement as defined in clause 6.2.1.6 using the CSI-RS based side conditions in clause 6.6.7.3.3, the enhanced beam correspondence requirements shall be further verified for the UE with the CSI-RS based enhanced beam correspondence side conditions in clause 6.6.7.3.3.

For the beam correspondence requirement for power class 6 UEs in initial access and in RRC_INACTIVE, the following applicability rules apply:

- If a UE meets beam correspondence requirements in initial access, it is considered to have met the beam correspondence requirements in RRC_INACTIVE.

### 6.6.8	Beam correspondence for power class 7

#### 6.6.8.1	General

The beam correspondence requirement for power class 7 UEs in RRC_CONNECTED consists of two components: UE minimum peak EIRP (as defined in Clause 6.2.1.7), and UE spherical coverage (as defined in Clause 6.2.1.7). The beam correspondence requirement is fulfilled if the UE satisfies one of the following conditions, depending on the UE's beam correspondence capability IE beamCorrespondenceWithoutUL-BeamSweeping, as defined in TS 38.306 [14]:

--	If beamCorrespondenceWithoutUL-BeamSweeping is supported, the UE shall meet the minimum peak EIRP requirement according to Table 6.2.1.7-1 and spherical coverage requirement according to Table 6.2.1.7-3 with its autonomously chosen UL beams and without uplink beam sweeping.  Such a UE is considered to have met the beam correspondence tolerance requirement.

-	If beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceSSB-based-r16 are supported, the UE shall meet the minimum peak EIRP requirement according to Table 6.2.1.7-1 and spherical coverage requirement according to Table 6.2.1.7-3 using the side conditions for SSB based enhanced beam correspondence requirements as defined in Clause 6.6.8.3.2.

-	If beamCorrespondenceWithoutUL-BeamSweeping and beamCorrespondenceCSI-RS-based-r16 are supported, the UE shall meet the minimum peak EIRP requirement according to Table 6.2.1.7-1 and spherical coverage requirement according to Table 6.2.1.7-3 using the side conditions for CSI-RS based enhanced beam correspondence requirements as defined in Clause 6.6.8.3.3.

The beam correspondence requirement for power class 7 UEs in initial access and in RRC_INACTIVE consists of UE spherical coverage only:

-	UE shall meet the spherical coverage requirement for initial access and RRC_INACTIVE according to Table 6.2.1.7-3, with its autonomously chosen UL beams and without uplink beam sweeping, using the side conditions for initial access and in RRC_INACTIVE as defined in Clause 6.6.8.3.4.

#### 6.6.8.2	Void

#### 6.6.8.3	Side Conditions

##### 6.6.8.3.1	Side Condition for beam correspondence based on SSB and CSI-RS

The beam correspondence requirements are only applied under the following side conditions:

-	The downlink reference signals including both SSB and CSI-RS are provided and Type D QCL shall be maintained between SSB and CSI-RS.

-	The reference measurement channel for beam correspondence is fulfilled according to the CSI-RS configuration in Annex A.3.

-	For beam correspondence, conditions for L1-RSRP measurements are fulfilled according to Table 6.6.8.3.1-1 and Table 6.6.8.3.1-2.

Table 6.6.8.3.1-1: Conditions for SSB based L1-RSRP measurements for beam correspondence

| Angle of arrival | NR operating bands | Minimum SSB_RP Note 2 | SSB Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm / SCSSSB | dB |
|  |  | SCSSSB = 120 kHz |  |
| All angles Note 1 | n257 | -93.2 | ≥6 |
|  | n258 | -93.2 |  |
|  | n261 | -93.2 |  |
| NOTE 1: Void NOTE 2: Values specified at the radiated requirements reference point to give minimum SSB Ês/Iot, with no applied noise. |  |  |  |

Table 6.6.8.3.1-2: Conditions for CSI-RS based L1-RSRP measurements for beam correspondence

| Angle of arrival | NR operating bands | Minimum CSI-RS_RP Note 2 | CSI-RS Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm / SCSCSI-RS | dB |
|  |  | SCSCSI-RS = 120 kHz |  |
| All angles Note 1 | n257 | -93.2 | ≥6 |
|  | n258 | -93.2 |  |
|  | n261 | -93.2 |  |
| NOTE 1: For UEs that support multiple FR2 bands, the Minimum SSB_RP values for all angles are increased by MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1. NOTE 2: Values specified at the radiated requirements reference point to give minimum CSI-RS Ês/Iot, with no applied noise. |  |  |  |

##### 6.6.8.3.2	Side Condition for SSB based enhanced Beam Correspondence requirements

The beam correspondence requirements for beam correspondence based on SSB are only applied under the following side conditions:

-	The downlink reference signal SSB is provided, and CSI-RS is not provided.

-	For beam correspondence, conditions for L1-RSRP measurements are fulfilled according to Table 6.6.8.3.1-1.

##### 6.6.8.3.3	Side Condition for CSI-RS based enhanced Beam Correspondence requirements

The beam correspondence requirements for beam correspondence based on CSI-RS are only applied under the following side conditions:

-	The downlink reference signals including both SSB and CSI-RS are provided.

-	The reference measurement channel for beam correspondence is fulfilled according to the CSI-RS configuration in Annex A.3.

-	For beam correspondence, conditions for L1-RSRP measurements are fulfilled according to Table 6.6.8.3.1-2 and SSB signal is provided according to Table 6.6.8.3.3-1.

Table 6.6.8.3.3-1: SSB signal conditions for CSI-RS based beam correspondence requirements

| Angle of arrival | NR operating bands | Minimum SSB_RP Note 2 | SSB Ês/Iot |
| --- | --- | --- | --- |
|  |  | dBm / SCSSSB | dB |
|  |  | SCSSSB = 120 kHz |  |
| All angles Note 1 | n257 | -98.2 | ≥1 |
|  | n258 | -98.2 |  |
|  | n261 | -98.2 |  |
| NOTE 1: For UEs that support multiple FR2 bands, the Minimum SSB_RP values for all angles are increased by MBS,n, the UE multi-band relaxation factor in dB specified in clause 6.2.1. NOTE 2: Values specified at the radiated requirements reference point to give minimum SSB Ês/Iot, with no applied noise. |  |  |  |

##### 6.6.8.3.4	Side Condition for Beam Correspondence requirements in initial access and RRC_INACTIVE

The beam correspondence requirements for beam correspondence in initial access and RRC_INACTIVE are only applied under side conditions in Clause 6.6.8.3.2.

#### 6.6.8.4	Applicability

For UEs supporting more than one type of beam correspondence, the following applicability rules apply:

-	If a UE meets enhanced beam correspondence requirements either based on SSB or based on CSI-RS, it is considered to have met the beam correspondence requirements based on SSB and CSI-RS.

-	For a UE supporting either SSB based or CSI-RS based enhanced beam correspondence, the UE shall meet the supported enhanced beam correspondence requirements.

-	For a UE supporting both SSB based and CSI-RS based enhanced beam correspondence, the UE shall meet both SSB based and CSI-RS based enhanced beam correspondence requirements and the following applicability rules for verifying the requirements apply:

-	The enhanced beam correspondence requirements shall be verified with the SSB based enhanced beam correspondence side conditions in clause 6.6.8.3.2. If UE meets the SSB based enhanced beam correspondence requirements using the side conditions in clause 6.6.8.3.2 and meets the minimum peak EIRP requirement as defined in clause 6.2.1.7 using the CSI-RS based side conditions in clause 6.6.8.3.3, where the link direction is determined in the SSB based enhanced beam correspondence test, the UE is considered to have met both the SSB based and CSI-RS based enhanced beam correspondence requirements.

-	Otherwise, if UE does not meet the minimum peak EIRP requirement as defined in clasue 6.2.1.7 using the CSI-RS based side conditions in clause 6.6.8.3.3, the enhanced beam correspondence requirements shall be further verified for the UE with the CSI-RS based enhanced beam correspondence side conditions in clause 6.6.8.3.3.

For the beam correspondence requirement for power class 7 UEs in initial access and in RRC_INACTIVE, the following applicability rules apply:

- If a UE meets beam correspondence requirements in initial access, it is considered to have met the beam correspondence requirements in RRC_INACTIVE.

### <End of Changes>