3GPP TSG-RAN4 Meeting # 110	R4-2402416

# Athens, Greece,  26th February  - 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.115-1 | CR |  | rev | - | Current version: | 18.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.115-1: Addition of test procedures for NCR demodulation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Perf |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | This CR adds test procedures and test requirements for NCR demodulation according to the agreed work-split. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test requirements and procedures added for PDSCH and PDCCH |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test procedures or requirements for conducted NCR demodulation testing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New sections 8, 8.1, sub-sections |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.106 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 8 	Conducted performance requirements

## 8.1	NCR-MT Performance requirements

### 8.1.1	General

#### 8.1.1.1	Scope and definitions

Conducted performance requirements specify the ability of the NCR-MT to correctly demodulate signals in various conditions and configurations. Conducted performance requirements are specified at the antenna connectors (NCR-MT type 1-C) or TAB connector(s) (NCR type 1-H).

Conducted performance requirements for the NCR-MT are specified for the fixed reference channels and the propagation conditions defined in TS 38.106 [2] annex F and annex G, respectively.

The SNR used in this clause is specified based on a single carrier and defined as:

SNR = S / N

Where:

S	is the total signal energy in a slot on a single antenna connector or TAB connector.

N	is the noise energy in a bandwidth corresponding to the transmission bandwidth over the duration of a slot.

### 8.1.2	Demodulation performance requirements

#### 8.1.2.1	General

#### 8.1.2.2	Performance requirements for PDSCH

##### 8.1.2.2.1	Definition and applicability

The performance requirement of PDSCH is determined by two means; a minimum required throughput for a given SNR and a minimum SNR at which 1% BLER for the first transmission is achieved. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in annex F. The throughput performance requirements assume HARQ re-transmissions, whereas the first transmission BLER requirements do not consider HARQ.

##### 8.1.2.2.2	Minimum requirement

The minimum requirement is in TS 38.106 [2] clause 8.3.1.

##### 8.1.2.2.3	Test purpose

The test shall verify the receiver’s ability to achieve throughput  and 1st transmission BLER under multipath fading propagation conditions for a given SNR.

##### 8.1.2.2.4	Method of test

8.1.2.2.4.1	Initial conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

8.1.2.2.4.2	Procedure

1)	Connect the NCR tester generating the wanted signal, multipath fading simulators and AWGN generators to all NCR-MT TAB connectors or antenna connectors  for diversity reception via a combining network as shown in annex D.9.

2)	Adjust the AWGN generator and adjust the AWGN power level to -77.2 dBm / 38.16MHz.

3)	The characteristics of the wanted signal shall be configured according to the corresponding DL reference measurement channel defined in annex F and the test parameters in table 8.1.2.2.4.2-1.

Table 8.1.2.2.4.2-1: Test parameters for testing PDSCH

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active BWP index |  |  | 1 |
| Default TDD UL-DL pattern (Note 1) |  |  | 7D1S2U, S=6D:4G:4U |
| PDSCH transmission scheme |  |  | Transmission scheme 1 |
| Carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 1) | RBs | 0 |
|  | Subcarrier spacing | kHz | 30 |
| DL BWP configuration #1 | Cyclic prefix |  | Normal |
|  | RB offset | RBs | 0 |
|  | Number of contiguous PRB | PRBs | 106 |
| PDSCH DMRS configuration | Antenna ports indexes |  | {1000} for Rank 1 tests {1000, 1001} for Rank 2 tests {1000-1002} for Rank 3 tests {1000-1003} for Rank 4 tests |
|  | Position of the first DMRS for PDSCH mapping type A |  | 2 |
|  | Number of PDSCH DMRS CDM group(s) without data |  | 1 for Rank 1 and Rank 2 tests 2 for Rank 3 and Rank 4 tests |
|  | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | Specific to each Reference channel |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PT-RS configuration |  |  | PT-RS is not configured |
| Maximum number of code block groups for ACK/NACK feedback |  |  | 1 |
| Maximum number of HARQ transmission |  |  | 4 |
| HARQ ACK/NACK bundling |  |  | Multiplexed |
| Redundancy version coding sequence |  |  | {0,2,3,1} |
| PDSCH & PDSCH DMRS Precoding configuration |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Note 1: The same requirements are applicable to TDD with different UL-DL patterns. Note 2: Point A coincides with minimum guard band as specified in TS 38.101-1 [2] for tested channel bandwidth and subcarrier spacing. |  |  |  |

4)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex F.

5)	Adjust the equipment so that required SNR specified in table 81.2.2.5.1-1 is achieved at the IAB-MT input.

6)	For each of the reference channels in table 8.1.2.2.5.1-1 measure the throughput and the 1st transmission BLER.

##### 8.1.2.2.5	Test requirement

The throughput measured according to clause 8.1.2.2.4.2 shall not be below the limits for the SNR levels specified in table 8.1.2.2.5-1.

Table 8.1.2.2.5-1: Minimum performance PDSCH

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | MCS | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Test metric | SNR (dB) |
| 1 |  | 10 / 15 | 4 | TDLA30-10 | 2x2 | 70% TPUT |  |
| 2 |  | 10 / 15 | 4 | TDLA30-10 | 2x2 | 1% BLER |  |
| 3 |  | 10 / 15 | 4 | TDLA30-10 | 2x4 | 70% TPUT |  |
| 4 |  | 10 / 15 | 4 | TDLA30-10 | 2x4 | 1% BLER |  |
| 5 |  | 10 / 15 | 13 | TDLA30-10 | 2x2 | 70% TPUT |  |
| 6 |  | 10 / 15 | 13 | TDLA30-10 | 2x2 | 1% BLER |  |
| 7 |  | 10 / 15 | 13 | TDLA30-10 | 2x4 | 70% TPUT |  |
| 8 |  | 10 / 15 | 13 | TDLA30-10 | 2x4 | 1% BLER |  |
| 9 |  | 40 / 30 | 4 | TDLA30-10 | 2x2 | 70% TPUT |  |
| 10 |  | 40 / 30 | 4 | TDLA30-10 | 2x2 | 1% BLER |  |
| 11 |  | 40 / 30 | 4 | TDLA30-10 | 2x4 | 70% TPUT |  |
| 12 |  | 40 / 30 | 4 | TDLA30-10 | 2x4 | 1% BLER |  |
| 13 |  | 40 / 30 | 13 | TDLA30-10 | 2x2 | 70% TPUT |  |
| 14 |  | 40 / 30 | 13 | TDLA30-10 | 2x2 | 1% BLER |  |
| 15 |  | 40 / 30 | 13 | TDLA30-10 | 2x4 | 70% TPUT |  |
| 16 |  | 40 / 30 | 13 | TDLA30-10 | 2x4 | 1% BLER |  |

#### 8.1.2.3	Performance requirements for PDCCH

##### 8.1.2.3.1	Definition and applicability

The performance requirement of PDCCH is determined by a maximum allowed missed detection rate for a given SNR. The required missed detection rate is expressed for the FRCs listed in annex F.

##### 8.1.2.3.2	Minimum requirement

The minimum requirement is in TS 38.106 [2] clause 8.3.2.

##### 8.1.2.3.3	Test purpose

The test shall verify the receiver's ability to achieve missed detection rate under multipath fading propagation conditions for a given SNR.

##### 8.1.2.3.4	Method of test

8.1.2.3.4.1	Initial conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

8.1.2.3.4.2	Procedure

1)	Connect the NCR tester generating the wanted signal, multipath fading simulators and AWGN generators to all NCR-MT TAB connectors or antenna connectors for diversity reception via a combining network as shown in annex D.9.

2)	Adjust the AWGN generator and adjust the AWGN power level to -77.2 dBm / 38.16MHz.

3)	The characteristics of the wanted signal shall be configured according to the corresponding DL reference measurement channel defined in annex A and the test parameters in table 8.1.2.3.4.2-1.

Table 8.1.2.3.4.2-1: Test parameters for testing PDCCH

| Parameter | Unit | 1 Tx Antenna |  |  | 2 Tx Antenna |
| --- | --- | --- | --- | --- | --- |
| CCE to REG mapping type |  | interleaved |  | interleaved |  |
| Interleaver size |  | 3 |  |  |  |
| REG bundle size |  | 2 | 6 |  |  |
| Shift Index |  | 0 |  |  |  |

4)	The multipath fading emulators shall be configured according to the corresponding channel model defined in annex F.

5)	Adjust the equipment so that required SNR specified in table 8.1.2.3.5.1-1is achieved at the NCR-MT input.

6)	For each of the reference channels in table 8.1.2.3.5.1-1, applicable for the NCR-MT, measure the missed detection.

##### 8.1.2.3.5	Test requirement

For the parameters specified in Table 8.1.2.3.4.2.1-1, the average probability of a missed downlink scheduling grant (Pm-dsg) shall be below the specified value in Table 8.1.2.3.5-1 for SCS supported by the NCR-MT.

Table 8.1.2.3.5-1: Minimum performance for PDCCH

| Test number | Bandwidth (MHz) |  | CORESET RB | CORESET duration | Aggregation level | Reference Channel | Propagation Condition | Antenna configuration and correlation Matrix | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | SCS |  |  |  |  |  |  | Pm-dsg (%) | SNR (dB) |
| 1 | 10 | 15 | 24 | 2 | 2 |  | TDLA30-10 | 1x2 | 1 |  |
| 2 | 10 | 15 | 24 | 2 | 2 |  | TDLA30-10 | 1x4 | 1 |  |
| 3 | 10 | 15 | 48 | 2 | 4 |  | TDLA30-10 | 1x2 | 1 |  |
| 4 | 10 | 15 | 48 | 2 | 4 |  | TDLA30-10 | 1x4 | 1 |  |
| 5 | 10 | 15 | 48 | 1 | 8 |  | TDLA30-10 | 2x2 | 1 |  |
| 6 | 10 | 15 | 48 | 1 | 8 |  | TDLA30-10 | 2x4 | 1 |  |
| 7 | 40 | 30 | 102 | 1 | 2 |  | TDLA30-10 | 1x2 | 1 |  |
| 8 | 40 | 30 | 102 | 1 | 2 |  | TDLA30-10 | 1x4 | 1 |  |
| 9 | 40 | 30 | 102 | 1 | 4 |  | TDLA30-10 | 1x2 | 1 |  |
| 10 | 40 | 30 | 102 | 1 | 4 |  | TDLA30-10 | 1x4 | 1 |  |
| 11 | 40 | 30 | 90 | 1 | 8 |  | TDLA30-10 | 2x2 | 1 |  |
| 12 | 40 | 30 | 90 | 1 | 8 |  | TDLA30-10 | 2x4 | 1 |  |