3GPP TSG-RAN4 Meeting # 110	R4-2402417

# Athens, Greece,  26th February – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.115-2 | CR |  | rev |  | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to 38.115-2: Addition of CSI reporting performance test description and requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_netcon_repeater-Perf |  |  |  |  |  | Date: |  |  | 2024-02-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Test procedures and test requirements for OTA CSI reporting requirements should be added to the ocnformance specification according to the work split. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New section 8.1.3 added with CSI reporting performance tests and requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No OTA CSI test procedure for NCR-MT. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New section 8.1.3 and sub-sections |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR 38.106 CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications"

[2]	3GPP TS 38.106: "NR repeater radio transmission and reception"

[3]	Recommendation ITU-R M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000"

[4]	ITU-R Recommendation SM.329: "Unwanted emissions in the spurious domain"

[5]	3GPP TS 38.104: "NR Base Station (BS) radio transmission and reception"

[6]	3GPP TS 38.141-2: "NR; Base Station (BS) conformance testing; Part 2: Radiated conformance testing"

[7]	IEC 60 721-3-3: "Classification of environmental conditions - Part 3-3: Classification of groups of environmental parameters and their severities - Stationary use at weather protected locations"

[8]	IEC 60 721-3-4: "Classification of environmental conditions - Part 3: Classification of groups of environmental parameters and their severities - Clause 4: Stationary use at non-weather protected locations"

[9]	IEC 60 721: "Classification of environmental conditions"

[10]	IEC 60 068-2-1 (2007): "Environmental testing - Part 2: Tests. Tests A: Cold"

[11]	IEC 60 068-2-2: (2007): "Environmental testing - Part 2: Tests. Tests B: Dry heat"

[12]	IEC 60 068-2-6: (2007): "Environmental testing - Part 2: Tests - Test Fc: Vibration (sinusoidal)"

[13]	3GPP TR 37.941: "Radio Frequency (RF) conformance testing background for radiated Base Station (BS) requirements"

[14]	3GPP TS 38.101-2: "NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone".

[15]	3GPP TS 38.214: “NR; Physical layer procedures for data”.

[16]	3GPP TS 38.101-4: “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”

### 8.1.3	CSI reporting requirements

#### 8.1.3.1	General

##### 8.1.3.1.1	Applicability of requirements

Unless otherwise stated, the tests shall apply only for each subcarrier spacing declared to be supported

Unless otherwise stated, for each subcarrier spacing declared to be supported, if NCR-MT supports multiple TDD UL-DL patterns, only one of the supported TDD UL-DL patterns shall be used for all tests.

##### 8.1.3.1.2	Common test parameters

Parameters specified in Table 8.1.3.1.2-1 are applied for all test cases in this clause unless otherwise stated.

Table 8.1.3.1.2-1: Test parameters for CSI test cases

| Parameter |  |  | Unit | Value |
| --- | --- | --- | --- | --- |
| PDSCH transmission scheme |  |  |  | Transmission scheme 1 |
| Duplex mode |  |  |  | TDD |
| PTRS epre-Ratio |  |  |  | 0 |
| Actual carrier configuration | Offset between Point A and the lowest usable subcarrier on this carrier (Note 3) |  | RBs | 0 |
|  | Subcarrier spacing |  | kHz | 120 |
| DL BWP configuration #1 | Cyclic prefix |  |  | Normal |
|  | RB offset |  | RBs | 0 |
|  | Number of contiguous PRB |  | PRBs | 66 |
| Active DL BWP index |  |  |  | 1 |
| PDSCH configuration |  | Mapping type |  | Type A |
|  |  | k0 |  | 0 |
|  |  | Starting symbol (S) |  | 2 |
|  |  | Length (L) |  | 12 |
|  |  | PDSCH aggregation factor |  | 1 |
|  |  | PRB bundling type |  | Static |
|  |  | PRB bundling size |  | 2 |
|  |  | Resource allocation type |  | type 0 |
|  |  | RBG size |  | Config 2 |
|  |  | VRB-to-PRB mapping type |  | Non-interleaved |
|  |  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration |  | DMRS Type |  | Type 1 |
|  |  | Number of additional DMRS |  | 1 |
|  |  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
|  |  | DMRS ports indexes |  | {1000} for Rank1 {1000,1001} for Rank2 |
|  |  | Number of PDSCH DMRS CDM group(s) without data |  | 2 |
| PTRS configuration |  | Frequency density (KPT-RS) |  | 2 |
|  |  | Time density (LPT-RS) |  | 1 |
|  |  | Resource Element Offset |  | 2 |
| NZP CSI-RS for CSI acquisition |  | Frequency Occupation |  | Start PRB 0 Number of PRB = BWP size |
| Redundancy version coding sequence |  |  |  | {0,2,3,1} |

#### 8.1.3.2	Reporting of Channel Quality Indicator (CQI)

##### 8.1.3.2.1	Definition and applicability

The performance requirement of CSI reporting is determined by the reporting variance and the BLER performance using the transport format indicated by the reported CQI. The purpose is to verify that the reported CQI values are in accordance with the CQI definition given in TS 38.214 [15]. To account for sensitivity of the input SNR the reporting definition is considered to be verified if the reporting accuracy is met for at least one of two SNR levels separated by an offset of 1 dB.

##### 8.1.3.2.2	Minimum requirement

The minimum requirement for NCR-MT type 2-O is in TS 38.106 [2] clause 9.4.1.

##### 8.1.3.2.3	Test purpose

The test shall verify the receiver's ability to report correct median CQI and expected BLER performance under AWGN conditions.

##### 8.1.3.2.4	Method of test

8.1.3.2.4.1	Initial conditions

Test environment:	Normal, see annex B.2.

RF channels to be tested for single carrier:	M; see clause 4.9.1.

RF channels to be tested for carrier aggregation: MBW Channel CA; see clause 4.9.1.

Direction to be tested: OTA REFSENS receiver target reference direction (see D.54 in table 4.6-1).

8.1.3.2.4.2	Procedure

1)	Place the NCR-MT with its manufacturer declared coordinate system reference point in the same place as calibrated point in the test system, as shown in annex E.6.

2)	Align the manufacturer declared coordinate system orientation of the NCR-MT with the test system.

3)	Set the NCR-MT in the declared direction to be tested.

4)	Connect the NCR-MT tester generating the wanted signal and AWGN generators to a test antenna via a combining network in OTA test setup, as shown in annex E.6. Each of the demodulation branch signals should be transmitted on one polarization of the test antenna(s).

5)	The characteristics of the wanted signal shall be configured according to the corresponding DL reference measurement channel defined in annex F, and according to additional test parameters listed in table 8.1.3.2.4.2-1.

Table 8.1.3.2.4.2-1: Test parameters for testing CQI reporting requirements for FR2

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  |  | MHz | 100 |  |  |  |
| Subcarrier spacing |  |  | kHz | 120 |  |  |  |
| Duplex Mode |  |  |  | TDD |  |  |  |
| SNRBB |  |  | dB | 8 | 9 | 14 | 15 |
| Propagation channel |  |  |  | AWGN |  |  |  |
| Antenna configuration |  |  |  | 2×2 with static channel specified in Annex J.1 |  |  |  |
| Beamforming Model |  |  |  | As specified in Annex J.3.1 |  |  |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource Type |  |  | Periodic |  |  |  |
|  | Number of CSI-RS ports (X) |  |  | 2 |  |  |  |
|  | CDM Type |  |  | fd-CDM2 |  |  |  |
|  | Density (ρ) |  |  | 1 |  |  |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1 ) |  |  | 6 |  |  |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  |  | 13 |  |  |  |
|  | NZP CSI-RS-timeConfig periodicity and offset |  | slot | 5/1 |  |  |  |
| ReportConfigType |  |  |  | Periodic |  |  |  |
| CQI-table |  |  |  | Table 1 |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |
| timeRestrictionForChannelMeasurements |  |  |  | Not configured |  |  |  |
| timeRestrictionForInterferenceMeasurements |  |  |  | Not configured |  |  |  |
| cqi-FormatIndicator |  |  |  | Wideband |  |  |  |
| pmi-FormatIndicator |  |  |  | Wideband |  |  |  |
| Sub-band Size |  |  | RB | 8 |  |  |  |
| csi-ReportingBand |  |  |  | 111111111 |  |  |  |
| CSI-Report periodicity and offset |  |  | slot | 5/4 |  |  |  |
| Codebook configuration |  | Codebook Type |  | typeI-SinglePanel |  |  |  |
|  |  | Codebook Mode |  | 1 |  |  |  |
|  |  | (CodebookConfig-N1,CodebookConfig-N2) |  | Not configured |  |  |  |
|  |  | CodebookSubsetRestriction |  | 010000 |  |  |  |
|  |  | RI Restriction |  | N/A |  |  |  |
| CQI/RI/PMI delay |  |  | ms | 1.75 |  |  |  |
| Maximum number of HARQ transmission |  |  |  | 1 |  |  |  |
| Measurement channel |  |  |  | As specified in Table A.2.6-3, M-FR2-A.3.5-2 |  |  |  |
| Note 1: The same requirements are applicable for TDD with different UL-DL pattern. Note 2: SSB, TRS, CSI-RS and/or other unspecified test parameters with respect to TS 38.101-4 [16] are left up to test implementation, if transmitted or needed. Note 3: If the NCR-MT reports in an available uplink reporting instance at slot #n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). |  |  |  |  |  |  |  |

7)	Adjust the test signal mean power so the calibrated radiated SNR value at the NCR-MT receiver is as specified in clause 8.1.3.2.5.1, and that the SNR at the NCR-MT receiver is not impacted by the noise floor.

The power level for the transmission may be set such that the AWGN level at the RIB is equal to the AWGN level in table 8.1.3.2.4.2-2.

Table 8.2.3.2.4.2-2: AWGN power level at the NCR-MT input

| NCR-MT type | Sub-carrier spacing (kHz) | Channel bandwidth (MHz) | AWGN power level |
| --- | --- | --- | --- |
| NCR-MT type 2-O | 120 | 100 | EISREFSENS_50M + ΔFR2_REFSENS + 18 dBm / 95.04 MHz |
| NOTE 1: ΔFR2_REFSENS = -3 dB as described in clause 7.1, since the OTA REFSENS reference direction (as declared in D.54 in table 4.6-1) is used for testing. NOTE 2: EISREFSENS_50M as declared in D.28 in table 4.6-1. |  |  |  |

8)	For reference channels applicable to the NCR-MT, measure the median CQI and the BLER at (median CQI +1) and (median CQI -1).

##### 8.1.3.2.5	Test requirement

8.1.3.2.5.1	Test requirement for NCR type 2-O

For the parameters specified in Table 8.1.3.2.4.2-1, the minimum requirements are specified by the following:

a)	the reported CQI value shall be in the range of ±1 of the reported median more than 90% of the time;

b)	if the PDSCH BLER using the transport format indicated by median CQI is less than or equal to 0.1, the BLER using the transport format indicated by the (median CQI + 1) shall be greater than 0.1. If the PDSCH BLER using the transport format indicated by the median CQI is greater than 0.1, the BLER using transport format indicated by (median CQI – 1) shall be less than or equal to 0.1.